DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Tilehurst

App Ref: 231020/CLP

Address: 124 St Michaels Road, Tilehurst, Reading, RG30 4SE

Proposals: A new single storey side extension to the existing building as an enlargement as per Class A under permitted development and hip to gable roof extension.

Case Officer: Chukwudi Onwudinanti

Applicant: Mr Mark Reed

Date received (valid from): 21 July 2023

Target Decision date: 20 October 2023

INTRODUCTION

The application site is not a listed building or located in a conservation area. It is not in an area subject to any Article 4 restrictions.  PD Rights have not been removed.

PROPOSAL AND SUPPORTING INFORMATION

This proposal is for a Certificate of Lawfulness for various works to the dwellinghouse. The proposals include:

- a single storey side extension, measuring 1.85m wide and 6.5m deep.

- Hip to gable roof extension (for loft conversion)

- Rear dormer

- Three velux roof windows on the existing rear extension

- One new velux roof window to the rear slope

- Extension of the height of the existing rear extension by 0.2m and change from pitched to flat roof

- Render finish applied to new gable wall to the side

- Removal of two existing chimneys

- Window/door changes on west (side) and front (south) elevations

Plans and Documents Considered received 26/07/2023

Drawing No: 04 Rev A: Proposed Plans & Elevations

Drawing No: 03 Rev A: Existing Plans & Elevations

Drawing No: 02 Rev A: Block Plan

Location Plan

RELEVANT PLANNING HISTORY

Application No: 050207

Proposal: Single storey rear extension.

Decision: Permitted

Decision Date: 21/7/2005

Application NO: 142027

Proposal: Conversion of loft space with rear dormer including full internal refurbishment of property.

Decision: Permitted

Decision Date: 17/2/2015

CONSULTATIONS

No consultations required for this application.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

RELEVANT PLANNING POLICY AND GUIDANCE

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance September 2019

INTRODUCTIONA Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

xxx

The following elements of the proposals are considered under Class A:

- A single storey side extension, measuring 1.85m wide and 6.5m deep

- Extension of the height of the existing rear extension by 0.2m

- Render finish applied to new gable wall to the side

The following element of the proposals are considered under Class B:

- a hip to gable roof conversion,

- Rear dormer

The following elements of the proposals are considered under Class C:

- Three velux roof lights on the existing rear extension

- One new velux roof window to the rear slope

The following element of the proposal is considered under Class G:

- Removal of two chimneys

## ASSESSMENT UNDER CLASS A

The proposal must be assessed to ensure that it complies with the conditions Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘The enlargement, improvement or other alteration of a dwellinghouse’.

A.1 Development is not permitted by Class A if:

A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use) Complies

A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse) Complies

A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse Complies

A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse Complies –

A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; or (ii) fronts a highway and forms a side elevation of the original dwellinghouse Complies

A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, (ii) exceed 4 metres in height Complies

A.1 (g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or (ii) exceed 4 metres in height Complies

A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse N/A as single storey / Complies

A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres Complies

A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse  Complies

A.1 (k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse N/A

A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if:

A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles N/A as not on article 2 (3) land

A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse N/A as not on article 2 (3) land

A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse  N/A as not on article 2 (3) land

CONDITIONS UNDER CLASS A

Development is permitted by Class A subject to the following conditions:

A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse - Complies, as per Drawing No: 04 Rev

A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed Complies

A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse N/A

ASSESSMENT UNDER CLASSES B and C

The proposed hip to gable roof conversion with rear dormer and front / rear rooflights must be assessed to ensure that they comply with the relevant conditions of Class B and Class C Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘additions etc to the roof of a dwellinghouse’ and ‘other alterations to the roof of a dwellinghouse’ respectively.

The relevant conditions of the GPDO for a loft conversion with rear dormer which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than

(i) 40 cubic metres in the case of a terrace house:

(ii) 50 cubic metres in any other case

N/A

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, N/A / complies or

(ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; N/A / Complies

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies, as per Drawing No: 04 Rev A: Proposed Plans & Elevations

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, N/A / complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, N/A / complies

C.1 Development is not permitted by Class C if:

C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

C.1 (b) the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies (Rear and front rooflights are 0.11m)

C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

C.1 (d) it would consist of or include (i) the installation, alteration, or replacement of a chimney, flue or soil and vent pipe, complies or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment, N/A / complies

Conditions

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

C.2 (a) obscure-glazed; complies and

C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, complies

OVERALL RECOMMENDATION

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the single storey side extension, hip to gable roof extension, altered single storey rear extension to create flat roof, with increased height and three velux roof windows, one new velux roof window to the rear slope of the dwelling, one velux sun tunnel to the front roof slope, render to the side gable wall, removal of two existing chimney.

and window/door changes on west (side) and front (south) elevations.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Class B and Class C and G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). It can be confirmed that the proposed operations would be lawful for planning purposes on the basis of the following:

Drawing No: 04 Rev A: Proposed Plans & Elevations

Drawing No: 03 Rev A: Existing Plans & Elevations

Drawing No: 02 Rev A: Block Plan

Location Plan

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Construction and Demolition

Encroachment and Party Wall Act