| Ref. | Species | Location | Life Stage | Survey Notes | Condition | Inspect Period | Recommendations |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T004 | Pedunculate Oak (Quercus robur) | Residential garden and neighbouring gardens. , | Mature | Mature tree in corner of plot. Light ivy growth. Where accessible base clear. No fungal fruiting bodies evident or audible decay. Swept stem northeast. Asymmetrical canopy low over neighbour's shed. Significant Dieback in several branches in upper crown. Large low dead branches over garden of #263. | In decline (potential to recover) | 3 Years | Remove deadwood Reduce lateral spread on tips of low branch over shed back by up to 2-3m, graded to leave a natural looking canopy shape. Crown lift smaller secondary branches to create 3- 4m clearance above ground level, particularly in canopy over neighbouring shed. Timescale: 13-Mar-2023 (6 Months) |
| T008 | Common Ash (Fraxinus excelsior) | Residential garden and neighbouring garden. | Over Mature | The stems from 0.6m. Dense ivy growth. Where accessible no fungi or decay. Weak fork on south side with area of bark inclusion. Linear split on stem above which is a large, sweeping lateral branch growing out over neighbour's garden. Fused stems on north side of canopy. Major deadwood where foliage aborted due to lack of light. Rapid growth back cracks on underside of low lateral limbs. Weak fork in central stem approx. 6m above ground level. Swing on dead branch. | Poor | 3 Years | Remove swing (immediate). Remove deadwood. Reduce remaining canopy by around 6m and continue to maintain in a reduced form Timescale: 13-Mar-2023 (6 Months) |
| T012 | Pedunculate Oak (Quercus robur) | Residential garden | Early Mature | Young slender tree. Light ivy growth around trunk. Where accessible base satisfactory. Hammock tied around trunk. High asymmetrical canopy. Epicormic growth on trunk. Deadwood and low pendulous branch in canopy | Fair | 3 Years | Remove deadwood Timescale: 15-Mar-2023 (6 Months) |
| T014 | Pedunculate Oak (Quercus robur) | Residential garden | Early Mature | Base satisfactory. Well-formed buttress roots. Hammock tied around trunk. High asymmetrical with some larger pieces of deadwood in canopy. Reasonable foliage density and vitality | Fair | 3 Years | Remove deadwood Timescale: 15-Mar-2023 (6 Months) |
| T019 | Common Ash (Fraxinus excelsior) | Residential garden | Mature | Light ivy growth. Where visible, no significant defects around base. Single stem, no defects. High canopy. Few dead tips but not significant or obvious indication of Ash Dieback. Larger low branches dead having been aborted due to lack of light | Fair | 3 Years | Remove deadwood Timescale: 15-Mar-2023 (6 Months) |
| T020 | Hornbeam (Carpinus betulus) | Residential garden | Mature | The stems from 0.4m. bark inclusion in forks. Light ivy cover on lower trunks. Broad canopy. One or two dead but very small low branches. Foliage dense and even. Reasonable vitality. Large broken branch from another tree prepped up against trunk. | Fair | 3 Years | Remove dead propped branch Timescale: 15-Dec-2022 (3 Months) |
| T023 | Common Ash (Fraxinus excelsior) | Residential garden, neighbouring outbuilding | Mature | Base and trunk completely obscured by dense ivy growth. Linear cavity to 1.8m on southeast side of trunk where some wound wood is visible. Also cavity on north side at base where there may have been a second trunk. Stem hollow – metal probe can be inserted 300mm Neighbour has recently built a brick outbuilding within 200mm of base. Trunk obscured by ivy. High canopy. Dead low lateral branch on west side of canopy. Foliage even, minor dead twigs and some tip dieback. | Fair | 3 Years | Fell to high stump. Tree has significant structural defect and has been undermined by adjacent building works Timescale: 15-Dec-2022 (3 Months) |
| T024 | Pedunculate Oak (Quercus robur) | Residential garden, neighbouring outbuilding | Mature | Base and trunk completely obscured by dense ivy growth. Slender tree. Where visible no obvious defects on trunk. Asymmetrical leggy canopy with extended low branch and deadwood over neighbouring garden. Foliage dense and even | Fair | 3 Years | Sever ivy and inspect base and lower trunk. Remove deadwood Timescale: 15-Mar-2023 (6 Months) |
| T025 | Common Ash (Fraxinus excelsior) | Residential garden, neighbouring outbuilding | Mature | Base and lower trunk partially obscured by dense ivy growth which has been cut and is doing back. Minor bark wounds where ivy has been cut. Dieback of epicormic shoots from base. Swept lower trunk. Upper canopy resumes vertical growth. Broad canopy. Large pieces of deadwood from aborted shaded branches. Foliage even, minor dead twigs and some apical dieback. | Fair | 3 Years | Remove deadwood Timescale: 15-Mar-2023 (6 Months) |
| T026 | Pedunculate Oak (Quercus robur) |  | Semi Mature | Slender tree with high canopy. Ivy cut and mostly fallen from trunk. Base and trunk no obvious defects. Suppressed form with asymmetrical and slightly chlorotic foliage. Several small to medium sized pieces of deadwood over bin area | Fair | 3 Years | Remove deadwood Timescale: 15-Mar-2023 (6 Months) |
| T031 | Common Ash (Fraxinus excelsior) | Rear garden boundary tree | Early Mature | Previously three stems, two of which have been removed leaving stump at base. Central tree in linear boundary group. Canopy largely obscured by that of the adjacent beeches which are heavily reduced and managed as pollards. | Fair | 3 Years | Continue to maintain as a pollard Timescale: Ongoing |
| G036 | Mixed Species Group (Group, mixed species) | Residential garden | Young | Mixed species group but predominantly Hornbeam. Very young trees average 100mm diameter. Suppressed by density. Some bark damage on selective individuals. | Fair | 3 Years | Fell 7 trees in group with extensive stem wounding to enable other trees in group to succeed. Timescale: 15-Sep-2023 (1 Year) |