DELEGATED HOUSEHOLDER APPLICATION REPORT

Application No: 220038

Address: 122 Park Lane, Tilehurst, Reading RG31 4DU

Applicant: Mr Jack Mintern

Proposal: 2 storey side and rear extension to existing semi detached dwelling with loft conversion and addition of single storey front porch

Target date: 14/03/2022

Case officer: Marcelina Rejwerska

- Site Description and Surrounding Area:

The proposal site is a two storey end-terrace dwelling on Park Lane in Tilehurst. The property is set back from the road by way of a paved driveway. There is an existing lean-to on the side of the property and a single storey rear extension. Park Lane is a predominately residential road comprised of short terraces, semi-detached and detached dwellings which are similar in design. The property is not listed nor in a conservation area.

- Proposal:

Planning permission is sought for a two storey side and rear extension, loft conversion and addition of a single storey front porch.

Side and rear extensions

The proposed side extension will measure 2.29m in width, aligning with the existing front elevation of the main dwelling to accommodate a new entrance hall and utility room. The proposed extension wraps around the property to include a rear extension which will measure 5.2m in depth. The proposed extension at the rear reduces in width above the ground floor to 4.85m.

Loft conversion

The proposed loft conversion includes a hip to gable roof alteration, a new rooflight at the front elevation of the main dwelling, one new rooflight at the rear elevation and addition of a new full length window at the rear elevation.

Front porch

The proposed front porch includes reorientation of the front entrance to the dwelling with a canopy pitched roof above the entrance.

Plans Considered:

SRHA-0921-02-002-C Proposed Site Plan

SRHA-0921-02-003-E Proposed Floor Plans

SRHA-0921-02-004-D Proposed Elevations

Received 25/05/2022

SRHA-0921-01-001-A Location Plan

SRHA-0921-01-003-A Existing Plan

SRHA-0921-01-004-A Existing Elevations

SRHA-0921-01-002-A Existing Block Plan

Received 17/01/2022

- Relevant Planning History:

None.

- Consultation Responses:

Internal Consultees

None consulted.

External Consultation

The following addresses were formally consulted via letter on 17/01/2022:

120, 124 Park Lane

4 Glenwood Drive

2 objections received concerning potential overshadowing to the neighbouring property.

In addition, a site notice was displayed on site by the applicant.

- Discussions with applicant/agent

Officer requested amendment to reduce the size and scale of the side extension. The applicant requested the determination to be made on the application as submitted.

- Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

- Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

The following local and national planning policy and guidance is relevant to this application:

National Policy

National Planning Policy Framework

National Planning Policy Guidance

Reading Borough Local Plan (2019)

CC8 Safeguarding Amenity

CC7 Design and the Public Realm

H9 House Extensions and Ancillary Accommodation

H10 Private and Communal Outdoor Space

Design Guide to House Extensions Supplementary Planning Document (2021)

- Appraisal

Design and Character

The proposal is considered overbearing in terms of size and scale. The proposed side, rear and loft extension in sum would result in a structure that is visually dominant and disproportionately large for its setting in relation to the wider street. This would be contrary to Policy H9 which requires in accordance with policy H9 which requires extensions to “Respect the character and appearance of the house in terms of scale, location, materials and design”

Despite other properties within the immediate vicinity of the application site having side or rear extensions, the sum of the alterations proposed within this application would drastically change the character of the property, resulting in a visually dominant structure. The proposal would therefore appear out of character for Park Lane as a whole. This is contrary to Policy H9 which requires extensions to “Respect the character and pattern of neighbouring properties and appearance of the street as a whole in terms of scale, location, materials and design, the arrangement of doors, windows and other principal architectural features and any important existing building line.”

Safeguarding Amenity

This proposal has also been assessed against Reading Borough Local Plan policy CC8: Safeguarding Amenity, which states that proposals are acceptable where they will not cause a detrimental impact on the living environment of existing residential properties with regard to:

- Privacy and overlooking

  - Proposed new windows are not considered to create a risk of overlooking.

- Access to sunlight and daylight

  - The proposed side/rear extension will overshadow a side elevation window at the neighbouring 120 Park Lane. The application does not demonstrate how this risk will be mitigated.

- Visual dominance and overbearing effects of a development

- Harm to outlook

  - Proposal considered visually dominant due to overbearing size, therefore harmful to outlook of the wider street scene.

Private and Communal Outdoor Space

It is considered that this proposal will not represent an unacceptable loss in private outdoor space in accordance with Policy H10.

- Conclusion

The proposed works would result in extensions which are excessive in terms of its size and overly prominent due to clear views from the street. This arrangement would harm the character of the existing house and the appearance of the street scene, contrary to policies H9 and CC7. It is therefore recommended that this proposed development is refused planning permission.

RECOMMENDATION:

Refuse Planning Permission for the following reason:

- The proposed side/rear extension and loft conversion by reason of their excessive scale and prominent siting, would appear obtrusive within the street scene and would fail to respect the character of the existing house and the appearance of the street as a whole. As such the proposal is contrary to Policies CC7 and H9 of Reading Borough Local Plan (2019).

- The proposed side/rear extensions by reason of their scale and siting would overshadow habitable rooms at the neighbouring property, causing detrimental impact to their living conditions. This is contrary to policy H9 of Reading Borough Local Plan (2019).

Informatives

- The Local Planning Authority has worked positively and proactively with the applicant to see if relevant concerns could be resolved to allow permission to be granted. However, that has not been possible within the prescribed target or agreed extended timescale and therefore the decision to refuse planning permission has been taken.

2. This decision relates to the following submitted drawings:

SRHA-0921-02-002-C Proposed Site Plan

SRHA-0921-02-003-E Proposed Floor Plans

SRHA-0921-02-004-D Proposed Elevations

Received 25/05/2022

SRHA-0921-01-001-A Location Plan

SRHA-0921-01-003-A Existing Plan

SRHA-0921-01-004-A Existing Elevations

SRHA-0921-01-002-A Existing Block Plan

Received 17/01/2022