DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Coley

App Ref: 211991/HOU

Address: 40 Edenham Crescent, Reading, RG1 6HU

Proposals: Proposed 2 storey rear and side extension.

Case Officer: James Schofield

Date received (valid from): 10 December 2021      Target Decision date: 4 February 2022

# 1.	INTRODUCTION

The application relates to a modern two-storey semi-detached dwelling with a detached garage located to the side and rear of the dwelling. The dwelling is located on the western side of Edenham Crescent and backs on to the railway lines.

# 2.	PROPOSAL AND SUPPORTING INFORMATION

The proposed development is for a single-storey rear and two-storey side extensions. Plans considered;

Dwg No. P1276-01 Location & Block Plan

Dwg No. P1276-02 Existing Plans and Elevations

As received 10th of December 2021.

Dwg No. P1276-03 Rev C Proposed Plans and Elevations

As received 6th of June 2022.

# 3.	RELEVANT PLANNING HISTORY

N/A.

# 4.	CONSULTATIONS

## 4.1	Neighbours

A site notice was delivered to the agent and our records indicate it was displayed for 21 days. The following addresses were consulted by letter.

38 Edenham Crescent, Reading, RG1 6HU

42 Edenham Crescent, Reading, RG1 6HU

One letter of representation has been received objecting to the proposal. The grounds for objection principally relate to two concerns. The first is the alteration of the streetscene with the single detached garage being replaced by a side extension with a sloping roof which the objector believes to be detrimental to the character of the dwelling and wider area. This will be addressed in the body of the assessment. The second is the potential impact a reduction in the width of access to the rear garden could have on use of the site and potential future boundary disputes. This concern is noted, however, this is a potential legal dispute as opposed to a material consideration for the planning system.

5.	Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore, in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

# 6.	LEGAL AND PLANNING POLICY CONTEXT

# Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'. The following local and national planning policy and guidance is relevant to this application:

National Planning Policy

National Planning Policy Framework (2021)

National Planning Practice Guidance

Reading Borough Local Plan (adopted 2019)

CC7	Design and the Public Realm

CC8	Safeguarding Amenity

EN12	Biodiversity and the Green Network

H9	House Extensions and Ancillary Accommodation

H10	Private and Communal Outdoor Space

SPD

Design Guide to House Extensions SPD (adopted 2021)

# 7.	APPRAISAL

## 7.1 Principle of Development

## The application seeks permission for a part single part double-storey side and rear extension to an existing dwelling. Such domestic works are supported in principle by Policy H9 (House Extensions and Ancillary Accommodation) of the Local Plan (2019) subject to a number of criteria ensuring the effects of such extensions and alterations respect its context and are not harmful. These considerations will be examined in more detail in the following sections.

## Character, scale and siting

Local Policy CC7 states that, development must be of a high design quality that maintains and enhances the character and appearance of the area in Reading in which it is located. Policy H9 states that an extension will be acceptable where it: respects the character of the house, respects the pattern of neighbouring properties, and does not present a large blank façade to public areas.

The amended scheme design detail and materials appear to be in keeping with that of the original dwelling. The proposed depth of the rear extension adjacent to no.38 has been reduced to 5 metres but taken further off the boundary so while still longer than policy guidance of 4 metres in appearance would not be harmful to the character of the dwelling or the wider area. The side extension would be visible from the street, but not a demonstrable or harmful change from the visual impact of the existing detached garage. The proposal is set back and down from the main dwelling and would remain visually subordinate to the original.

As such, the proposal accords with the aims of policies CC7 and H9 of the Local Plan (2019) with respect to character, scale and siting.

## 7.3 Amenity

## Policy CC8 states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties. Policy H10 aims to ensure suitable outdoor private and communal outdoor amenity space is prioritised in development decision. The proposal includes single-storey side extensions. The SPD ‘Design Guide to House Extensions’ provides guidance on design considerations for single-storey rear extensions, which outlines that the design should not detract from the existing dwelling, not encroach on a line taken 45 degrees from the mid-point of the nearest neighbouring habitable room, should avoid reducing back-to-back distance for dwellings like these to less than 20 meters, and should not harm common building lines or positive repeating design features.

The extension would by virtue of its siting and scale not impede upon on any 45-degree line of sight from neighbouring windows. The rear extension would be at a depth of 5 meters at the boundary with number 38 Edenham Crescent and a depth of 7.4 meters near the boundary with number 42 Edenham Crescent. Number 42 Edenham Crescent has a detached garage closest to this boundary which would reduce the impact of the proposal. Siting and height (2.45 to eaves) would help to reduce significant overshadowing for neighbouring sites. Over half of the existing private outdoor amenity space would be maintained if the proposal were to be built. Only one new side facing window is proposed. This is proposed to serve an ensuite bathroom and is labelled to be obscure-glazed on the plans. As such, a condition requiring this window remain obscure glazed is considered to be sufficient to safeguard the privacy amenity of neighbouring sites.

Further to this analysis, the proposal is considered to accord with the aims of policy CC8 and H10 of the Local Plan (2019).

## 7.4	Biodiversity and sustainability

The site backs on to the railway line. Part ‘b’ of Policy EN12 indicates that all development sites should not result in a net loss of biodiversity or geodiversity. These aims are supported by Policy H9 which states that an extension will only be acceptable where it does not result in the loss of biodiversity within gardens.

The site does not have any notable vegetation within the proposal site, protected trees or identified Green Links which could be negatively impacted by the proposal.

The proposal is not considered likely to result in a net loss of biodiversity or geodiversity. As such the proposal is considered to accord with the biodiversity and sustainability considerations of Policies H9 and EN12 of the local plan.

# 8.	CONCLUSION

The proposed development has been assessed and the proposed works are in accordance with the above relevant policies and planning permission can be granted.

# 9.	RECOMMENDATION

GRANT

# 10.	CONDITIONS

- Standard time limit (TL1)

- Standard approved plans (AP1)

- Materials to match (M1)

- Obscure-glazed (PD3)