DELEGATED APPLICATION REPORT			                         Target Date: 19/07/21

Application No: 210760/FUL

Address: 73 Norfolk Road

Proposal: Various internal and external alterations including conversion of ground floor retail shop to 1x2 bed flat, single storey rear extension and loft conversion with provision of L-shaped rear box dormer and roof lights

Case Officer: Tom Hughes

Site Description and Surrounding Area

The application site comprises a two storey mid-terraced property, vacant at the time of the application. The pre-existing use of the ground floor is a retail shop, with a 2x1 bed flat located across the ground and first floors. The property benefits from a two storey rear gable, and single storey rear extensions. The surrounding area is residential in character. The site topography slopes to towards Kensington Park, located to the rear of the site.

Proposal

The proposal is for internal and external alterations, to facilitate the conversion of the property to 2x2 bed flats. The ground floor would comprise one flat, with the existing flat altered to extend across the first and second floors. The second floor would be facilitated by the provision of an L-shaped rear box dormer and the provision of roof lights. The existing shop front would be replaced by two separate entrance doors. The existing rear extension would be partially demolished, replaced by a new full-width single storey rear extension, projecting to an additional depth of 3.94m.

Plans Considered

The agent supplied the following, received 14/05/21:

Drawing No: 21/185/05 – Site Location and Block Plan

Drawing No: 21/185/01 – Existing Floor Plans

Drawing No: 21/185/02 Rev. A – Proposed Ground and First Floor Plans

Drawing No: 21/185/03 Rev. A – Proposed Second Floor Plan

Drawing No: 21/185.04 – Proposed Elevations

Subsequently the agent supplied the following, received 24/05/21:

Design and Access Statement

Relevant Planning History

900246/FUL – Single storey side and rear extension – Permitted 12/10/90

930601/FUL – Rear conservatory – Permitted 16/09/93

Consultation Responses

Neighbouring owners and occupiers at 60, 62, 64, 71 and 75 Norfolk Road and were consulted by letter. A site notices was displayed. One letter of representation was received, with regard to the following:

- Proposals would exacerbate existing parking pressures

Reading Borough Transport Development Control

Raised no objections to the proposal, on the condition of future occupants not being eligible for on-street parking permits. Conditions regarding bicycle and waste storage were recommended.

Reading Borough Ecology

Raised no objections to the proposal, as the proposals would be unlikely to adversely affect protected species or priority habitats.

Equality Act 2010

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Plan (2019)

Policy CC1 – Presumption in Favour of Sustainable Development

Policy CC2 – Sustainable Design and Construction

Policy CC3 – Adaption to Climate Change

Policy CC5 – Waste Minimisation and Storage

Policy CC6 – Accessibility and the Intensity of Development

Policy CC7 – Design and the Public Realm

Policy CC8 – Safeguarding Amenity

Policy EN12 – Biodiversity and the Green Network

Policy EN18 – Flooding and Drainage

Policy H1 – Provision of Housing

Policy H2 – Density and Mix

Policy H3 – Affordable Housing

Policy H5 – Standards for New Housing

Policy H8 – Residential Conversions

Policy H9 – House Extensions and Ancillary Accommodation

Policy H10 – Private and Communal Outdoor Space

Policy TR3 – Access, Traffic and Highway-Related Matters

Policy TR5 – Car and Cycle Parking and Electric Vehicle Charging

Affordable Housing SPD (2021)

Revised Parking Standards and Design SPD (2011)

Residential Conversions SPD (2013)

Design Guide to House Extensions SPD (2021)

Appraisal

Principal of change of use

While the loss of a retail shop is regrettable, the site is not located within a designated Centre, where retail uses receive protection from some types of change of use. The shop has been vacant for some time, and at the time of the application was not positively contributing to the character of the area. The provision of an additional unit of residential accommodation in the Borough also helps to meet an identified need for housing. Subject to other policy considerations outlined below, the principal of the change of use of the ground floor from a retail shop to a flat is acceptable.

Design and impact on the character of the surrounding area

The existing building benefits from sizeable single storey rear extensions and a predominantly glazed shopfront. To facilitate the proposals, a further single storey extension would be erected, the loft be converted and extended via an L-shaped rear box dormer, and the shop front be replaced by individual entrances for the flats.

As a result of the proposed extension, the building would be extended to the rear by a total depth in excess of 10m, almost doubling the footprint of the original building. Considered cumulatively with the large dormer extension, the proposals constitute a significant overdevelopment of the original building. Both elements appear over-scaled, lacking subservience to the original building. The additional bulk and mass afforded by the design of the dormer creates a contrived and awkward roofscape, which has a jarring appearance when viewed alongside the existing roofscape, detracting from the buildings original character. The proposals are of poor design, contrary to Policies CC7 and H8 of the Local Plan.

Impact on neighbouring amenity

Due to its height and proximity to the shared boundaries, significant loss of privacy through overlooking would occur to the rear of 71 and 75 Norfolk Road from the proposed dormer window. By virtue of its depth, height and proximity to the boundary, the proposed rear extension would cause a significant detrimental impact to the residential amenities of 71 Norfolk Road in terms of loss of light, visual dominance and overbearing effects. The proposals are contrary to Policy CC8 of the Local Plan on this basis.

Affordable housing

Policy H3 requires that on sites of 1-4 dwellings, a financial contribution will be made that will enable the equivalent of 10% of the housing to be provided as affordable housing elsewhere in the Borough. No affordable housing contribution has been proposed. Not viability information has been supplied. In the absence of a completed legal agreement to secure affordable housing, the proposal fails to contribute adequately to the housing needs of the Borough. The proposal is therefore contrary to Policy H3 of the Local Plan and the Affordable Housing SPD.

Amenity of future occupiers

Each of the proposed flats are of a suitable size, and all habitable rooms benefit from sufficient natural light and outlook. The proposals would afford an acceptable standard of accommodation for future occupiers, in accordance with Policy H8.

Transport

Given the proximate location of the site to the town centre, and the availability of local public transport routes, a car-free development can be accepted. Were permission to be granted, the applicant would be advised that future occupants would not be eligible to apply for on-street parking permits.

Policy H8 requires that bin and cycle storage of an appropriate size and standard for the units proposed be provided. In the absence of any details of bin or cycle storage, the proposal is contrary to Policy H8.

Conclusion

The proposal has been assessed and found to be unacceptable as it is contrary to the identified policy and guidance.

RECOMMENDATION

REFUSE