Delegated Application Report 				              Target Date: 10/12/21

Address: 4 Ivanhoe Close

Application No: 211666/CLP

Proposal: Use as a 5 bed, 6 person occupancy, house in multiple occupation (Class C4), and erection of rear conservatory

Case Officer: Tom Hughes

Introduction and Proposal

4 Ivanhoe Close is a three storey townhouse (Class C3), part of a modern residential development to the rear of properties on Western Elms Avenue. The building is not listed, nor is it located in a conservation area. The site is not located within an Article 4 area.

The proposal is for the change of use of the property from a dwellinghouse to a 5 bed, 6 person occupancy house in multiple occupation (Class C4). The proposal also includes the erection of a conservatory to the rear.

Plans Submitted

The agent supplied the following, received on 11/10/21:

Supporting Statement

Drawing No: 02-00 – Site Location

Drawing No: 02-02 – Existing Site Plan

Drawing No: 02-07 – Proposed Site Plan

Drawing No: 02-02 – Existing Ground Floor Plan

Drawing No: 02-03 – Existing First Floor Plan

Drawing No: 02-04 – Existing Second Floor Plan

Drawing No: 03-00 – Proposed Ground Floor Plan

Drawing No: 03-01 – Proposed First Floor Plan

Drawing No: 03-02 – Proposed Second Floor Plan

Drawing No: 02-05 – Existing Front and Rear Elevations

Drawing No: 02-05 – Existing Side Elevation

Drawing No: 04-00 – Proposed Front and Rear Elevations

Drawing No: 04-00 – Proposed Side Elevations

Supporting Information

Relevant Planning History

990631/FUL – Erection of 1x3 storey block of 21no. apartments and associated works, following demolition of 15 Western Elms Avenue – Refused 02/05/01; APP/E0345/A/01/1068233 – Dismissed 30/10/01

990275/FUL - Erection of 1x3 storey block of 21no. apartments and associated works, following demolition of 15 Western Elms Avenue – Refused 06/02/02

101418/FUL – Demolition of house and garage, formation of new access and erection of 1no. replacement house and 5no. new houses – Refused 17/09/10; APP/E0345/A/10/2138413/NW – Allowed 04/05/11

140918/FUL – Retain existing house (15 Western Elms Avenue), delete approved 4 bed house and garage on Plot 6, relocate access road and erect 5no. houses with parking (as per approval 101418/FUL) – Permitted 02/10/14

141893/APC – Discharge of conditions 3, 4, 6, 7, 8 and 9 of planning permission 140918/FUL – Discharged 22/01/15

152124/NMA – Non-material amendment to 140918/FUL – Agreed 18/12/15

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order (as amended)

Permitted development rights for householders – Technical Guidance (2019)

Assessment

A Certificate of Lawfulness for a proposed use or development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed use as a 5 bed, 6 person occupancy, house in multiple occupation (Class C4) must be assessed to ensure that it complies with the conditions of Class L, Part 3, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘small HMOs to dwellinghouses and vice versa’.

The proposed rear conservatory must be assessed to ensure that it complies with the conditions of Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘enlargement, improvement or other alteration of a dwellinghouse’.

Use as a 5 bed, 6 person occupancy, house in multiple occupation (Class C4)

Permitted development

L. Development consisting of a change of use of a building-

(a) from a use falling within Class C4 (houses in multiple occupation) of the Schedule to the Use Classes Order, to a use falling within Class C3 (dwellinghouses) of that Schedule;

(b) from a use falling within Class C3 (dwellinghouses) of the Schedule to the Use Classes Order, to a use falling within Class C4 (houses in multiple occupation) of that Schedule.

Development not permitted

L.1 Development is not permitted by Class L if it would result in the use-

(a) as two or more separate dwellinghouses falling within Class C3 (dwellinghouses) of the Schedule to the Use Classes Order of any building previously used as a single dwellinghouse falling within Class C4 (houses in multiple occupation) of that Schedule; complies, it would not

(b) as two or more separate dwellinghouses falling within Class C4 (houses in multiple occupation) of that Schedule of any building previously used as a single dwellinghouse falling within Class C3 (dwellinghouses) of that Schedule. complies, it would not

Rear conservatory

A.1 Development is not permitted by Class A if:

A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies, it has not

A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); complies, it would not

A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; complies, it would not

A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; complies, it would not

A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principle elevation of the original dwellinghouse; complies, it would not or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; complies, it would not

A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, complies, it would not or (ii) exceed 4 metres in height; complies, it would not

A.1 (g) until 30th May 2019, for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, complies or (ii) exceed 4 metres in height; complies

A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, complies or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse; complies

A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres; complies

A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, complies (ii) have more than a single storey, complies or (iii) have a width greater than half the width of the original dwellinghouse; complies

A.1 (ja) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (e) to (j);

A.1 (k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, complies (ii) the installation, alteration or replacement of a microwave antenna, complies (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, complies or (iv) an alteration to any part of the roof of the dwellinghouse. complies

A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if:

A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles; n/a

A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse; n/a

A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse; n/a

A.2 (d) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (b) and (c). n/a

Conditions

Development is permitted by Class A subject to the following conditions:

A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse; complies

A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, n/a and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; n/a and

A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. n/a

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended), Class L, Part 3, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, and Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.

It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 11/10/21:

Drawing No: 02-07 – Proposed Site Plan

Drawing No: 03-00 – Proposed Ground Floor Plan

Drawing No: 03-01 – Proposed First Floor Plan

Drawing No: 03-02 – Proposed Second Floor Plan

Drawing No: 04-00 – Proposed Front and Rear Elevations

Drawing No: 04-00 – Proposed Side Elevations

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed use as a 5 bed, 6 person occupancy, house in multiple occupation (Class C4), and the erection of a rear conservatory.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

HMO Management