Ward: Norcot

App Ref: 201513/FUL

Address: The Flat, 615 Oxford Road, Reading, RG30 1HL

Proposals: Second storey rear extension and change of use from C4 HMO to 3 flats (1 x 2-bedroom, 2 x studio)

Case Officer: Claire Ringwood

Applicant: Mr Abbas

Date received (valid from): 28th October 2020

Target Decision date (extension of time): 29th March 2022

INTRODUCTION

The site this enquiry relates to is the land and buildings associated with 615 Oxford Road, Reading. The site contains a ground floor A1 (barbers), with an access to the HMO above (and the basement storage below) from Beecham Road. The building is a three storey end of terrace property (excluding the basement) which includes a two storey rear extension.

It has not been possible to visit the site due to the current pandemic however Google Maps and Google Streetview enabled Officers to view the building externally and following comments from the previous planning officer and their delegated report along with the information and plans submitted by the agent has allowed the application to be fully assessed.

PROPOSAL

The proposal is for a change of use from a HMO split over first and second floor to 3 flats (2 x studios and 1 x 2 bedroom). The proposed development would require a second floor rear extension (over the top of the existing extension). The development also includes bicycle parking and bin storage at ground floor (accessed from Beecham Road).

PLANS SUBMITTED

The agent submitted the following plans, received on 28th October 2020:

Drawing No: P101 – Location Plan

Drawing No: P200 – Existing & Proposed Block Plan

Drawing No: P201 – Existing Plans, Elevations and Section AA

Drawing No: P202 – Proposed Plans, Elevations and Section AA

The agent submitted the following amended plans, received on 4th May 2021:

Drawing No: P202 Rev B – Proposed Plans, Elevations and Section AA

RELEVANT POLICIES AND SUPPLEMENTARY PLANNING GUIDANCE

Legal and Planning Policy Context

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Reading Borough Local Plan (2019)) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).  The Council's adopted Supplementary Planning Guidance and Supplementary Planning Documents are also material planning considerations.

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

Reading Borough Council Local Plan (2019) (hereafter “the local plan”):

CC6: Accessibility and The Intensity of Development

CC7: Design and The Public Realm

CC8: Safeguarding Amenity

CC9: Securing Infrastructure

EN12: Biodiversity and The Green Network

EN15: Air Quality

EN16: Pollution and Water Resources

H1; Provision of Housing

H2: Density and Mix

H3: Affordable Housing

H5: Standards for New Housing

H8: Residential Conversions

H9: House Extensions and Ancillary Accommodation

H10: Private And Communal Outdoor Space

TR3: Access, Traffic And Highway-Related Matters

TR5: Car And Cycle Parking And Electric Vehicle Charging

SUPPLEMENTARY PLANNING GUIDANCE

SPG - Planning Obligations under s.106 (2015)

SPD - Sustainable Design and Construction (2019)

SPD – Revised Parking Standards and Design SPD (2011)

SPD – Affordable Housing (2021)

SPD- Residential Conversions (2013)

RESPONSE TO PUBLICITY (REPRESENTATIONS RECEIVED)

Letters were sent to neighbouring residents at 1a Beecham Road, 613, 615, 615B Oxford Road.  No representations have been received at the time of writing.

CONSULTATION RESPONSES

Environmental Protection: No objection subject to a condition relating to hours of working.

Ecologist: No objections.

Transport: Objection due to the non-provision of on-site parking.

PLANNING HISTORY

09/00098/PREAPP (090918) - Pre-application advice for change of use of existing A1 unit to mixed use comprising A1, A3 and D1 use ISSUED 3 March 2009

171920/PRE - Pre-application advice for the conversion of the existing dwelling into 8 flats ISSUED 5 January 2018

180651/PRE - Follow up advice following pre-application 171920 now amended to 3 bedsits and a one bedroom flat ISSUED 1 May 2019

191458/PREAPP – Pre-application advice for second storey rear extension and change of use from C4 HMO to 4 flats (2 x 1 bedroom, 2 x 2 bedroom) at 615 Oxford Road. ISSUED 29 November 2019

200136/FUL - Second storey rear extension and Change of use from C4 HMO to 4 flats (1 x 2 bedroom, 3 x studio).  Refused 08/04/2020 and dismissed at appeal 06/10/2020.

This application also involved converting the basement into a studio flat and constructing a lightwell to the front.  The reasons for refusal were:

- Covered lightwell would be an unusual addition to the streetscene and considered harmful to the character and appearance of the area;

- Insufficient light and no outlook to the basement flat;

- Failure to demonstrate parking;

- Absence of a completed legal agreement to secure an affordable housing contribution.

The Planning Inspector dismissed the appeal and found there was no harm to the character and appearance of the area although the proposal failed to provide the required contribution to affordable housing, would harm the living conditions of future occupiers and would not provide adequate provision for parking.

SUMMARY OF DISCUSSIONS WITH APPLICANT/AGENT

Discussions have been undertaken with regards to ensuring appropriate cycle and bin storage is proposed and to retain bin storage for the ground floor shop.  Amended plans were submitted on 4th May 2021.

APPRAISAL

The main issues raised by the application are:

- Principle of development

- Residential amenity

- Impact on the character and appearance of the area

- Mix of dwellings

- Transport matters

- Sustainability

- Other matters including S106

- Principle of development

The NPPF (Para. 61) states that “the size, type and tenure of housing needed for different groups in the community should be assessed and reflected in planning policies”, it also encourage efficient use of land taking into account: identified need for housing, local market conditions, the availability and capacity of infrastructure, the desirability of maintaining the existing character of the area, and the importance of securing well-designed, attractive, and healthy places. The Local Plan supports the provision of additional dwelling units to meet the requirement for additional dwelling per annum. Policy H8 (Residential conversions) states that “proposals to convert properties into self-contained flats … will be assessed against the impact on the amenity and character of the surrounding area”.

Policy H2 refers to the appropriate mix of residential development. This is assessed on a site by site basis taking into account the characteristics of the area in which it is located. The site is located on previously developed land and already in use as an HMO. As such the principle of the proposed development could be acceptable subject to the following material planning considerations.

The proposal would accord with Policy H8 and the Residential Conversions SPD in that the existing dwelling has 4 or more bedrooms and has a floor space greater than 120m2.

At least one unit of accommodation created would be suitable for family occupation with a minimum of two bedrooms, the proposed 1 x 2-bed and 2 x studios are considered acceptable.

Moreover, the provision of two additional residential units will assist in the Council meeting its annual housing targets (policy H1).

- Residential Amenity

Notwithstanding section 1 above, the local plan includes a number of policies which seek to ensure that all new developments will not cause a detrimental impact on the living environment of existing residential properties or unacceptable living conditions for new residential properties.

Specifically, Policy CC8 states that new development should consider:

- Privacy and overlooking;

- Access to sunlight and daylight;

- Visual dominance and overbearing effects of a development;

- Harm to outlook;

- Noise and disturbance;

- Artificial lighting;

- Vibration;

- Dust and fumes;

- Smell;

- Crime and safety; or

- Wind, where the proposals involve new development of more than 8 storeys.

In addition, policy H5 states that:

“New build housing should be built to the following standards, unless it can be clearly demonstrated that this would render a development unviable:

a. All new build housing outside the Central Area as defined on the Proposals Map will comply with the nationally-described space standard …”

Policy H8 states that “proposals to convert properties into self-contained flats or for multiple occupation will only be acceptable where:  …. There are no unacceptable adverse impacts to residents of the scheme or surrounding properties arising from noise and disturbance in terms of the number and layout of units proposed and the proximity to other properties;  There is no inappropriate stacking and location of rooms between units;  ……….; and   The resulting property or properties would provide adequate internal floorspace and headroom for residents.”

As such, it is clear that all new dwellings should comply with the above mentioned standards/requirements.

Floor space requirements

Policy H5 requires all development to comply with internal space standards as outlined within the local plan. The first floor flats, would be 36m2 which is just less than the required space for a 1-bed flat (37sqm including a shower room). In this particular instance, the proposed development allows for rationally shaped flats which would maximise the area within these two flats, and would allow a sufficient level of internal space based on single occupancy. The plans indicate an internal layout with space for a single bed, although the Council would be unable to ensure single occupancy of these units, it is highly unlikely that these flats would be suitable for 2-persons and as such would be acceptable.

The second floor flat, includes areas of head heights less than 2.3m, however 75% of the flat is proposed to exceed 2.3m, and as such is acceptable in this regard. The floor space within this 2-bed flat is considered acceptable.

Other amenity issues

Stacking

Policy CC8 above, also includes comment on the stacking of rooms within  residential uses and the effect on amenity. The proposed two-bed flat at second floor would be above the 2 x studio flats at first floor. Due to the nature of the studio flats it is not possible to arrange bedrooms above/below bedrooms and living/dining rooms above/below living/dining rooms.  Although this is unfortunate the Planning Inspector did not find harm in the living conditions of future occupiers when considering the previous appeal and therefore this arrangement alone would not warrant the refusal of the application.

First and second floor flats

In this instance, the proposed second floor rear extension has been assessed in relation to outlook, and overbearing impacts on neighbouring properties. The proposed second floor rear extension would sit above the existing first floor rear extension. At eaves level, the proposed extension would be no higher than the main roof, and would not be significantly higher than the ridge height of the existing extension. There are no windows from the adjoining properties that would have direct outlook in the direction of the proposed works. In addition, the rear of the adjoining properties are generally built out with single storey rear extensions, limiting the views from rear gardens etc. Therefore, the development would not have any overbearing impacts on adjoining properties. As such the proposal is considered acceptable in this regard.

The proposed development would include windows to the rear elevation at first and second floor level.  These windows are secondary windows to the rooms and will be conditioned to be obscure glazed in order to protect the privacy of neighbouring properties.  There are two existing side windows directly on the boundary with 613 Oxford Road and these are to remain and as these windows face on to the roof of the neighbouring property is it not considered that these need to be obscure glazed.

In relation to daylight/sunlight, the proposed first floor studios are each served by 3 windows for lighting.  Due to the open plan nature, it is considered that these would allow sufficient daylight/sunlight into these flats. The second floor flat, is also considered to be sufficiently served by windows for daylight/sunlight.

Noise and Air Quality

The application site is within a Air Quality Management Area and located along the busy Oxford Road.  Environmental Protection raised no objection as the proposal is for a change of use however they did recommend a condition restricting the hours of work.  Notwithstanding this other than the front facing studio the bedrooms are to the rear of the site away from Oxford Road and the proposed use is not considered to intensify the existing use as residential.

As such, the proposed units are considered acceptable and are in accordance with Policies CC8, EN15 and EN16.

- Impact on the character and appearance of the area

The proposed development includes a number of external alterations, most notably a second floor rear extension above the existing single two-storey rear extension. Policy CC7 states that “all development must be of high design quality that maintains and enhances the character and appearance of the area of Reading in which it is located. The various components of development form”.

The host dwelling to which the application enquiry relates is a unique building within the vicinity, as it is taller than the two-storey buildings which are a feature along Oxford Road and Beecham Road. Two-storey rear extensions are common along the adjoining terrace and generally follow guidance contained within Policy H9 (House Extension and Ancillary Accommodation) and the associated SPD. Policy H9 requires extensions to dwellings to:

  - Respects the character and appearance of the house in terms of scale, location, materials and design;

  - Respects the character and pattern of neighbouring properties and appearance of the street as a whole in terms of scale, location, materials and design, the arrangement of doors, windows and other principal architectural features and any important existing building line;

  - Does not present an overbearing impact on neighbours or a large blank façade to public areas; and

  - Does not result in a loss of biodiversity within gardens.

The proposed rear extension in this instance, would respect the building, to which it would be attached, in terms of scale and appearance in appearing subservient to it and in accordance with Policy CC7.

4. 	Mix of Dwellings

The application site appears to be currently in use as a HMO. The proposed development would result in the loss of a large unit of residential accommodation. As such, it would be required to make provision within the development for a 2-bed ‘family unit’. The proposed development would result in one x 2-bed unit and is likely to be acceptable in this regard.

5. 	Transport matters

In accordance with the current parking standards the HMO would generate a parking provision of 2 parking spaces for the 6 rooms, the proposed 3 new units would require 4.5 parking spaces.  Transport advised the under provision of parking would result in additional competition for on street parking.  The Planning Inspector also found the previously refused application did not provide adequate provision for car parking.

There is no parking for the existing HMO at the site and there is no other possible way of adding parking at the site therefore officers consider that stipulating a parking provision renders any future application unacceptable.  Officers are mindful of the Planning Inspector’s decision, however, it is felt that a no parking scheme can be accepted in this instance as the site is in a very accessible area close to bus routes, walking distance to and from the town centre and close to Reading West train station.  Beecham Road is currently unrestricted, however, there are plans to extend the Residential Parking Permit Scheme in to this area.  Although this is still in the early stages, conditions will be imposed which will restrict the entitlement to parking permits should a residents parking permit scheme be introduced in the area.

The rear of the ground floor is accessed from Beecham Road and is currently used as the refuse store.  The proposal separates this area so that it has a cycle store for 6 spaces, an area for the bins for the proposed units and the retention of an area for bin storage for the A1 unit.  The cycle store can be easily accessed and the bin arrangement will remain as existing.  As such this is considered acceptable and in accordance with the Council’s adopted standards (as contained within the SPD). These elements will be secured by condition.

6.	Sustainability

Policy CS1 and the Revised Sustainable Design and Construction SPD (2019) requires new development and conversions to comply with BREEAM conversion standards, and optional building regulations requirements. Conditions are included to secure the appropriate level of sustainable construction.

7.	Other matters including section 106

S106 Financial Contributions to Affordable Housing

The Council’s adopted Policy H3: Affordable Housing states “on sites of 1 – 4 dwellings, a financial contribution will be made that will enable the equivalent of 10% of the housing to be provided as affordable housing elsewhere in the Borough.” The applicant has agreed to a financial contribution of £11,875 and has entered into a Section 106 legal agreement on this basis.

CIL

The proposal would be a Community Infrastructure Levy (CIL) liable development. The applicant has provided the appropriate CIL Additional Information Form, and refers in the submitted DAS to self build exemption. An informative would be attached to the decision notice to advise the applicant of their responsibilities in this respect.

Equalities Act

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010.  The key equalities protected characteristics include age, disability, gender, gender reassignment, marriage and civil partnership, pregnancy and maternity, race, religion or belief, sexual orientation.  There is no indication or evidence (including from consultation on the application) that the protected groups have or will have different needs, experiences, issues and priorities in relation to the particular planning application.  In terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

RECOMMENDATION

Grant subject to S106 legal agreement.

Conditions

- Time Limit – 3 years

- Materials (to match existing)

- AP1 – Plans

- Rear windows obscure glazed and high opening

- DC8 - Bin storage in accordance with approved details

- DC5 - Cycle storage in accordance with approved details

- Hours of construction and demolition

- DE2 No parking permits

- DE3 No parking permits

- BREEAM – Interim

- BREEAM – Post Construction

Informatives

IF5 Terms and Conditions

I11 - CIL

IF4 - S106

IF3 – Highways

IF6 – Building Control

IF7 – Complaints about construction

IF8 – Encroachment

I10 – Noise between residential properties

IF1 – Positive and Proactive