DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Caversham

App Ref: 211933/HPA

Address: 5 Kings Road, Caversham, Reading, RG4 8DS

Proposals: Rear extension measuring 6.0m in depth, with a maximum height of 3.8m, and 3.m in height to eaves level. Notification of the construction of an extension under class A Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.

Case Officer: Chukwudi Onwudinanti

Applicant: Mrs S Vasiljeva

Date received (valid from): 30 November 2021

Target Decision date: 14 February 2022

INTRODUCTION

The proposal site is in a mid-terrace dwelling, located on the southern side of Kings Road. The surrounding area is residential, characterised by rows of terraced buildings of similar style and scale.

PROPOSAL AND SUPPORTING INFORMATION

Notification of the construction of an extension under class A Part 1 of Schedule 2 of the     Town and Country Planning (General Permitted Development) (England) Order 2015.

The proposal is for a rear extension measuring 6.0m in depth and 3.7m wide, with a maximum height of 3.1m, and 2.8m in height to eaves level. Materials are to match existing.

Plans considered (Received 30th September 2021)

PL-01- Location Plan

PL-03- Existing and Proposed Block Plan

PL-05 – Existing Plans

PL-06 – Existing Elevations

PL-07 – Proposed Plans

PL-08 Rev A – Proposed Elevations

PL-09- Proposed Sections

CONSULTATIONS

The following addresses were consulted by letter.

3 Kings Road, Caversham, Reading, RG4 8DS

7 Kings Road, Caversham, Reading, RG4 8DS

Flat 3, 9 George Street, Caversham, Reading, RG4 8DH

Flat 2, 9 George Street, Caversham, Reading, RG4 8DH

Flat 1, 9 George Street, Caversham, Reading, RG4 8DH

2 Queens Road, Caversham, Reading, RG4 8DL

No letters of representation have been received.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance September 2019

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘The enlargement, improvement or other alteration of a dwellinghouse’.

A.1 Development is not permitted by Class A if:

A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use) - complies

A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse) - complies

A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse - complies

A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse - complies

A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; or (ii) fronts a highway and forms a side elevation of the original dwellinghouse - complies

A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, (ii) exceed 4 metres in height N/A

A.1 (g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or (ii) exceed 4 metres in height - complies

A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse N/A

A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres – complies, the eaves are at 2.8m height.

A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse N/A

A.1 (k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse N/A

A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if:

A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles N/A

A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse N/A

A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse N/A

Conditions

Development is permitted by Class A subject to the following conditions:

A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse - complies

A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. N/A

A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. N/A

Equality Act

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010. The key equalities protected characteristics include age, disability, gender, gender reassignment, marriage and civil partnership, pregnancy and maternity, race, religion or belief, sexual orientation.  It is considered that there is no indication or evidence (including from consultation on the current applications) that the protected groups have or will have different needs, experiences, issues and priorities in relation to this particular planning application.

Conclusion

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposal to convert an existing single garage into a habitable room.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) and it has been confirmed that the proposal would be lawful for planning purposes on the basis of the following drawings and information received 30th September 2021:

PL-01- Location Plan

PL-07 – Proposed Plans

PL-08 Rev A – Proposed Elevations

PL-09- Proposed Sections

RECOMMENDATION: GRANT CERTIFICATE