DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Caversham

App Ref: 211920/CLP

Address: 5 Kings Road, Caversham, Reading, RG4 8DS

Proposals: Certificate of Lawfulness for proposed loft conversion with two rooflights to the front and an L shaped rear dormer.

Case Officer: Chukwudi Onwudinanti

Applicant: Mrs S Vasiljeva

Date received (valid from): 26 November 2021

Target Decision date: 14 February 2022

INTRODUCTION

The proposal site is in a mid-terrace dwelling, located on the southern side of Kings Road. The surrounding area is residential, characterised by rows of terraced buildings of similar style and scale.

PROPOSAL AND SUPPORTING INFORMATION

The proposal is for the conversion of existing loft space into a rear box dormer, and insertion of two roof lights to the front roof slope. The dormer measures in one part, 3.6m by 5.4m and in the second part 2.3m by 3.0m. It has a height of about 1.9m in all parts, with a flat roof cover. The dormer - The two front roof lights do not project from the slope of the existing roof.

Plans considered

PL-01: Location Plan

PL-03: Existing and Proposed Block Plan

PL-05: Existing Plans

PL-06: Existing Elevations

PL-07: Proposed Plan

PL-08: Proposed Elevations

PL-09: Proposed Sections

RELEVANT PLANNING HISTORY

None

CONSULTATIONS

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance September 2019

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed loft conversion with rear dormer and front rooflights must be assessed to ensure that it complies with the conditions ofsss Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘additions etc to the roof of a dwellinghouse’ and ‘other alterations to the roof of a dwellinghouse’ respectively.

The relevant conditions of the GPDO for a loft conversion with rear dormer which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, complies, the cubic content of the resulting roof space would exceed the cubic content of the original roof space by 20.73 cubic metres. or (ii) 50 cubic metres in any other case Not Applicable

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies or

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

C.1 Development is not permitted by Class C if:

C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

C.1 (b) the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies

C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

C.1 (d) it would consist of or include (i) the installation, alteration, or replacement of a chimney, flue or soil and vent pipe, complies or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment, complies

Conditions

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

C.2 (a) obscure-glazed;  complies and

C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, complies

Recommendation

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed loft conversion with rear dormer and two rooflights on the front roofslope.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the following received on 26th November 2021:

Drawing No. PL-01: Location Plan

Drawing No. PL-07: Proposed Plan

Drawing No. PL-08: Proposed Elevations

Drawing No. PL-09: Proposed Sections

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives:

Building Regulations

Construction and demolition

Encroachment and Party Wall
Terms

Positive and proactive Working

Location Plan (not to scale)