DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Minster

App Ref: 210633 / HOU

Address: 11 Downshire Square, Reading, RG1 6NJ

Proposals: Remodel of second floor and proposed rear dormer

Case Officer: Beatrice Malama

Date received (valid from): 22nd April 2021      Target date: 17th June 2021

SITE DESCRIPTION

The site is a three story semidetached house located in a predominantly residential neighbourhood comprising of detached and semidetached of varying style, design and age. Also located within the neighbourhood are facilities such as schools and churches. The property is not listed however it falls within a conservation area and Air Quality Management Area.

PROPOSED DEVELOPMENT

The proposed development involves second floor alterations and rear dormer extension. The proposed second floor remodelling would involve conversion of three bedrooms and storage into an ensuite bedroom. The proposed rear dormer which replaces the existing smaller dormer measures approximately 3.5m deep, approximately 2.8m high and approximately 6.2m wide. The proposed dormer extension would be set down from the ridgeline of the existing house and set back from the eaves of the existing house by approximately 0.30m. Exterior finishes are designed to complement the original house.

Plans considered;

Drawing No: 001 – Location Plan

Drawing No: 002 – Block Plan

Drawing No: 003 – Site Plan

Drawing No: 010 – Existing Plans and Elevations

Drawing No: 012 – Proposed Plans and Elevations

As received on 22nd April 2021

RELEVANT PLANNING HISTORY

| Application No | Description | Decision | Date |
| --- | --- | --- | --- |
| 050735 | Demolition of existing conservatory. Erection of new single storey rear extension to form new garden room/dining room (Resubmission of 04/01220/FUL) | Determination made | 16/08/2005 |
| 041309 | Demolition of existing conservatory. Erection of new single storey rea r extension to form new garden room/dining room | Determination made | 3/12/2004 |
| 040981 | Demolition of exisiting conservatory and erection of new single storey rear extension to form new garden room/dining room | Determination made | 18/08/2004 |
| 940090 | Single storey kitchen rear extension. | Determination Made | 7/10/1994 |

CONSULTATIONS

Confirmation that the site notice was displayed on 29th April 2021 was received. The following addresses were consulted by letter and no letters of representations were received.

89, 91 and 93 Brunswick Street, RG1 6NY

9 and 13 Downshire Square, RG1 6NJ

Internal Consultation – Conservation Area Advisory Committee

No comments received

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore, in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with policies in the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

National Planning Policy

National Planning Policy Framework (2019)

National Planning Practice Guidance

Reading Borough Local Plan (2019)

CC7- Design and the Public Realm

CC8 - Safeguarding Amenity

H9 - House Extensions and Ancillary Accommodation

SPG/SPD

A Design Guide to House Extensions SPD (2021)

Assessment

Character and appearance

Policy CC7 (Design and the Public Realm) requires that all developments be of high design quality, whilst Policy H9(House Extensions and Ancillary Accommodation) requires that an extension “respects the character and appearance of the house in terms of scale, location, materials and design and should not harm the character and pattern of neighbouring houses and the appearance of the street scene”.

Proposed exterior finishes including windows would complement that on the existing house. There are existing rear dormer extensions in the area therefore an extension of this kind is not unconventional nor would it be considered to have a detrimental impact on the character and appearance of neighbouring houses and the street. Although the proposed rear dormer is significant in scale, the impact on the overall appearance of the existing house and the neighbouring houses would not be considered detrimental. In addition, the extension would not be considered harmful on the character and appearance of neighbouring houses and the street scene as the proposed development would not be visible to the public from the road. The proposed development does not harm the character or appearance of the existing house nor have a detrimental impact on the area.

Residential Amenity

CC8 (Safeguarding Amenity) seeks to prevent development causing detrimental impact on the neighbouring properties; policy H9 (House Extensions and Ancillary Accommodation) provides that extensions should not cause an overbearing impact on neighbours”.

The proposed rear dormer would have an obscured glazed window on the rear elevation and no windows on the side elevation therefore the extension would be considered unlikely to cause loss of privacy to the neighbours. In addition, the proposed extension would not cause an overbearing impact on neighbours due to its location. The extension is unlikely to cause any loss of sunlight, daylight or overshadowing to the neighbouring houses.

Conclusion

The proposed extension is considered to be in accordance with the Reading Borough Local Plan (2019) and the Design Guide to House extensions (2021) and therefore the application is recommended for approval subject to conditions.

Recommendation:

Grant

CONDITIONS

- Time Limit (Standard) TL1

- Approved Plans Ap1

- Materials (As Specified) M3