DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Tilehurst

App Ref: 210783 / HOU

Address: 19 Glenwood Drive, Reading, RG31 4HE

Proposals: Two storey side and rear extension including single storey porch to

front and new dropped kerb.

Case Officer: Beatrice Malama

Date received (valid from): 17th May 2021      Target date: 12th July 2021

Site description

The site is a two story dwelling house located at the end a terraced property of four dwellings.  The area is predominantly residential characterised by a mixture of detached, semi-detached and terraced properties of varying styles and designs. The property is not listed, nor located within a conservation area.

Proposal

The proposal comprises a two-storey and part single rear and side extension including the front porch. The proposed ground and first floor extensions measure approximately 3m deep from the existing rear wall and 2.3 m deep from the existing side wall. The southern side elevation measures approximately 9.3m and 6.2m long at ground and first floor respectively. The ground floor rear extension measures approximately 7m wide whereas the first floor rear extension measures approximately 5.3 wide. The proposed development is set down resulting in a lower ridgeline than that of the existing house. The roof design, windows and doors would be similar to that of the existing building. The proposed development would extend very close to the site boundary to the south.

Plans considered;

Drawing No: 01 C – Existing and Proposed Plans and Elevations.

As received on 17th May2021

Relevant Planning History

No relevant planning history

Consultations

Confirmation that the site notice was displayed on 25th May was received and the following addresses were consulted by letter.

35 City Road, Tilehurst, RG31 4HA

17, 18, 20 Glenwood Drive, Tilehurst, RG31 4HE

One letter of representation has been received from the neighbour at No. 20 Glenwood Drive raising the following concerns:

- Obstruction of the public access way located to the south of the site.

- Safety of pedestrians using the path due to the presence of two opening windows on the side elevation at ground floor level.

- Parking

These concerns will be addressed later in the report under residential amenity and transport sections.

Internal Consultation – Transport

Additional supporting information was requested and submitted by the applicant.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore, in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with policies in the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

National Planning Policy

National Planning Policy Framework (2019)

National Planning Practice Guidance

Reading Borough Local Plan (2019)

CC7 (Design and the Public Realm)

CC8 (Safeguarding Amenity)

H9 (House Extensions and Ancillary Accommodation)

TR5: Car and cycle parking and electric vehicle charging

SPG/SPD

Design Guide to House Extensions (2021)

Revised Parking Standards and Design (2011)

Assessment

Character and appearance

Policy CC7 (Design and the Public Realm) requires that all developments be of high design quality.

Policy H9 House extensions and ancillary accommodation in Reading Borough Local Plan 2019 states that “Extensions respect the character and appearance of the house in terms of scale, location, materials and design and should not harm the character and pattern of neighbouring houses and the appearance of the street scene. Extensions should reflect the building line, or, preferably, be set back from the building line”.

The Council’s Supplementary Planning Guidance - Design Guide to House Extensions states that “two storey side extensions should normally be designed to be smaller in scale than the main house. To achieve this the proposed front elevation should be set further back than the front elevation of the original house and a gap between neighbouring buildings be maintained by bringing the upper floor away from the side boundary”.

The quality of the proposed design would not have significant adverse impact on the existing building and the street scene. In order to retain the character and primacy of the original house, the proposed extension has been set down resulting in the roof ridgeline of the extension being lower than that of the existing house and the overall effect of the extension being in proportion with the existing house. Exterior walls of the proposed extension and the design of the windows, doors and roof would closely match that of the existing house. The proposal would retain the character of the existing house and would not extend beyond the existing building line to the front. The proposal therefore does not harm the character or appearance of the existing house nor have a detrimental impact on the wider area.

Residential Amenity

Policy CC7 in Reading Borough Local Plan 2019 states that “All development must be of high design quality that maintains and enhances the character and appearance of the area of Reading in which it is located”.

Policy CC8 (Safeguarding Amenity) of the Reading Local Plan states that “development will only be acceptable where it will not cause a significant detrimental impact to the living environment of neighbouring residents in terms of privacy and overlooking, access to sunlight and daylight and visual dominance and overbearing effects of a development”. Policy H9 House extensions and ancillary accommodation in Reading Borough Local Plan 2019 states that an extension should “not present an overbearing impact on neighbours”.

Although the nature of the proposal would result in a significant addition to the existing house, it will unlikely represent an overbearing impact on the neighbouring properties. In addition, the proposed extension to the rear would unlikely cause loss of privacy or overlooking of the neighbouring property at No. 20 as the proposed rear elevation has no side windows on the side wall facing No. 20 and the closest point of the extension observes the 45-degree code. Furthermore, part of the proposed front elevation and part of the rear extension would be single story thus making the proposed first floor extension more subservient to the original house. The proposal therefore would be considered unlikely to present an overbearing impact on the neighbouring properties and the street scene.

The proposal to extend the southern side elevation beyond the existing building line would result in a development that would be closer to the neighbour’s side elevation at No. 18 with the gap between the two houses being reduced however the proposed first floor windows serving the bathroom and the staircase would continue to be positioned at an angle that would avoid harmful overlooking of the neighbour’s secondary bedroom and non-habitable rooms. Furthermore, the existing fencing at the neighbour’s property would act as a screening between houses and thus limit any potential loss of privacy to the neighbour at No. 18 at ground floor level. The extension would be considered unlikely to cause any harmful loss of sunlight, daylight or overshadowing to the neighbouring houses due to the orientation of the terrace.

With regard to neighbour concerns regarding safety of pedestrians using the footpath located on the southern side of the site, although the proposal would result in the side elevation being closer to the path with two windows at ground floor level, the windows would be considered unlikely to cause harm to pedestrians as the windows would be non-opening up to 1.7m from the ground. In addition, the extension would be considered unlikely to obstruct the public access way as the extension would not extend beyond the site boundary.

Transport

As the proposal would involve widening of the dropped kerb, the revised plans indicate that the applicant had consulted highways and guidance provided accordingly. With regard to off road parking, the proposal has made provisions for two off road parking spaces. The proposal therefore complies with the requirements within the Council’s adopted Parking Standards and Design SPD that requires that 2 off road parking spaces be provided for a 3+ bedroom dwelling in Zone 2.

Regarding the neighbour’s concerns about parking of construction vans, the proposal has made provisions for onsite parking of construction vans and offloading of construction materials.

Conclusion

The proposed extension is considered to be in accordance with the Reading Borough Local Plan (2019) and the Design Guide to House extensions (2021) and therefore the application is recommended for approval subject to conditions.

Recommendation:

Grant

CONDITIONS

- Time Limit (Standard) TL1

- Approved Plans Ap1

- Materials (As Specified) M3