HTA Landscape Comments in response to Darren Cook

Transport Development Control Manager

Please see my comments on the revisions below, I thought it easier to make comments on each drawing individually.

Drawing HTA-L_DR_1900 Rev F Landscape GA

The disabled bays along the eastern boundary of the north south road have been provided with a continuous dropped kerb so that access can be gained to and from the bays however the extent of this area is excessive and would not be acceptable for parallel bays of this nature.  Individual dropped kerbs would be required given the distance between the bays.  I also have concern regarding the dropped kerbs associated with these bays which are adjacent to the landscaped areas as it is expected that soil and other materials will spill out from the landscaped areas into the path and carriageway.  Given that the carriageway is to be a porous block paving this will affect the functioning of the drainage structure and therefore the location of the dropped kerb should be relocated away from the landscaped areas. HTA Landscape Comments: Comments have been addressed on landscape GA’s. In addition, kerb and edging drawings 1910/1911 have been produced so as to aid clarification.

There are also three disabled bays located on the reconfigured Wensley Road adjacent to Wensley Court, it has previously been agreed that a continuous dropped kerb can be provided given the close proximity to the bays but some physical feature is required i.e. a series of bollards to ensure that vehicles do not drive up onto the footway and collide with pedestrians.  Revised drawings should include these bollards. HTA Landscape Comments: The drawings have been updated to show upstand kerbs with local flush kerbs to allow access from the disabled parking bays.

The loading bay alongside Wensley Court provides a dropped kerb in the centre of the bay but this would be impractical and therefore the dropped kerb should be provided at either end of the loading bay so that access can be gained to the rear of the vehicle. HTA Landscape Comments: Drawings updated.

Along the northern edge of the play area discussions have been had with the Highway engineer regarding the levels and vehicles travelling south along Wensley Road in particular larger delivery vehicles and it has been suggested that a trief kerb be provided which has been accepted as part of the Highway Agreement process however only a standard kerb has been illustrated.  This must be revised or clarified. HTA Landscape Comments: We feel the trief kerb is not preferable in this area as it is intended to read as an open/accessible area of public realm, a trief kerb would reduce eas of access to certain user groups.

All of the dropped kerbs adjacent to the landscaped areas result in the bottom of a dropped kerb being located adjacent to a full height kerb at a 90⁰ which is not acceptable.  This would not just be a hazard but would result in soil spilling out in the carriageway effecting the drainage structure. HTA Landscape Comments: Noted, all instances have been updated on GA’s and new kerb and edging drawings.

To the south of Wensley Court no dropped kerb has been provided for the disabled bay.  The location of this should be carefully designed to ensure that soil does not spill out form the adjacent landscaping area. HTA Landscape Comments: Parking bay and associated drop/flush kerbs flipped to avoid conflict with planting.

Along the southern edge of the play area the block paving is broken up by an area of resin bound gravel, given that this is to be located on an area to be adopted this must be removed and replaced with block paving. HTA Landscape Comments: Resin has been removed as requested.

The Highway Authority have accepted that the three different colours can be used on the site but have stressed that future maintenance may mean that similar colours will not be replaced like for like due to issues with storing correct colours of materials.  However, the scheme should ensure consistency and therefore the footways surrounding the play area should be in Slate block paving to match the surrounding footways across the site. HTA Landscape Comments: The slate block perimeter around the open space and play area is a key part of the design intent in creating a central pedestrian dominant open space. The southern and eastern sections of footway around the play space have 4m wide paths for emergency access and it is important that these retain a different in character to the highways. As such both HTA and Glenmans feel that it is appropriate to retain the paving as specified.

Where the east bound footway meets Lesford Road a review must be undertaken as to how this connects to the existing Lesford Road footway because at present they will not tie in with one another.  This will require a further footway outside of the red line or the final landscaped area adjacent to the parking is removed and replaced with a footway. HTA Landscape Comments: This area has been designed to tie into existing footway, however due to the constraints of the drawing size and the fact that it is outside the site boundary, the area beyond is not shown. However, the works shown within the red line would allow the footway to tie into the existing surveyed footway beyond the red line.

An area of Heather block paving is provided outside Riversley Court however as this is already a pedestrianised area with only limited access for emergency vehicles this should be provided as Slate block paving as stated above. HTA Landscape Comments: This area of paving is key in providing a clear visual contrast to denote a safe pedestrian route across the widened eastern footway to the open space. It is also key in improving the visual connectivity of Riversley Court to the play space. As such we would propose to retain it as currently specified.

Drawing HTA-L_DR_1901 Rev F Landscape GA

The east west path to the west of the site is provided with tactile paving on either side but there are sections that do not align with the opposite side of the road.  The tactile paving should therefore be revised so that it aligns on both sides of the road. HTA Landscape Comments: Drawings have been updated to align both areas of tactile.

The car park to the south of Riversley Court is provided with a flush kerb around the western, northern and eastern edges of the car park with bollards provided along the northern boundary only.  To ensure that vehicles do not drive onto the adjacent footways and landscaped areas physical features should be provided along the western and eastern boundaries also. HTA Landscape Comments: Raised kerbs with flush access have been provided to Western boundary to prevent vehicle access. The eastern boundary is a combination of raised kerb and retaining wall with tree planting behind the kerb, as such vehicle access is unlikely.

No dropped kerbs have been provided for the disabled bays located within the parking court for Irving Court, these must be provided. HTA Landscape Comments: Dropped kerbs have been provided.

All comments made above regarding drawing HTA-L_DR_1900 Rev F Landscape GA that relate to areas that cross over on to this drawing will also apply. HTA Landscape Comments: All kerbs have been review across the masterplan in light of the above comments and new kerb and edging drawings produced to aid in clarification of the proposals.