DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Southcote

App Ref: 210846 / HOU

Address: 64 Brunel Road, Reading, RG30 3JL

Proposals: Proposed two storey pitched roof side / single storey flat roof rear extension Case Officer: Beatrice Malama

Date received (valid from): 24th May 2021      Target date: 19th July 2021

Site description

The application site is a two story semi detached dwelling house located in predominantly residential area characterised by a mixture of semi-detached and terraced properties of similar styles and designs. The property is not listed, nor located within a conservation area.

Proposal

The proposal comprises a two-storey side extension and single story rear extension. The proposed rear extension measures 4m deep from the existing rear wall and approximately 6m wide and a maximum height of 3m; whereas the side ground floor extension would measure approximately 2.3 deep from the existing side wall and approximately 11.7m along the side elevation. The proposed first floor side extension measures approximately 2.3m deep from the existing side wall and 6.8m along the side. The ground floor side extension would extend forward of the existing building line to the front by approximately 1m whereas the proposed first floor extension would be set back from the existing building line to the front by 0.5m. The proposed two story side extension would have a pitched roof whereas the proposed single story rear extension would have a flat roof with a pitched rooflight. The proposed side extension stretches along the boundary fencing shared with No. 62 Brunel Road and the proposed rear extension would extend up to the site boundary both to the western and eastern sides. Exterior finishing materials including the design of windows and doors would be similar to that on the existing house.

Plans considered;

Drawing No: BR-01– Existing and Proposed Plans and Elevations.

As received on 24th May2021

Relevant Planning History

No relevant planning history

Consultations

A site notice was sent to the applicant on 4th June 2021 and the following addresses were consulted by letter and no letters of representation were received.

66 Brunel Road, Reading, RG30 3JL

62 Brunel Road, Reading, RG30 3JL

Internal Consultation

Transport - No Objections to this application subject to conditions

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore, in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with policies in the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

National Planning Policy

National Planning Policy Framework (2019)

National Planning Practice Guidance

Reading Borough Local Plan (2019)

CC7 (Design and the Public Realm)

CC8 (Safeguarding Amenity)

H9 (House Extensions and Ancillary Accommodation)

TR5: Car and cycle parking and electric vehicle charging

SPG/SPD

Design Guide to House Extensions (2021)

Revised Parking Standards and Design (2011)

Assessment

Character and appearance

Policy CC7 (Design and the Public Realm) requires that all developments be of high design quality.

Policy H9 House extensions and ancillary accommodation in Reading Borough Local Plan 2019 states that “Extensions respect the character and appearance of the house in terms of scale, location, materials and design and should not harm the character and pattern of neighbouring houses and the appearance of the street scene. Extensions should reflect the building line, or, preferably, be set back from the building line”.

The Council’s Supplementary Planning Guidance - Design Guide to House Extensions states that “two storey side extensions should normally be designed to be smaller in scale than the main house. To achieve this the proposed front elevation should be set further back than the front elevation of the original house and a gap between neighbouring buildings be maintained by bringing the upper floor away from the side boundary”.

The proposed design and materials would match that on the existing house therefore the proposed development would be considered unlikely to harm the character and appearance of the existing house, neighbouring houses and the street scene. In order to retain the character and primacy of the original house, the proposed side extension has been set down and set back resulting in the roof ridgeline of the extension being lower than that of the existing house and the overall effect of the extension being in proportion with the existing house. Exterior walls of the proposed extension and the design of the windows, doors and roof would closely match that of the existing house. The proposal therefore does not harm the character or appearance of the existing house nor have a detrimental impact on the wider area.

Residential Amenity

Policy CC7 in Reading Borough Local Plan 2019 states that “All development must be of high design quality that maintains and enhances the character and appearance of the area of Reading in which it is located”.

Policy CC8 (Safeguarding Amenity) of the Reading Local Plan states that “development will only be acceptable where it will not cause a significant detrimental impact to the living environment of neighbouring residents in terms of privacy and overlooking, access to sunlight and daylight and visual dominance and overbearing effects of a development”. Policy H9 House extensions and ancillary accommodation in Reading Borough Local Plan 2019 states that an extension should “not present an overbearing impact on neighbours”.

Although the nature of the proposal would result in a significant addition to the existing house, it will unlikely represent an overbearing impact on the neighbouring properties. In addition, the proposed extension to the rear would unlikely cause loss of privacy or overlooking of the neighbouring properties as the proposed rear elevation is single story and has no windows on the sides. Furthermore, the proposed first floor extension is set back from the existing building line to the front and set down resulting in the ridgeline being lower than that on the existing house and thus making the proposed first floor extension more subservient to the original house. The proposal would be considered unlikely to cause harmful overbearing impact on the neighbouring houses and the street scene.

The proposal to extend the side elevation beyond the existing building line would result in a development that would be closer to the neighbour’s side elevation at No. 62 with the gap between the two houses being reduced however there are no windows on the proposed side extension therefore there would be no overlooking of the neighbour’s house. With regard to loss of sunlight, daylight and overshadowing, although the neighbour’s property has side facing kitchen door which is partly shaded by the existing neighbour’s house and the side facing first floor bathroom window, the proposal is not considered to make the living conditions of the neighbouring house at No. 62 significantly worse. The extension would be considered unlikely to cause harmful loss of sunlight, daylight or overshadowing to the neighbouring houses.

Transport

The proposed extension will result in the loss of the existing detached garage located towards the rear of the property.  Plans have not been submitted illustrating the existing or proposed parking layout, however a view of the property using google arial photos confirms that there is adequate parking in the front garden area and the property is served via an existing dropped kerb.  If any alterations are proposed to the dropped kerb a license will be required from the highways department.

Conclusion

The proposed extension is considered to be in accordance with the Reading Borough Local Plan (2019) and the Design Guide to House extensions (2021) and therefore the application is recommended for approval subject to conditions.

Recommendation:

Grant

CONDITIONS

- Time Limit (Standard) TL1

- Approved Plans Ap1

- Materials (To Match) M1