Delegated Application Report 				          Target Date: 21/07/21

Address: 30 Brackendale Way, Reading, RG6 1DZ

Application No: 210868 / CLP

Proposal: Certificate of Lawfulness for proposed conversion of existing loft to form habitable accommodation, including 4no rooflights to the front elevation and 3no dormers to the rear elevation.

Case Officer: Beatrice Malama

Introduction and Proposal

The application site is a two storey detached house located in a predominantly residential area comprising of detached and semi-detached houses of similar design and style. The street is characterised by houses with hipped roofs creating a uniform style in the street scene. The property is not Listed and it does not fall within a Conservation Area.

The proposal involves conversion of existing loft to form habitable accommodation, including 4no rooflights to the front elevation and 3no dormers to the rear elevation. The proposed dormers would each measure a maximum of approximately 3.2m deep, a maximum height of approximately 2.1m and approximately 1.6m wide. The proposed dormers would be set back from the existing eaves by approximately 1.2m. Materials to be used on the exterior of the extension would match that of the existing housing.

Plans Submitted

The following plans were submitted and received on 26TH May 2021:

Drawing No: 6857:21:1 – Existing Plans and Elevations

Drawing No: 6857:21:2 – Proposed Plans and Elevations

Relevant Planning History

| Application No. | Description | Decision | Date |
| --- | --- | --- | --- |
| 940951 | Erection of one detached dwelling (amendment to previous consent) | Determination Made | 22/09/1994 |
| 930638 | Erection of four bedroom detached house | Granted | 14/12/1993 |

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance.

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO).

Permitted development rights for householders – Technical Guidance September 2019

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed conversion of existing loft to form habitable accommodation, including 4no rooflights to the front elevation and 3no dormers to the rear elevation must be assessed to ensure that it complies with the conditions of Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relate to ‘additions etc to the roof of a dwellinghouse’ and ‘other alterations to the roof of a dwellinghouse’.

The relevant conditions of the GPDO for conversion of existing loft to form habitable accommodation, including 4no rooflights to the front elevation and 3no dormers to the rear elevation, which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, n/a or (ii) 50 cubic metres in any other case; complies, the extended roof would increase the volume of the original roof by 16.13 cubic metres.

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

C.1 Development is not permitted by Class C if:

C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

C.1 (b) the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies

C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

C.1 (d) it would consist of or include (i) the installation, alteration, or replacement of a chimney, flue or soil and vent pipe, complies or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. complies

Conditions

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

C.2 (a) obscure-glazed; n/a and

C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. n/a

Positive and Proactive Working

The Local Planning Authority has worked positively and proactively with the applicant to resolve relevant matters to allow permission to be granted within the prescribed target or agreed extended timescale for the decision to be issued. All the missing information related to this application was supplied to the planning officer via email.

Recommendation

Evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed conversion of existing loft to form habitable accommodation, including 4no rooflights to the front elevation and 3no dormers to the rear elevation.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B, Class C and Class G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes based on the drawings received on 26th May 2021:

The following plans were submitted and received on 26TH May 2021:

Drawing No: 6857:21:1 – Existing Plans and Elevations

Drawing No: 6857:21:2 – Proposed Plans and Elevations

Therefore, the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Bats

Figure 1: Attached Plan