# DELEGATED APPLICATION REPORT

Address: 89 Wokingham Road, Reading, RG6 1LH

Application No: 211186 / FUL

Proposal: Proposed siting of extraction and ventilation equipment to roof to rear

Applicant: Raja Khan

Target Decision date: 07/10/21

Agreed Extension: 13/10/21

Case officer: Beatrice Malama

1. Site description and Surrounding Area

The application site is a two - storey building located at 89 Wokingham Road in the middle of a parade of shops comprising of shops at ground floor level and mostly dwelling flats at first floor level. The ground floor of the site was until recently used as a care agency and it is now being converted into a restaurant whereas the first floor is a dwelling flat. Located to the west of the site is Alfred Sutton Primary School and to the rear is a terrace of dwelling houses. The property is not listed nor is it located within a conservation area.

2. The Proposal

The proposal is for the installation of extraction and ventilation equipment to rear roof of the site. The application does not include change of use of the premises from Financial & professional services to restaurant as both these fall under class E of The Town and Country Planning (Use Classes) (Amendment) (England) Regulations 2020.

3. Consultation

Internal Consultation

Environmental Protection – no objections subject to conditions

Ecology – No objections

External Consultation

A site notice was sent to the agent on 16/08/2021 and the following addresses were formally consulted by letter:

26 Norris Road, Reading, RG6 1NJ

24 Norris Road, Reading, RG6 1NJ

22 Norris Road, Reading, RG6 1NJ

89 Wokingham Road, Reading, RG6 1LH

91 Wokingham Road, Reading, RG6 1LH

91a Wokingham Road, Reading, RG6 1LH

Flat 89 Wokingham Road, Reading, RG6 1LH

87a Wokingham Road, Reading, RG6 1LH

87 Wokingham Road, Reading, RG6 1LH

No responses have been received.

4. Plans and Documents Considered:

Drawing No: TQRQM21198174542609 – Site Location Plan

Drawing No: TQRQM21198174741077 – Site Block Plan

Drawing No: Unnumbered - Existing and Proposed Plans

Proposed Ventilation Services (Odour Control Information)

Proposed System Specifications 1

Proposed System Specifications 2

Received 17th July 2021

Plant Noise Assessment Report Ref: R/PNA/1/210812

Sound data

Supporting Letter/Planning Statement dated 12 August 2021

Received 12 August 2021

Drawing No: AP173341-DRG-001-R3 – Proposed Elevations

Received 13 August 2021

5. Relevant Planning History

None relevant to this proposal

# 6. Discussion with applicant/ agent

# The agent was contacted regarding the siting and height of the Jet Cowl in relation to the ridge of the building and it was confirmed that the equipment would not go above the building’s ridgeline.

# 7. Relevant Planning Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  The Development Plan for Reading relevant to the application site comprises the Reading Borough Local Plan (2019).

The ‘National Planning Policy framework’ (‘’NPPF’’) 2021 states clearly that its content is to be a material consideration in the determination of applications.  The ‘NPPF’ states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Reading Borough Local Plan, 2019) according to their degree of consistency with the ‘NPPF’ (the closer the policies in the plan to the policies in the ‘NPPF’, the greater the weight that may be given). Accordingly, the ‘NPPF’ and the following development plan policies and supplementary planning guidance are relevant:

8. Relevant Policies

The application has been assessed against the following policies:

National

National Planning Policy Framework;

National Planning Policy Guidance;

Reading Borough Local Plan (2019)

CC7 	Design and the Public Realm

EN16	Pollution and Water Resources

EN17 	Noise Generating Equipment

9. Appraisal

a) Impact of the proposal on the surrounding area

The property, like all the adjoining properties, has a flat roof to the rear and a pitched roof to the front facing Wokingham Road. The property is surrounded by properties with commercial use at ground floor level and residential at first floor. Ground floor uses of the properties surrounding the site include shops, restaurants and drying cleaning services among many other uses.

The proposed extraction unit would be located at the rear of the site on a flat roof and would extend up to the level of the building’s roof ridge however it would not be visible from Wokingham Road. The proposal is not considered to be out of keeping with the equipment likely found in the vicinity to support these types of units.

The proposal therefore is not considered to conflict with Design Policy CC7 of the Reading Borough Local Plan (2019).

b) Environmental Health Considerations

The proposal is considered acceptable in terms of Environmental Health as the proposal has met the necessary environmental health requirements. There is no objection from Environmental Protection subject to a condition limiting noise levels.

The proposal is therefore considered acceptable in relation to Policies EN16 and EN17 of the Reading Borough Local Plan (2019).

10. Recommendation

Grant planning permission, subject to the following conditions:

Conditions

- TL1

- Approved Plans AP1

- Noise Levels of Plant/Equipment Restricted N8

Informatives

- Terms

- Building Control

- Complaints about construction

- Encroachment

- Positive and Proactive