DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Caversham

App Ref: 210619 / HOU

Address: 19 Priory Avenue, Reading, RG4 7SE

Proposals: Rear and side infill extension and loft conversion with rear dormer, including 2 no. roof lights to the front and 1 no. roof light to rear existing flat roof

Case Officer: Beatrice Malama

Date received (valid from): 21st April 2021      Target date: 16th June 2021

SITE DESCRIPTION

The site is a two story mid terrace house located in a predominantly residential neighbourhood comprising of terraces, semidetached and detached properties of similar style, design and age. The property is not listed and does not fall within a conservation area.

PROPOSED DEVELOPMENT

The proposed development comprises of a single story rear/side extension, loft conversion including rear dormer and two new roof lights to the front and one to the rear on an existing flat roof. The proposed side/rear extension measures 1.58m deep from the existing side wall and 6m long. The proposed side extension would have a lean to pitched roof with the highest part of the proposal measuring approximately 2.3m high while the shortest would be 1.75m high. The proposed side extension would extend very close to the boundary shared with No. 21 Priory Avenue. Part of the boundary fencing between the site and No 21. would be extended upwards by 0.25m to match with the highest part of the fence which measures approximately 1.75m. The gutter on the side extension would be hidden.

The proposed rear dormer measures approximately 4m wide and 3.7m deep from and 2.64m high. The dormer would be set down from the ridgeline of the existing house and set back from the eaves of the existing house by 0.25m.

Exterior finishing materials including windows and doors on the proposed development would complement that on the existing house.

Plans considered;

Drawing No: PA-01 – Location Plan

Drawing No: PA-02 – Block Plan

Drawing No: PA-03 – Existing Site Plan

Drawing No: PA-07 – Existing Elevations

Drawing No: PA-05 – Existing Floor and Roof Plans

Drawing No: PA-04 – Proposed Site Plan

Drawing No: PA-06 – Proposed Floor and Roof Plans (Rev)

Drawing No: PA-08 – Proposed Elevations (Rev)

Drawing No: PA-09 – Proposed 3D External Views (Rev)

As received on 20th April 2021

RELEVANT PLANNING HISTORY

No Planning History

CONSULTATIONS

Confirmation that the site notice was displayed on 29th April 2021 was received. The following addresses were consulted by letter.

38 St Annes Road, Caversham, RG4 7PA

21 Priory Avenue, Caversham, RG4 7SE

Flats 1, 2 & 3; 17 Priory Avenue, Caversham, RG4 7SE

Two letters of representation were received from the neighbours at No. 17 and 21 Priory Avenue

Concerns raised include increased pressure on the existing sewer lines, parking, loss of daylight and sunlight and an overbearing impact of the proposed development on the neighbouring houses. In order to address some of the concerns raised such as an overbearing impact and loss of daylight and sunlight, the agent made changes to the proposed plans and amended plans submitted.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore, in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with policies in the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

National Planning Policy

National Planning Policy Framework (2019)

National Planning Practice Guidance

Reading Borough Local Plan (2019)

CC7- Design and the Public Realm

CC8 - Safeguarding Amenity

H9 - House Extensions and Ancillary Accommodation

SPG/SPD

A Design Guide to House Extensions SPD (2021)

Assessment

Character and appearance

Policy CC7 (Design and the Public Realm) requires that all developments be of high design quality, whilst Policy H9(House Extensions and Ancillary Accommodation) requires that an extension “respects the character and appearance of the house in terms of scale, location, materials and design and should not harm the character and pattern of neighbouring houses and the appearance of the street scene”.

Proposed exterior wall finishes would complement that on the existing house and the neighbouring houses. Although no precedence has been set on the terrace regarding rear dormer extensions, there are existing rear dormer extensions in the area therefore an extension of this kind is not unconventional nor would it be considered to have a detrimental impact on the character and appearance of neighbouring houses and the street.

Although the proposed side elevation extends very close to the boundary shared with No. 21 Priory Avenue filling up all the existing space between the boundary and the building line on the side elevation, the impact on the overall appearance of the existing house would not be considered detrimental as the scale, proposed design and materials complement that of the original house. In addition, the extension would not be considered harmful on the character and appearance of neighbouring houses and the street scene as the proposed development would not be visible to the public from the road. The proposed development does not harm the character or appearance of the existing house nor have a detrimental impact on the area.

Residential Amenity

CC8 (Safeguarding Amenity) seeks to prevent development causing detrimental impact on the neighbouring properties; policy H9 (House Extensions and Ancillary Accommodation) provides that extensions should not cause an overbearing impact on neighbours”.

As the proposed side extension is single story there would be no overlooking of neighbouring properties and the existing boundary fencing between the site and the neighbour at No. 21 acts as a form of screening between the two properties therefore the side extension is unlikely to cause loss of privacy to the neighbour. In addition, the proposed rear dormer would have glazed doors and no windows on the side elevation therefore the extension would be considered unlikely to cause loss of privacy to the neighbours. Furthermore, the proposed dormer doors would be positioned at an angle that would avoid harmful overlooking of the neighbour’s house. The extension is unlikely to cause any loss of sunlight, daylight or overshadowing to the neighbouring house.

Car Parking

A concern was raised regarding increased pressure on parking as a result of the proposed development however it would be considered unlikely for the proposal to result in increased pressure on the current parking as the house would retain the existing number of bedrooms. Although the proposal to convert the loft into habitable space would create an additional bedroom, the resulting room would only replace one of the existing bedrooms which would be converted into an office.

Sewer Lines

The agent was advised to inform Thames Water regarding the proposed development as this is not a planning matter.

Conclusion

The proposed extension is considered to be in accordance with the Reading Borough Local Plan (2019) and the Design Guide to House extensions (2021) and therefore the application is recommended for approval subject to conditions.

Recommendation:

Grant

CONDITIONS

- Time Limit (Standard) TL1

- Approved Plans Ap1

- Materials (As Specified) M3