DELEGATED APPLICATION REPORT - Application No’s: 200219 and 200220

Address: 2 Tilehurst Road ‘Bellevue Hotel’, Tilehurst, Reading

Proposal: 200219 - Proposed change of use from C1 use (Guest House of 18No bedrooms) to

Sui Generis use as a 'larger' Home in Multiple Occupation - HMO of 18No bedrooms

200220 – Listed Building Consent application

Case Officer: Ethne Humphreys

Determination Date: Originally 6/4/2020; EOT until 31/7/2020

Site Description and Surrounding Area:

The application site relates to the Grade II listed ‘Bellevue Hotel’ located on the northern side of Tilehurst Road, on the corner with Belle Vue Road.

Photograph from 2006

The listing description is as follows:

“Previously called Bellevue. Built before 1839 as a detached 2 1/2 storey stucco villa on basement. Front: 3 bays the centre a 2 storey canted 3 window bay with round headed windows on ground floor, open arcade to basement, segmental attic window. Glazing bar and cross glazed sash windows. Central break is higher with pediment gable formed of bracket verges and eaves. Wings have impost string carried over centre and cornices and blacking courses with central panels surmounted by elegant oval shaped urns, possibly of Coade stone. Flight of steps to doorway to right, flanked by panels and with radiating and wreathed fanlight. Slate roof with prominent corniced chimney stacks. North front similar but grander with bullseye attic window and arched recess with urn over centre window.”

The surrounding area is a mix of residential, commercial and retail properties, with largely residential on the north side of Tilehurst Road and a vets centre and petrol station opposite the site to the south.

The site is within the Castle Hill/Russell Street/Oxford Road Conservation Area (as extended in 2020) and is also within an Air Quality Management Area.

The Conservation Area Appraisal notes that:

“The Grade II Bellevue Villa is the only other remaining early estate house of this type in the character area and is located on Tilehurst Road. It functions currently as a bed and breakfast. Its exterior is reasonably well maintained, although the garden is given over to parking and could be more sensitively landscaped. It still retains original windows and has two Regency styled Coade stone decorative urns set on the roofline at each side.”  Officer note – there are 4 decorative urns in total with two on the front and two on the rear.

The site is not within an Article 4 Area.

Proposal:

The proposal is for the change of use of the hotel (C1) to that of a Sui Generis house of multiple occupation (HMO) with 18 bedrooms.

Associated listed building consent is also sought for internal alterations including:

- Ground-floor: conversion of a store room into a bathroom;

- Lower Ground-floor: removal of 2 walls between bedrooms B5, B5 and B7, to create a bathroom in B6. Wall nibs retained and steel beams over

Whilst no external works to the building itself are proposed, cycle storage for 10 cycles is proposed at the front of the site, as well as bin storage to the front of the building.

5 car parking spaces are proposed at the front of the sit.

Plans considered:

Drawing No: Location Plan TR02-10

Drawing No: Block Plan TR02-11

Drawing No: Topo Survey TR02-12

Drawing No: Existing Floor Plans TR02-13

Drawing No: Proposed Floor Plans TR02-14

Drawing No: Elevations TR02-15

Drawing No: Bins and Cycles TR02-BR-16

Received 10th February 2020

Other documents:

Planning, Design and Access and Heritage Statement

Received 10th February 2020

Summary of correspondence with agent

The agent has been advised that the application is likely to be refused, the agent has opted to continue with the application.

No formal pre-application advice was sought prior to the submission of this application.

Relevant Planning History:

93-00358-FUL Change of use from Guest House (partial - 7 rooms) to full guest house

(Class C1), including associated landscaping/car parking/grounds improvements. Permitted

90-01002-LBC Formation of one new opening in side boundary wall & improvement to existing opening in same for access closing existing vehicle access for safety. Permitted

Internal Consultation Responses

Transport: No objection subject to condition.

Environmental Protection: No objection subject to condition.

Heritage Officer: Objection due to impact on setting of listed building and conservation area.

Environmental Protection and Nuisance (standing advice): The property will be classed as a HMO and must meet the requirements of the Housing Act 2004.

From 1st October 2018 in Reading Borough Council area properties containing 5 or more persons within an HMO require a mandatory HMO licence.

Further information on fire safety standards in HMOs is available at:

https://www.gov.uk/government/collections/fire-safety-law-and-guidance-documents-for-business

Adequate facilities are required in all dwellings for personal washing, sanitary conveniences and for the storage, preparation and cooking of food.

If adequate facilities are not provided this could result in the need for enforcement action under the HHSRS.

Mandatory licensable HMOs have a set of national minimum standards relating to:

- Sanitary conveniences and

- Personal washing facilities

For amenity standards in HMOs visit our website:  http://www.reading.gov.uk/hmo

There is no prescribed space standard for HMOs and the property will be assessed under the HHSRS. However, the Council has provided guidance to meet an optimum standard, which is intended to prevent, avoid or minimise any related hazard.  For licensable HMOs, numbers of persons permitted per room, and an overall maximum number of persons permitted per property, will be detailed on the HMO licence.

Where cooking facilities are provided in a separate room, each bedroom must be minimum:

- 6.5 m2 if occupied by one person;

- 10.5 m2 if occupied by two persons.

For rooms with cooking facilities within the room, the following minimum room sizes shall apply:

- 10.5 m2 if occupied by one person;

- 14.5 m2 if occupied by two persons.

External Consultation Responses:

Conservation Area Advisory Group - Concerns regarding:

- Unacceptable living conditions for 18 separate households

- Overcrowding

- Lack of green space

Reading Civic Society – Concerns regarding:

- Use as HMO

- Increase in activity

- Insufficient communal space

- Insufficient kitchen space

- Stacking

- Internal alterations to listed building

- Insufficient outdoor space

- Failure to enhance setting of Conservation Area and listed building

Baker Street Area Neighbourhood Association – Concerns regarding:

- No protection or enhancement of Conservation Area

- Adequacy of rooms sizes

- Lack of suitable outdoor amenity space

- Position of cycle and bin storage

Neighbour letters were sent to the occupiers of nearby properties and a site notice was put up.

2 letters of objection received, concerned with:

- Increase in parking

- Personal safety

- Impact on community

- Noise levels

- Loss of privacy

- Impact on value of house

- Questions over prospective tenants

Correspondence from Councillors:

Cllr Page and Cllr Rowland initially requested that the application be ‘called-in’ for determination by the Planning Applications Committee. Upon further discussions with members, it was agreed for the application to proceed under delegated powers (owing to the officer level recommendation to refuse planning permission). This was agreed with by the Chair of the Planning Applications Committee.

Equality Act 2010:

When determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore, in terms of the key equalities protected characteristics, it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance:

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise. Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'. For this Local Planning Authority the development plan is now in one document – the Reading Borough Local Plan (November 2019), which fully replaces the Core Strategy, the Sites and Detailed Policies Document and the Reading Central Area Action Plan.

Section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires the local planning authority to have special regard to the desirability of preserving a listed building or its setting or any features of special interest which it possesses.

Section 72(1) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires the LPA to pay special attention to the desirability of preserving or enhancing the character or appearance of the Conservation Area.

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

National Planning Policy Framework 2019

National Planning Guidance

Reading Borough Local Plan 2019

Policy CC1: Presumption in Favour of Sustainable Development

Policy CC2: Sustainable Design and Construction

Policy CC3: Adaptation to Climate Change

Policy CC5: Waste Minimisation and Storage

Policy CC8: Safeguarding Amenity

Policy EN1: Protection and Enhancement of the Historic Environment

Policy EN3: Enhancement of Conservation Areas

Policy EN15: Air Quality

Policy EN16: Pollution and Water Resources

Policy EN18: Flooding and Drainage

Policy H3: Affordable Housing

Policy H8: Residential Conversions

Policy H10: Private and Communal Outdoor Space

Policy TR1: Achieving The Transport Strategy

Policy TR3: Access, Traffic and Highway-Related Matters

Policy TR5: Car and Cycle Parking and Electric Vehicle Charging

Supplementary Planning Guidance/Documents

Sustainable Design and Construction (2019)

Revised Parking Standards and Design (2011)

Residential Conversions (2013)

Affordable Housing SPD (2013)

Other relevant documentation/guidance/legislation

Castle Hill/Russell Street/Oxford Road Conservation Area Appraisal (2020)

Historic England Good Practice Advice in Planning Note 1: Conservation Area Designation, Appraisal and Management (Historic England, 2016)

Historic Environment Good Practice Advice in Planning Note 2: Managing Significance in Decision-Taking (Historic England, 2015a)

Historic Environment Good Practice Advice in Planning Note 3: The Setting of Heritage Assets (Historic England, 2015b)

Principles of Conservation (Historic England, 2008)

Guide to the Conservation of Historic Buildings (British Standards Publication BS 7913:2013, 2015)

National Design Guide: Planning practice for beautiful, enduring and successful places (2019)

APPRAISAL:

Policy H8 states that “Proposals to convert properties into self-contained flats or for multiple occupation will be assessed against the impact on the amenity and character of the surrounding area, particularly in terms of intensification of activity, loss of privacy, loss of external amenity space, the provision and location of adequate on-site car parking, and the treatment of bin storage areas and other related servicing”.

The Policy provides further detail and states: “Proposals to convert properties into self-contained flats or for multiple occupation will only be acceptable where:

- The proposals to convert the physical character of the area in terms of scale, location, materials and design, the arrangement of doors, windows and other principal architectural features;

- The proposal would not, either individually or cumulatively, unduly dilute or harm an existing mixed and sustainable community through the significant loss of single family housing;

- There are no unacceptable adverse impacts to residents of the scheme or surrounding properties arising from noise and disturbance in terms of the number and layout of units proposed and the proximity to other properties;

- There are no inappropriate stacking and location of rooms between units;

- Bin and cycle storage is of an appropriate size and standard for the units proposed and should be located at ground floor level with easy access; and

- The resulting property or properties would provide adequate internal floorspace and headroom for residents.”

Additionally, in the case of sui generis houses in multiple occupation (HMOs):

The property to be converted measures more than 120 square metres gross;

There is sufficient communal space.

Along with these adopted local planning polices any appraisal for a change of use to a Sui Generis large HMO is assessed against the Council’s ‘Residential Conversions Supplementary Planning Document SPD (2013)’. Section A of the SPD, deals with the ‘General Assessment of all Conversions’ (i.e from C3 dwellinghouses to flats or HMOs (both small C4 use and sui generis HMOs). In addition para 4.2 refers to the requirement to assess large HMOs under the threshold guidance in Section B. The SPD sets out a number of checklist items.

The main issues to be considered are:

- Principle of development

- The impact on amenity of proposed and existing residents of nearby properties

- Design Considerations and Impact on Listed Building and Conservation Area

- Transport Impacts

- Sustainability

- Environmental Matters

Principle of development

In terms of whether a property is suitable to be converted to a large HMO, Policy H8 requires a property to measure more than 120m2. The property meets this requirement and therefore the conversion into a large HMO would be acceptable in principle.

Further to the above, Policy DM8 seeks to prevent Sui Generis HMOs from unduly diluting or harming a mixed and sustainable community.  To assess whether a property is suitable for conversion the ‘tipping point’ calculation is used (threshold), which is defined in the Residential Conversions SPD as “planning permission will not normally be granted where the proportion of HMOs will result in HMOs representing 25% or more or the residential properties within a circle of 50m radius measured from the application site” (para. 5.43).

The concentration of HMOs in the area surrounding the application site has been calculated as a percentage of the total estimated number of existing HMOs (C4 or sui generis) against the total number of residential properties, i.e those falling with C3, C4 or sui generis HMO use. Available data from Environmental Health, Council Tax, extant (unimplemented) permissions for HMOs, data on property websites, and data held by the Planning Enforcement Team, has been used. Where the radius includes entire buildings falling within an A, B, C1 or C2 use class, D or Sui Generis Use Class apart from a Sui Generis HMO, they have been discounted from the total number of buildings in the radius.

The total number of relevant properties (i.e. excluding those wholly in A, B, C1 or C2 uses) within the 50m radius, including the application site, is 28.  The total number of properties in HMO use, using the above sources of data, is estimated to be 2 (Including the application site) and therefore the overall percentage is calculated as 2% which is well below the threshold of 25%. The majority of properties within the radius (specifically as part of Belle Vue Road) remain as single family houses, and, given this, and that the property was previously a guest house, it is considered that the proposal would not have a significantly detrimental impact on the existing character or composition of the immediate area.

Subject to addressing other policy issues, the principle of the use for a HMO is therefore considered by officers to be acceptable.

The impact on amenity of proposed and existing residents of nearby properties

The Residential Conversions SPD sets out a number of checklist items which provide further detail related to policies H8, CC8 and H10 of the Reading Borough Local Plan 2019.

Policy CC8 requires development to not cause a detrimental impact on the living environment of existing residential properties or unacceptable living conditions for new residential properties, in terms of: Privacy and overlooking; Access to sunlight and daylight; Visual dominance and overbearing effects of a development; Harm to outlook; Noise and disturbance; Artificial lighting; Vibration; Dust and fumes; Smell; Crime and safety.

Policy H10 deals specifically with private and communal space and requires such space to allow for sitting out, children’s play areas, home food production, green waste composting, refuse storage, drying space.

Room sizes/Communal Space

The SPD identifies minimum internal floorspace standards (Appendix 1) as 6.5m2 for a single bedroom without cooking facilities (and for single occupancy – as indicated by the applicant) Whilst acknowledging that the majority of the bedrooms proposed are larger than this, not all bedrooms do meet this. Specifically, the following bedrooms, as annotated on the proposed floor plans:

B6 – 6.42m2

B5 – 6.48m2

Further to the above, not all rooms on the lower ground floor appear to have access to sufficient natural daylight. Specifically, B5 (obscure glazed window with two vents covering), B6 (size of lightwell) and B7 (size) as annotated on the proposed floor plans.  The photograph below shows the arch over the low recessed window below the bay being the only window serving room B6 and the small low window on the side elevation – also the only window serving room B7.  It is clear from the location of windows serving the rooms at lower ground level that these, and certainly not the front facing rooms, were never intended to be used as habitable spaces.

Views of windows

The SPD states that the floor area for a shared kitchen shall not be less than 7m2. Furthermore, where bedrooms are at the minimum size, the SPD seeks a provision of 1 communal area per 4-6 rooms, not including a kitchen. In this instance, the provision of communal space is in the form of a kitchen, which is 7.19m2 (as annotated on the proposed plans) and lounge area measuring 31.75m2 (as annotated on the proposed plans).

Whilst the proposed kitchen just meets the minimum size requirement, and despite the applicant’s assertion that it is suitable for residents, it is considered that the use of the area by more than a handful of residents at any one time would be extremely difficult to achieve.  The communal kitchen space would be barely large enough to accommodate and serve the needs of 5 adults to move around in comfortably, let alone the 18 residents proposed although it is accepted that it is unlikely that all 18 residents would use the kitchen at the same time.

With regard the communal lounge area, albeit fairly large in itself, for the number of bedrooms proposed (18) and to comply with the SPD (1 communal area per 4-6 rooms), there should be three communal rooms. It has not been demonstrated that the communal area would readily provide sufficient room for all 18 persons to use it at the same time. Whilst it is acknowledged that a smaller group could be accommodated, taking into account the limited space available in the kitchen there remains insufficient room to accommodate an 18 persons HMO. As such, it is considered that there is not a sufficient balance of communal and private space within the development.

Taking into account the combination of the limited communal space available and the poor sizes and amenities for some of the rooms it is considered that the proposal fails to comfortably accommodate the number of occupants proposed.

Amenity Space

The supporting planning statement comments that in respect of garden space the proposal has an equivalent garden area to that of the building footprint.  However, this claim relies on including the front garden area which is proposed for the parking of vehicles and a cycle/ bins store. The noise and activity associated with vehicles manoeuvring and the proximity to the busy Tilehurst Road, makes it an unappealing space for relaxing and/or playing. This would leave the rear of the site, which is currently a gravel car park, with no indication on the plans for any specific garden or seating area. As such, the proposal has failed to demonstrate how sufficient meaningful outdoor amenity space can be provided for the 18 future occupants proposed.

Layout/Stacking of Rooms

The SPD states that proposed layouts should avoid locating living rooms, bathrooms and kitchens next to, above, or below proposed and neighbouring bedrooms. In this instance, the proposed kitchen and communal area would be located immediately above the entirety of B5, B6 and B7 and below the entirety of F14, F15 and F16 (as annotated on the proposed plans).

Privacy and Overlooking/Overbearing Impacts

Given the nature of the proposals, that does not introduce any additional windows or external alterations (barring the cycle and bin units proposed), it is not considered that there would be any significant material loss of amenity to neighbouring properties specifically in terms of loss of light, privacy or overbearing impacts.

Noise and Disturbance

The comings and goings of occupiers of an HMO are likely to be different from users of a guest house; indeed, the pattern of movement is likely to be more sporadic and the incidence of visitors higher.  In this respect, the proposals would result in the unacceptable creation of noise and disturbance generated by movement from a minimum of 18 adults (who are likely to be accompanied by friends on occasion) to and from the site and emanating from internal and external areas on the site. It is clear that given the close proximity of the site to other residential properties (directly to the north and west) there would be a significant potential for an adverse impact on residential amenity arising out of increased levels of noise and disturbance. It is acknowledged that an on-site management plan could ensure adherence to a code of conduct. Although such measures could also restrict resident’s movements to and from the property, it would be difficult to ensure that behaviour could/would be properly monitored or enforced.

In these regards the numerous individual shortfalls identified, when considered individually and cumulatively, result in an overall conclusion that the quality of accommodation for future occupiers and impact on neighbouring properties would be contrary to Policies H8 and CC8 of the Reading Borough Local Plan 2019.

Design Considerations and Impact on Listed Building and Conservation Area

Policy CC7 seeks that development should preserve or enhance the character of the area of Reading in which it is located.

The site is a Grade II listed building and as such there is a duty imposed by Section 66(1) of the Planning Listed Buildings and Conservation Areas) Act 1990 requiring decision makers to have special regard to the desirability of preserving the building or its setting or any features of special architectural historic interest which it possesses.

Further to the above, the site is also located within the Castle Hill/Russell Street/Oxford Road Conservation Area and as such there is a duty imposed by Section 72(1) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requiring decision makers to have special regards to the desirability of preserving or enhancing the character or appearance of a Conservation Area.

Policy EN1 states that historic features and areas of historic importance and other elements of the historic environment, including their settings, will be protected and, where appropriate, enhanced.  The original interior of this Listed Building has already been altered when changed to a guest house and it is evident that over the last few years that the condition of the building generally has deteriorated.  The proposal would lead to more erosion of the internal fabric of the building with the original plan form of the building difficult to discern.  The proposal does not provide for any protection of the Listed Building’s interior and by proposing to intensify the use by providing more rooms it fails to take the opportunity to enhance or restore the interior back to its original character.

Policy EN3 states that the special interest, character and architecture of Conservation Areas will be conserved and enhanced and that development proposals within Conservation Areas must make a positive contribution to local character and distinctiveness. The Council will, therefore, have regard to both the quality of the townscape and the quality and interest of the area, rather than solely that of the individual building.

Paragraph 193 of the NPPF 2019 details that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be).  This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.

The application property is included in the Castle Hill/Russell Street/Oxford Road Conservation Area Appraisal under the ‘Key Positive Characteristics’, as well as reference made to its listed building status. It is a highly prominent property that makes an important contribution to the setting of the Conservation Area. It is acknowledged that there are no external changes proposed to the building itself. However, externally within the grounds, the proposals include a new cycle rack to the side of the house and also a bin storage area to the front of the property. The proposed cycle area would largely be hidden by the existing high boundary wall along the Belle Vue Road. The proposed bin store, however, would be highly visible from the front of the property, given the low boundary brick wall. The proposed occupation of the building by 18 persons would require a number of waste receptacles and it is likely that the storage proposed would be inadequate in size to meet the needs of the occupants of the building and would need to either be increased in sized or quantity or both. It is considered that the proposed bin storage, located to the front of the building and in such a visually prominent location, would significantly detract from the setting of the listed building (given its position directly within line of the front elevation), and the appearance of the Conservation Area.

It is noted that the Conservation Appraisal makes specific reference to bins in front gardens, as well as excessive quantities of bins causing visual blight along streetscapes, as negative features of the Conservation Area.

Whilst acknowledging that the scheme is for the conversion of the existing property, Policy EN3 states that proposals must make a positive contribution to enhance the area. It further states that positive consideration will be given to proposals which take opportunities to enhance the character of conservation area. Such opportunities include (but are not limited to):

- Sympathetic landscaping and planting

- Improving or resting green spaces, including front gardens, that are appropriate to the historic interest of a Conservation Area

Given that the site frontage has clearly been looked at by the applicant, with respect to the parking/cycle/bin storage arrangements, and with Heritage Policies quoted in the Planning, Design, Access and Heritage Statement, it is therefore disappointing that no regard has been had to the enhancement of the site and Conservation Area – which would also enhance the setting of the listed building.

Given the above, the proposals would detract from the setting of both the listed building and the Conservation Area and this fails to preserve or enhance the appearance of this designated area.

S72(1) of the Planning (Listed Buildings and Conservation Areas) Act 1990 places a statutory duty to pay special attention to the desirability of preserving or enhancing the character or appearance of the Conservation Area. The harm identified would be relatively localised in comparison to the total size of the conservation area; as such the harm to the Conservation Area would be less than substantial. In these circumstances paragraph 193 of the National Planning Policy Framework says that the harm should be weighed against the public benefits of the proposal.

The very limited public benefits of the proposal in this case lie in the provision of an HMO serving 18 persons of accommodation in an accessible accommodation. It would also bring a vacant building back into use. It is noted, however, that no information has been provided about how long the building has been vacant and therefore this in itself is not substantiated. Nevertheless, in weighing up the planning merits of the proposals and applying a critical planning balance, it is considered that these benefits would be limited and that consequently, the benefits of the scheme are not considered to outweigh the harm the proposal would cause to the character and appearance of the Conservation Area and the setting of the Listed Building.

Given the above, it is considered that the proposed development would not preserve or enhance local distinctiveness and the setting of the Listed Building; it would not protect or enhance the Listed Building or the contribution that the site makes to the appearance of this part of the Russell Street/Castle Hill/Oxford Road Conservation Area as a whole, and therefore the proposal is contrary to Policies CC7, EN1 and EN3 of the Reading Borough Local Plan 2019.

Transport Impacts

Policies TR1, 3 and 5 seek to address access, traffic, highway and parking related matters, relating to development.

The site is located along the C400 Tilehurst Road with daily vehicle flows of over 15,000 vehicles. Premier bus services can be accessed at bus stops located on the Tilehurst Road within easy walking distance of the site which provides a 20 minute frequency to and from the Town Centre and areas of Tilehurst.

The site currently benefits from a direct vehicular access off the Tilehurst Road and informal parking to the front and rear of the property.

In accordance with the adopted Parking Standards and Design SPD, a HMO development would be required to provide a parking provision of 0.25 parking spaces per lettable room; therefore a minimum of 5 off street parking spaces would be required. The planning statement and the submitted proposed site plan illustrates that 5No parking spaces can be accommodated on the existing drive at the front of the property. Should this application have otherwise been considered acceptable, this provision could have been secured by way of condition.

The development is located in an area were the Council’s Residential Parking Permit scheme operates; therefore, if this application had otherwise been considered acceptable, there would have been an assumption that any future occupants of the proposed units would not be issued with a resident or visitor parking permit which would have been covered by condition and an informative applied. This would ensure that the development does not harm the existing amenities of the neighbouring residential properties by adding to the already high level of on street car parking in the area.

In accordance with the Council’s Parking Standards and Design SPD, cycle storage for a HMO should be provided at a ratio of 0.5 secure cycle storage spaces for each letting room within a sheltered and lockable store. The application plans show that cycle storage can be provided at the front of the property. The proposed enclosure would fulfil these requirements; however, the cycle stands need to be spaced 1000mm apart (opposed to 700mm apart). Should the application have otherwise been considered acceptable, then this could have been dealt with by way of a suitably worded condition.

Whilst the location of the proposed bin store is considered unacceptable on heritage grounds, in pure transport terms, the location is considered acceptable as it is located within 10m of the public highway to enable on-street servicing to occur.

In view of the above, there is no transport objection subjection to conditions and informatives. Policies TR1, TR3 and TR5.

Other Issues

Sustainability – The proposed development would be required to meet a BREEAM ‘Very Good’ standard for the conversion as per Policy CC2 and the Council’s Sustainable Design and Construction SPD. Should the application have otherwise been acceptable, a condition would have been attached requiring a BREEAM completion certificate to the above standard.

Other Matters Raised in Representation

As a sui generis HMO, it would be required to be licenced by the Council and maintained on a register, although anti-social behaviour would be a civil matter.

Property value is not a planning consideration.

Whilst eligibility criteria for accommodation is not a planning consideration, should the application have otherwise been considered acceptable a condition relating to submission of a management plan would have been secured, which would have included occupant and management obligations and remedial procedures.

Overall Planning Balance and Conclusion

The very limited planning benefits associated with the proposed development are that the provision of the HMO units would count towards the Borough’s housing supply figures, on the basis of 4 HMO rooms equating to one dwelling, and the scheme would bring a currently vacant listed building in to use.  However, the appraisal above has identified a number of significant shortfalls, such as; the quality of accommodation proposed for future occupants, the detrimental impact on the living environment of existing nearby occupiers, the failure to protect and enhance the Listed Building and the harm to the character and appearance of the conservation area and setting of the Listed Building.

The benefits of the proposed development could be applied to any proposal that sought to make use of the vacant building to provide residential accommodation and as such, when applying an overall critical planning balance of all the material considerations, do not outweigh the many identified failures of the proposal. Accordingly, within the context of national and local planning policies and other material considerations, as detailed in the appraisal above, full planning permission and listed building consent is recommended to be refused.

RECOMMENDATION: - REFUSE both applications

1. Unacceptable for future occupants in terms of internal space

The proposed conversion to an 18 bed HMO fails to demonstrate that the combination of the bedroom sizes and internal communal space layout would be sufficient to meet the reasonable needs of the future occupants, failing to provide an appropriate balance between communal and private areas and therefore fails to comply with Policies CC8 and H8 of the Reading Borough Local Plan 2019 and Reading Borough Council’s Residential Conversions Supplementary Planning Document (2013).

2. Unacceptable for future occupants in terms of noise/stacking

The proposed conversion to an 18 bed HMO is considered harmful to the living environment of future occupiers in terms of noise and disturbance due to the poor stacking arrangement. The proposal is therefore contrary to Policies CC8 and H8 of the Reading Borough Local Plan 2019 and Reading Borough Council’s Residential Conversions Supplementary Planning Document (2013).

3. Unacceptable for future occupants in terms of external space

The proposed conversion to an 18 bed HMO fails to provide adequate functional external space for future occupiers and is therefore contrary to Policies H8 and H10 of the Reading Borough Local Plan 2019 and Reading Borough Council’s Residential Conversions Supplementary Planning Document (2013).

4. Unacceptable for future occupants in terms of internal space

The proposed conversion to an 18 bed HMO fails to demonstrate that the combination of the bedroom sizes and internal communal space layout would be sufficient to meet the reasonable needs of the future occupants, failing to provide an appropriate balance between communal and private areas and therefore fails to comply with Policies CC8 and H8 of the Reading Borough Local Plan 2019 and Reading Borough Council’s Residential Conversions Supplementary Planning Document (2013).

5. Unacceptable for future occupants in terms of natural light

The proposed conversion to an 18 bed HMO fails to demonstrate that all habitable rooms (on the lower ground floor) would benefit from sufficient natural light and ventilation which is contrary to Policies CC8 and H8 of the Reading Borough Local Plan 2019 and Reading Borough Council’s Residential Conversions Supplementary Planning Document (2013).

6. Impact on neighbours

The proposed conversion to an 18 bed HMO would represent a harmful intensification of the use, due to the increase in activity and noise and external amenity area and that would a detrimental impact on residential amenity and the character of the area. The proposal is therefore contrary to Policies CC8 and H8 of the Reading Borough Local Plan 2019 and Reading Borough Council’s Residential Conversions Supplementary Planning Document (2013).

7. Impact on conservation area – bin store

The proposed bin store, due to its design and scale and prominent location at the front of the property, is considered to result in an adverse effect on the setting of the listed building and failing to preserve or enhance the character and appearance of the Castle Hill/Russell Street/Oxford Road Conservation Area. This would be contrary to Policies CC7, EN1 and EN3 of the Reading Borough Local Plan 2019.

8. Failing to contribute positively to the character and appearance of Listed Building and Conservation Area – similar reason for refusing 200220

The Belle Vue Hotel is a Listed Building and identified as a positive feature in the Castle Hill/Russell Street/Oxford Road Conservation Area. The proposed development would not preserve or enhance local distinctiveness and the setting of the Listed Building; it would not preserve or enhance the Listed Building or the contribution that the site makes to the appearance of this part of the Castle Hill/Russell Street/Oxford Road Conservation Area as a whole, and therefore the proposal is contrary to Policies CC7, EN1 and EN3 of the Reading Borough Local Plan 2019 and Paragraph 193 of the NPPF 2019.

11

Informatives:

The local planning authority has worked with the applicant in a positive and proactive way by clearly setting out concerns relating to the proposal and providing an opportunity for the applicant to withdraw the application. However, the applicant chose not to withdraw the application, hence the issuing of this refusal notice.

This planning application was submitted without the benefit of first seeking pre-application advice from the Local Planning Authority, which can often result in the avoidance of refusal of planning permission.  Please visit the Council’s website for more information on our pre-application service.