Delegated Application Report 				              Target Date: 23/04/21

Address: 8 Knights Way

Application No: 210322/CLP

Proposal: Hip-to-gable roof alteration and provision of rear box dormer

Case Officer: Tom Hughes

Introduction and Proposal

The application site comprises a two storey semi-detached dwelling with a hipped roof. A detached garage is located to the rear.

The proposal is for a hip-to-gable roof alteration and the provision of a rear box dormer. The cubic content of the resulting roof space would exceed the cubic content of the existing roof space by 40.05 cubic metres. Two windows would be located to the rearward face of the dormer. In an email dated 05/03/21, the agent confirmed that the proposed external materials would be of a similar appearance to those of the existing dwelling.

Plans Submitted

The applicant supplied the following, received on 26/02/21:

Drawing No: 01 Rev. A – Site Location, Existing Floor Plans and Elevations

Drawing No: 02 Rev. C – Site Location, Block Plan, Proposed Floor Plans and Elevations

Relevant Planning History

201787/HOU – Single storey side and rear extension and garage refurbishment – Permitted 10/02/21

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed hip-to-gable roof alteration and provision of rear box dormer must be assessed to ensure that it complies with the conditions of Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘additions etc to the roof of a dwellinghouse’.

The relevant conditions of the GPDO for a hip-to-gable roof alteration and provision of rear box dormer, to a property which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, n/a or (ii) 50 cubic metres in any other case; complies, the resulting roof space would exceed the original roof space by 40.05 cubic metres

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies or

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies, as confirmed by the agent via email dated 05/03/21

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.

It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 26/02/21:

Drawing No: 02 Rev. C – Site Location, Block Plan, Proposed Floor Plans and Elevations

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed hip-to-gable roof alteration and provision of rear box dormer.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Bats