DELEGATED APPLICATION REPORT 			Target Date: 04/07/21

Application No: 210857 / HPA

Address: 541 Basingstoke Road, Reading, RG2 0SH

Proposal: Single storey rear extension

Case officer: Beatrice Malama

### Introduction

Under Class A, Part 1, of Schedule 2 to the Town and Country Planning (General Permitted Development) (England) Order 2015 (“the 2015 Order”) the following is allowed:

A single-story rear extension measuring 6.00m in depth, with a maximum height of 2.95m, and 2.87m in height to eaves level.

provided the development is not excluded by virtue of the criteria in Paragraph A.1 of the 2015 Order.

The site is a two story dwelling house located at the end of a terrace of four properties. The application site is not within a Conservation Area nor have PD rights been withdrawn or restricted by way of a direction made under Article 4 or condition attached to an express planning permission.

### Proposal

The proposal falls to be considered under Paragraph A.1 (g) and accordingly Condition A4 applies.

Information provided by the applicant regarding the rear extension indicate that the extension would:

- Extend beyond the rear wall of the original dwelling by 6.0m

- Have a maximum height of 2.95m

- Have a height to eaves of 2.87m

Plans Submitted

The following drawings were submitted by the applicant on 24th May 2021:

Location Plan, Block Plan, Existing and Proposed Ground Floor Plan, Existing Elevations, Proposed Elevations,

Planning History

None

Notification

In accordance with the relevant provision of the GPDO 2015 Order, the LPA notified on 1st June 2021 the adjoining owners and occupiers at Flats 1,3,4,5 Tiverton House

83a Ambrook Road, 79 Ambrook Road, and 539 and 543 Basingstoke Road of the proposed extension providing a period of 21 days for representations to be made. The 21day consultation period expired on 22nd June 2021. One objection letter was received from the neighbour at 543 Basingstoke Road raising concerns about potential loss of privacy.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular application.  Therefore, in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Policies

Legal and Planning Policy Context

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise. Material considerations include relevant policies in the National Planning Policy Framework (NPPF) – among them the ‘presumption in favour of sustainable development’.

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Plan (2019)

Policy CC7 – Design and the Public Realm

Policy CC8 – Safeguarding Amenity

Policy H9 – House Extensions and Ancillary Accommodation

Design Guide to House Extensions Supplementary Planning Document (2021)

Appraisal

The proposed development has been assessed under Class A, Part 1, of Schedule 2 to the Town and Country Planning (General Permitted Development) Order 2015, (as amended). In accordance with this legislation, adjoining owners and occupiers have been consulted on the proposal and a representation has been received. The prior approval of the Local Planning Authority is therefore required as to the impact of the proposed development on the amenity of the adjoining premises.

The case officer was able to make a full assessment of the site using Google Arial Photos accessed on 2nd July 2021.

The proposed extension would extend directly along the boundary with 539 Basingstoke Road, projecting to a total depth of 6m. The extension would have a flat roof, with a maximum height of 2.95m.

Although the proposed development would extend along the boundary with the neighbour with a significant depth from the existing rear wall, it would be considered unlikely to cause an overbearing impact on the neighbours. In addition, the veer away in the boundary would limit any potential harmful impact on the adjoining neighbours. Furthermore, as the proposal is a single story, it is unlikely to cause loss of privacy, light or overshowing to the neighbours. The existing boundary fencing would act as a form of screening between neighbours for added privacy.

With regard to the visual amenity, although the scale of the proposed extension would be substantial and the extension would have a flat roof different from that on the existing house, the impact on the existing house and that of the neighbours would not be considered detrimental. The proposal therefore satisfies the conditions of Policies CC7, CC8 and H9 of the Local Plan and the Design Guide to House Extensions SPD.

Recommendation

Grant prior approval subject to conditions