DELEGATED APPLICATION REPORT 			          Target Date: 29/03/21

Application No: 210266/HPA

Address: 84 Grange Avenue

Proposal: Single storey rear extension

Case officer: Tom Hughes

### Introduction

Under Class A, Part 1, of Schedule 2 to the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) (“the 2015 Order”) the following is allowed:

A single storey extension, beyond the rear wall of a house, between 3m and 6m in depth, and up to 4m in height

provided the development is not excluded by virtue of the criteria in Paragraph A.1 of the 2015 Order.

The application site is not within a Conservation Area nor have PD rights been withdrawn or restricted by way of a direction made under Article 4 or condition attached to an express planning permission.

Site Description and Surrounding Area

### 84 Grange Avenue comprises a mid-terraced dwelling, which benefits from a two storey rear gable. A further original single storey rear projection has been subsequently enlarged. The dwelling also benefits from a converted loft, with the provision of a rear box dormer. The surrounding area is residential in character. The site is located within an Article 4 area controlling the change of use from Class C3 dwelling to Class C4 HMO.

### Proposal

The proposal falls to be considered under Paragraph A.1 (g) and accordingly Condition A4 applies.

The agent has provided information on the proposed single storey rear extension. The extension would:

- Extend beyond the rear wall of the original dwelling by 6m

- Have a maximum height of 3.65m

- Have an eaves height of 3m

To facilitate the proposal, the existing single storey projection would be reduced back to the width of the original gable. The proposed extension would extend aside the original gable only, projecting to a total depth of 6m beyond the original rear elevation. A gap of approximately 1.5m would be maintained between the existing rear elevation and the extension, creating a void area. The existing bedroom window, and two proposed windows would look out into this void. A single door would be located to the rear of the extension. The proposal would have a pitched roof with a maximum height of 3.65m, and a height to eaves of 3m. The ridge would be located approximately 4.5m beyond the original rear elevation. Materials have been selected to match those of the existing dwelling.

Plans Submitted

The agent submitted the following, received on 19/02/21:

Site Location

Drawing No: 828/23 – Block Plan

Drawing No: 828/21 – Existing Floor Plan and Elevations

Drawing No: 828/22/b – Proposed Floor Plans and Elevations

Planning History

None.

Notification

In accordance with the relevant provision of the GPDO 2015 Order, the LPA notified on the adjoining owners and occupiers at 82 and 86 Grange Avenue of the proposed extension providing a period of 21-days for representations to be made. The 21 day consultation period expired on 22/03/21. One letter of representation have been received, with regard to the following:

- The extension would cause light loss and disturbance to 82 Grange Avenue

Policies

Legal and Planning Policy Context

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise. Material considerations include relevant policies in the National Planning Policy Framework (NPPF) – among them the ‘presumption in favour of sustainable development’.

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Plan (2019)

Policy CC7 – Design and the Public Realm

Policy CC8 – Safeguarding Amenity

Policy H9 – House Extensions and Ancillary Accommodation

Design Guide to House Extensions Supplementary Planning Document (2021)

Appraisal

The proposed development has been assessed under Class A, Part 1, of Schedule 2 to the Town and Country Planning (General Permitted Development) Order 2015, (as amended). In accordance with this legislation, adjoining owners and occupiers have been consulted on the proposal and a representation has been received. The prior approval of the Local Planning Authority is therefore required as to the impact of the proposed development on the amenity of the adjoining premises.

The case officer was able to make a full assessment of the proposal using Google Maps data, accessed on 19/03/21.

The proposed extension would extend directly along the boundary with 86 Grange Avenue, projecting to a total depth of 6m. The extension would have a pitched roof, with a maximum height of 3.65m. 86 Grange Avenue has not been extended to the rear, and benefits from a ground floor window serving a habitable room immediately adjacent to the boundary.

By virtue of its height and depth, directly along the boundary, significant harm would be caused to the residential amenities of 86 Grange Avenue in terms of visual dominance and overbearing effects, and light loss to the ground floor habitable room adjacent to the boundary. The proposal is contrary to Policies CC8 and H9 of the Local Plan and the Design Guide to House Extensions SPD on this basis.

Recommendation

Refuse prior approval