Target Date: 	25th January 2021

Address: 		31 St Saviours Road, Reading, RG1 6EJ

Application No: 	201707

Case Officer: 	Mr James Schofield

Proposal: 		Certificate of Lawfulness for proposed single storey extension and

rear dormer.

Introduction and Proposal

Plans Submitted

The applicant supplied the following;

Dwg No: AD4458 – Proposed

Dwg No: AD4458 – Existing

As received 1 December 2020

Relevant Planning History

No relevant planning history.

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed must be assessed to ensure that it complies with the conditions of Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to the ‘enlargement, improvement or other alteration of a dwellinghouse’.

The relevant conditions of the GPDO for a single storey rear extension, which is not listed or in a conservation area, are as follows:

| A.1 Development is not permitted by Class A if: |  |  |
| --- | --- | --- |
|  | A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); | complies |
|  | A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); | complies |
|  | A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; | complies |
|  | A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; | complies |
|  | A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principle elevation of the original dwellinghouse; | complies |
|  | or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; | complies |
|  | A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, | complies |
|  | or (ii) exceed 4 metres in height; | complies |
|  | A.1 (g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, | complies |
|  | or (ii) exceed 4 metres in height; | complies |
|  | A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, | N/A |
|  | or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse; | N/A |
|  | A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres; | complies |
| A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if: |  |  |
|  | A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles; | complies |
|  | A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse; | complies |
|  | A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse; | complies |
|  | A.2 (d) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (b) and (c). | complies |
| Conditions |  |  |
| Development is permitted by Class A subject to the following conditions: |  |  |
|  | A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse; | complies |
|  | A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, | N/A |
|  | and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; | N/A |
|  | and |  |
|  | A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse | N/A |
| B.1 Development is not permitted by Class B if: |  |  |
|  | B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); | complies |
|  | B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; | complies |
|  | B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; | complies |
|  | B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, or | complies |
|  | (ii) 50 cubic metres in any other case; | n/a |
|  | B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, or | complies |
|  | (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; or | complies |
|  | B.1 (f) the dwellinghouse is on article 2(3) land. | complies |
| Conditions |  |  |
| B.2 Development is permitted by Class B subject to the following conditions: |  |  |
|  | B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; | complies |
|  | B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; and | complies |
|  | (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; and | complies |
|  | B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and | n/a |
| Interpretation of Class B |  |  |
| B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not. |  |  |

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) Class A and B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.

It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on:

1st of December 2020

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed.

Informatives

Building Regulations

Construction and Demolition