DELEGATED APPLICATION REPORT

Target Date: 09/06/2018				Extended Target Date: 17/07/2020

Application No: 200557/FUL

Address: 13 Edgehill Street, Reading

Proposal: Retrospective change of use from C3 dwellinghouse to a 4 bedroom/4 person C4 small House in Multiple Occupation within the Article 4 area.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey end of terrace property.  The property has a two storey rear gable, a full width single storey rear extension and a flat roof rear dormer window.  Due to the levels on site there is a raised terrace area at the rear of the property.  The neighbouring property at 11 Edgehill Street is set much further back than the application property and is located at a much lower level.  The rear garden of 11 Edgehill Street is overlooked by the raised patio area at the application site but this is an existing arrangement.  The surrounding area is predominantly residential.

The property is in the Article 4 Area which removes permitted development rights to change from a C3 dwellinghouse to a C4 small HMO.

It has not been possible to visit the site due to the current pandemic however detailed photographs have been provided by the agent which have enabled Officers to assess the application fully in context with its neighbouring properties.

Proposal:

This is a retrospective application for the change of use from a C3 dwellinghouse to a 4 bedroom/4 person C4 small HMO with four rooms all with en-suites; one on the ground floor, two on the first floor and one in the roof.  A lounge, kitchen and WC are on the ground floor.

Plans considered:

The agent submitted the following plans and information, received on 14th April 2020:

Drawing No: EHS13:001 – Block Plan

Drawing No: EHS13:002 – Location Plan

Drawing No: EHS13:003 Rev A - Floor Plan Existing

Drawing No: EHS13: 004 – Typical Section

Completed Checklist

The agent submitted the following drawing, received on 4th June 2020:

Drawing No: EHS13:005 – Proposed Cycles Store

Relevant Planning History:

140472/HPA - Rear extension measuring 5.3m in depth, with a maximum height of 2.9m, and 2.8m in height to eaves level.  Refused 09/05/2014.

140816/HOU - Part retrospective application for single storey rear extensions.  Permitted 31/07/2014.

Consultation Responses:

Transport raised no objection subject to conditions and informatives.

Environmental Health – no comments at the time of writing.

Properties at 2, 2A, 4, 11 and 15 Edgehill Street were consulted.  One letter of observation was received with regards to: 1. Disappointing that the mechanism exists to allow applications for conversion of properties to HMO to be applied for retrospectively; 2. The use as a HMO is out of keeping with the usage of the rest of the street; 3. Noise and disturbance from the residents of the property and from the en-suites/fans.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 4th June 2020 that the principle of the change of use was acceptable however details of the proposed cycle store was required and additional information on the extractor fans.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance:

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

The following local and national planning policy and guidance is relevant to these applications:

National Planning Policy Framework

Part 5 – Delivering a sufficient supply of homes

Reading Borough Local Plan (November 2019)

CC8 (Safeguarding Amenity)

H8 (Residential Conversions)

H10 (Private and Communal Outdoor Space)

TR5 (Car And Cycle Parking And Electric Vehicle Charging)

Supplementary Planning Documents

Residential Conversions SPD (2013)

Revised Parking Standards and Design SPD (2011)

Legal context:

Reading Borough Council made an Article 4 Direction in May 2012, which came into force on 16th May 2013, which removes permitted development rights to convert from a dwellinghouse (C3 use) to a house in multiple occupation (C4 – small HMO for 3-6 unrelated persons sharing at least one basic amenity) in parts of Redlands, Katesgrove and Park Wards.  The application site is within the Article 4 Direction Area.

Appraisal:

Main Issues: The main issues are considered to be:

(a)	Whether the conversion of the property to C4 use has resulted in unduly diluting or 	harming a mixed and sustainable community;

- The impact on amenity of existing residents of the property and those of nearby properties;

- Effect on car/cycle parking

Along with the adopted local planning polices the assessment has mainly been based on the Adopted Residential Conversions SPD (2013).  Section A of the SPD, deals with the ‘General Assessment of all Conversions’ (i.e from C3 dwellinghouses to flats or HMOs (both small C4 use and sui generis HMOs).  Section B specifically covers the assessment of applications for HMOs within the area covered by the Article 4 Direction.  This application has been considered against both sections.  The SPD sets out 26 no. checklist items; the relevant ones of these are used to form the basis of the assessment within this report.

(a)	Whether the conversion of the property to C4 use has resulted in unduly diluting or harming a mixed and sustainable community

The SPD identifies that the ‘tipping point is when the concentration of HMOs becomes over dominant and the community is no longer considered to be mixed and sustainable.’  The SPD defines that “planning permission will not normally be granted where the proportion of HMOs will result in HMOs representing 25% or more or the residential properties within a circle of 50m radius measured from the application site” (para. 5.43).  The concentration of HMOs in the area surrounding the application site has been calculated as a percentage of the total estimated number of existing HMOs (C4 or sui generis) against the total number of residential properties, i.e. those falling with C3, C4 or sui generis HMO use.  Available data from Environmental Health, Council Tax, extant (unimplemented) permissions for HMOs, and data on sites such as Rightmove, and data held by the Enforcement Team, has been used.  The total number of residential properties within the 50m radius, including the application site, has been calculated as 85.  The total number of properties in HMO use, using the above sources of data, including the application site is 12, which equates to 14.12%, and is below the threshold maximum of 25%.  Therefore in this regard the principle of the conversion of the application property to a C4 small HMO is considered acceptable.

(b)	The impact on amenity of existing residents of the property and those of nearby properties nearby properties

The Residential Conversions SPD sets out a number of checklist items which provide further detail related to adopted policies CC8, H8, H9 and H10 of the Reading Borough Local Plan.

Room Sizes - The SPD identifies minimum internal floorspace standards (Appendix 1) as 6.5sqm for a single bedroom and 7sqm for a kitchen.  The kitchen is open plan with the lounge and is shown as approximately 20sqm and the smallest bedroom 11sqm.  Room sizes are therefore considered acceptable in this instance.

Communal Space – There is a lounge and kitchen which is considered acceptable for a small HMO.  A condition will be imposed to retain the communal space.

Amenity Space – There is an adequate sized rear yard/garden.

Visual Dominance and Overbearing – There are no internal or external alterations to the property and as such the change of use does not change the existing situation.

External windows - All habitable rooms benefit from external windows.

Access to sunlight/daylight – This is acceptable.

Layout/Stacking of Rooms - The Council’s SPD seeks to avoid layouts which locate living rooms, bathrooms and kitchens, next to, above, or below, proposed or neighbouring bedrooms.  The layout largely achieves this.

Sound Insulation - H8 refers to sound insulation between proposed and neighbouring units.  No sound insulation is proposed; this would only be a building regulations requirement where independent units were being created (self-contained flats).

Privacy and Overlooking - There are no new windows proposed and as such there are no issues with overlooking or loss of privacy.

Noise and disturbance: In terms of noise and disturbance there is no evidence to suggest that a small C4 HMO for up to four individuals would create significantly greater levels of noise than a fully occupied three bedroom family house.  A concern has been raised with regards to the noise from the fans within the en-suites, especially in bedroom 3.  This matter was raised with the agent and a new extractor fan has been installed in bedroom 3.

Bin storage: Bins are stored to the front of the property, although not ideal from an aesthetic point of view, as there is no side access to get to the rear garden there is no other viable option, without having to carry bins through the house.

(c) 	Affect car/cycle parking provision

The property is located in Zone 2, Primary Core Area, of the Revised Parking Standards and Design SPD.  This zone directly surrounds the Central Core and extends to walking distances of 2 kilometers from the centre of Reading. This zone is well served by public transport, with buses continuing either into or out of the Central Core Area via this zone. The property is located in a Residents Parking Permit Zone, Zone 10R.

In accordance with the adopted Parking SPD, the proposed HMO development would be required to provide 0.25 on-site parking spaces per bedroom totalling a required provision of 1 parking space for a 3-4 bedroom HMO.  There is no parking associated with the site and it is not possible to provide any off-street parking given the terraced nature of the street.

Conditions and informative will be included advising the applicant that any residents from the date of permission will not be automatically entitled to apply for a residents parking permit for the surrounding residential streets where parking is under considerable pressure. This will ensure that the development does not harm the existing amenities of the neighbouring residential properties by adding to the already high level of on street car parking in the area.

In accordance with the adopted Parking Standards and Design SPD, the development would be required to provide a cycle parking provision of 0.5 spaces per letting room. The agent submitted a cycle store drawing on 4th June 2020 which was acceptable to Transport.  The proposed cycle store measures 2.2m in depth, 1.3m in width with a maximum height of 1.9m and would be located to the front of the property directly in front of the ground floor bedroom window.  Officers understand the requirement for cycle storage to be provided however in this instance the proposed cycle store is considered detrimental to the appearance of the terraced street scene and harmful to the living environment of the occupier of the ground floor front bedroom and it will not be required in this particular application for the reasons given.

RECOMMENDATION:

APPROVE subject to the following conditions and informatives:

- AP1 - Approved plans

- HM1 - Tenants Management Plan (as submitted)

- HM2 - Retention communal space

- HM5 - HMO parking permits (notification to LPA)

- HM6 - HMO parking permits (notification to occupants)

Informatives:

- IF1 - Positive and proactive

- IF5 - Terms and Conditions

- I26 - Housing Act

- I13 - Parking permits