DELEGATED APPLICATION REPORT 					    Target Date: 17/02/20

Application No: 192034/HOU

Address: 4 Tofrek Terrace

Proposal: Part one, part two storey rear extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site comprises a two storey mid-terraced dwelling. A short gable projects to the rear. The surrounding area is predominantly residential. The neighbouring properties have been variously extended to the rear. The site topography slopes down gently in a northerly direction.

Proposal

The proposal is for a part one, part two storey rear extension. At the ground floor, the proposal would project to a depth of 5.55m, extending at a width of 6.75m. The single storey element would have a flat roof at a height of 3m. At the first floor, the proposal would project to a depth of 2.5m, extending at the same width as the ground floor. The two storey element would have a hipped roof with a maximum height of 6.9m and a height to eaves of 4.8m, set down from the existing ridgeline by 0.1m. A distance of 1m would be maintained to the boundary with 5 Tofrek Terrace. A distance of 0.6m would be maintained to the boundary with 3 Tofrek Terrace. To the rear elevation, a window and a set of bi-fold doors would be located at the ground floor with two windows located at the first floor. Materials have been selected to match those of the existing property.

Plans Considered

The agent submitted the following, received on 23/12/19:

Drawing No: 41619-09 – Site Location and Block Plan

Drawing No: 41619-01 – Existing Ground Floor Plan

Drawing No: 41619-02 – Existing First Floor Plan

Drawing No: 41619-03 – Existing Roof Plan

Drawing No: 41619-05 – Proposed Ground Floor Plan

Drawing No: 41619-06 – Proposed First Floor Plan

Drawing No: 41619-07 – Proposed Roof Plan

Drawing No: 41619-04 – Existing Elevations

Drawing No: 41619-08 – Proposed Elevations

The original proposal included a slightly deeper and wider single storey element. Following a site visit, the agent was advised on 19/02/20 of significant concerns held with the proposal. The proposal would be a significant overdevelopment of the original property, lacking in subservience and detracting from its character and appearance. Significant harm would also be caused to the residential amenity of the occupants of 3 Tofrek Terrace. The agent was advised that without significant amendments, the proposal was subject for refusal.

Subsequently the agent supplied the following, received on 11/03/20:

Drawing No: 41619-05 Rev. A – Proposed Ground Floor Plan

Drawing No: 41619-06 Rev. A – Proposed First Floor Plan

Drawing No: 41619-07 Rev. A – Proposed Roof Plan

Drawing No: 41619-08 Rev. A – Proposed Elevations

Relevant Planning History

None.

Consultation Responses

Neighbouring owners and occupiers at 3 and 5 Tofrek Terrace and Wilson Road Primary School were consulted by letter. Three letters of representation were received with regard to the following:

- Due to the slope, 3 Tofrek Terrace is below no. 4, and the extension would block light to the kitchen and bathroom windows and overshadow the rear decking area

- The proximity of the extension would make maintenance of 4 Tofrek Terrace impossible

- There is no clarity on the plans of proposed soil pipe runs

- Concern that the property would be converted to an HMO

- The extension would be overbearing, an overdevelopment, and would be detrimental to the health and wellbeing of neighbours

- The extension would take up half of the back garden

- The extension would cause a loss of light to the rear garden of 5 Tofrek Terrace

- Noise concerns as a result of the proposed development

Natural Environment

There are a number of trees within the front gardens of properties along Tofrek Terrace which are subject to Tree Preservation Order 108/07. One tree to the front of 4 Tofrek Terrace (T4 – Horse Chestnut) is no longer present and I cannot find any record as to why this tree was removed or that an application was submitted to the Borough Council for consent in order to carry out this work. This will be followed up as a separate matter directly with the current owners/occupiers of 4 Tofrek Terrace.

The proposed rear extension will be located well away from the protected trees, therefore I do not have any objection to the application and no tree/landscape conditions are considered necessary.

Ecology

Considering the location of the house, the extent of the works and the condition of the roof and its tiles, it is considered unlikely that the proposal will affect bats or other protected species. As such, there are no objections to the proposal on ecological grounds.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Plan (2019)

Policy CC7 – Design and the Public Realm

Policy CC8 – Safeguarding Amenity

Policy EN12 – Biodiversity and the Green Network

Policy EN14 – Trees, Hedges and Woodland

Policy H9 – House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

Design and impact on the character of the area

Policy CC7 states that all development must be of a high design quality that maintains and enhances the character and appearance of the area of Reading in which it is located. Policy H9 states that an extension should respect the character and appearance of the house and the pattern of neighbouring properties and appearance of the street as a whole in terms of scale, location, materials and design, the arrangement of doors, windows and other principal architectural features and any important existing building line. Extensions should not present an overbearing impact on neighbours and should not result in a loss of biodiversity within gardens. The Council’s Design Guide to House Extensions states that planning permission will not normally be granted for rear extensions that are longer than 4m, and that they should be located as far away from side boundaries as possible. The acceptability of rear extensions will be derived from the particular circumstances of the application site.

The proposal represents a significant enlargement of the original property, and considered cumulatively represents an overdevelopment of the host property. The original dwelling is of modest scale and proportion. At the ground floor, the proposal would more than double its original footprint. Though a slight reduction in depth and width have been made during the course of the application, the proposed extension will still lack subservience to the original dwelling, to the detriment of its appearance and character.

As above, the acceptability of rear extensions will be derived from the particular circumstances of the application site. During the course of the application, the agent was advised that while the proposal was deemed unsuitable there was scope to extend to the rear. Indeed, many of the properties within the terraced row have been variously extended, including over two storeys. As will be discussed below, due to the mid-terrace location, the impact of any extension will be keenly felt by neighbouring properties. Though a slight reduction to the proposal was made to the proposal during the course of the application, following officer advice, the identified harm was not sufficiently overcome. The proposal, for the reasons identified above, is therefore contrary to Policies CC7 and H9 of the Local Plan and the Council’s Design Guide to House Extensions.

Impact on neighbouring residential amenity

The proposed extension would be located 0.6m from the boundary with 3 Tofrek Terrace. This neighbouring property benefits from a two storey rear extension of approximately 2.5m depth. The flank elevation of this extension comprises the site boundary, with a bathroom window located at the first floor. The proposed two storey element projects to the same approximate depth as the extension at no. 3, before continuing to project by an additional 3m. Though the bathroom window of the neighbouring property does not constitute a habitable room, the impact of the proposed extension on this window would be stark, significantly reducing light and presenting visually dominant and overbearing effects. The total depth of the proposed extension would also be harmful to residential amenities of this neighbour generally.

The Proposed First Floor Plan includes an illustration to show that the proposed two storey element would not encroach on a line taken at 45 degrees from the midpoint of a window of the closest habitable room at 5 Tofrek Terrace. This is to ensure that no significant light loss would occur to this room. However, due to its depth, height and proximity to the boundary, the proposal is considered to have a visually dominant and overbearing effect on this neighbour. For these reasons, the proposal is contrary to Policy CC8 of the Local Plan and the Council’s Design Guide to House Extensions.

Other matters

As per the consultation response received from Natural Environment, the proposed extension would not cause harm to the protected trees to the site frontage. The proposal would therefore accord with Policy EN14.

Policy EN12 states that any development should not result in a net loss of biodiversity and geodiversity, and should provide a net gain for biodiversity wherever possible. No indication has been made how a net gain for biodiversity can be made as a result of the proposal. The proposal is therefore contrary to this policy.

RECOMMENDATION

REFUSE