# DESIGN, ACCESS AND PLANNING STATEMENT

IN SUPPORT OF

AN APPLICATION

#### FOR

QUEEN ANNE’S SCHOOL

READING
RG4 6DX

RELATING TO THE

DEVELOPMENT OF

A NEW ARTIFICIAL PITCH, FENCING

AND FLOODLIGHTS. ACOUSTIC FENCE.

ERECTION OF A PAVILION AND CHANGING ROOMS

FLOODLIGHTING OF TENNIS COURTS

ERECTION OF AN INDOOR TENNIS BUILDING

NEW CAR PARKING AND REVISIONS TO ACCESS

GROUND CONSOLIDATION TO REMOVE DIP IN THE NATURAL GRASS PLAYING FIELDS

# December 2019

### BODDINGTONS PLANNING

Chartered Planning Consultants,

Sports Development Specialists

Westfield House

31 Shirburn Street

WATLINGTON

Oxon. OX49 5BU

01491 614809

neil@boddingtonsplanning.com

Appendices to appraisal

Appendix 1  -	ILE Guidance notes

List of attached documents comprising the planning application.

- Design and Planning Statement

- OS extract 1:2500 with red line

- Proposed site plan

- Pavilions plans including floor plans and elevations

- Plan of new changing rooms including floorplan and elevations

- Noise report

- Ecology report

- Geotechnical report

- Drainage strategy and plan

- Plans and elevations of sport pitch

- Floodlighting diagrams

- Details of indoor tennis building

- Tree survey

- Pitch layout plans

- Plans of lighting column

- Pitch cross section

- Team shelter details

- Fence details

- Acoustic fence details

SUMMARY

The Masterplan

The application represents the culmination of two years’ work to draw together the sports masterplan for the school for the future.  It is the consolidation of the school’s main sports of tennis netball and lacrosse plus the introduction of other sports such as hockey.  It addresses the shortfall of changing for the swimming pool and the need to provide a central sports hub close to the pitches to allow viewing of the tennis courts and the new artificial pitch.  This building will contain changing facilities rehabilitation and storage rooms and a communal area for accommodating visiting teams and parents for teas close to the pitches.  The alteration of the access and the new parking area will take pressure off the existing main car park on sports fixture days.

The school has also been approached by Caversham Tennis Club who have expressed an interest in talking to the school about the possibility of joining with the school on the school grounds to make use of the existing and proposed tennis facilities.  These talks are at an early stage but there is some synergy between the ambitions of the school with regard to tennis and those of the Club for the future.  The needs of the tennis club are not yet fully known so the masterplan has taken little consideration of specific proposals that may come forward were the co joining come to pass but it is considered that any new proposals would not vary dramatically from this plan.

Proposals

- a new artificial pitch, sprint track, long jump pit, fencing and floodlights.

- erection of a pavilion and changing rooms

- floodlighting of tennis courts

- erection of an indoor tennis building

- additional car parking (including disabled) and revisions to access

- ground consolidation to remove dip in the natural turf playing fields

- Floodlighting. Lighting to artificial pitch and tennis courts as per minimum guidance for safe school play.

- Height of the columns. The columns would be to 10m on the tennis courts and 15m on the artificial pitch.

- Fencing. The fencing would be 3 metres in height of green plastic-coated rebound fencing around pitch and 4.5m behind the goals at either end.  This type of fencing is extremely visually porous. Details f the acoustic fence are included.

- Landscaping. Existing trees around the boundary of the site would be retained.  There is no potential for planting on the main grassed playing areas.  Planting in the areas adjacent to the courts would form part of a condition on the planning permission subject to agreement with ecologists as it is in the vicinity of the badger sett.

- Hours of operation.  The school is in an urban area on a busy road that generates both noise and lighting. It considers that the use of the floodlit facilities for the school would be until 6pm and a community use of the facilities thereafter. until 9pm on weekdays but not at the weekends.  This does not mean that it will be used every evening.

- Parking. There is no daytime parking connotation as the pitch will be for use of the pupils.  The existing car park by the school will be used as usual in connection with the school use of the facilities.  In the evenings all parking will be available for use.

- Spectators. This subject is a frequent objection by residents, but the reality is that the school currently has fixtures in the afternoon when they will play games against other school, when some parents will come to support in the same way as they do at present.   Parents will walk down from the school to the pitch to watch.  This does not generate poor behaviour or loud noise or large numbers.  They will be able to have a cup of tea in the pavilion after.  Games would take place within the normal and existing hours of the school.  Use of the pitch in the evenings will be for practice only and will not be used for formal games

- Drainage. The new pitches (ATP and open grass area) will be properly drained with a herringbone system across the slope. As the pitches will be less steep than the existing ground the drainage of the grass areas around will be improved as a soakaway will be located at the lower edge of the pitches.  Water run off does not occur on artificial pitches, as they are porous.  A detailed SUDS drainage strategy has been submitted.

- Sport England.  We have added the Sport England policies and consideration of the need to satisfy them. SE are particularly keen on the provision of community facilities.  We have had a pre application consultation with them and they have expressed support for the pitch so long as the remediation to the grass pitches is carried out.  The Council at pre app were loath to commit themselves to any opinion without the support of SE.  We are confident that they will support the scheme

- Ecology.  We have had a full emergent bat survey and badger survey carried on the site over the past year.  There are badgers on the site but little bat activity.  The consultants consider that the development would be likely to have little impact on the ecology of the site. As the proposals involve the demolition of the small pavilion close to the badgers, they are likely to be less disturbed as a result of the new facilities than previously.

- Pavilion. We have shown a pavilion of approx. 400m2 on two floors with a balcony.  The building would be located in the centre of the new sports hub overlooking the tennis courts and artificial pitch.  Due to its location it will not overlook any properties.

- Noise We have had a detailed noise assessment undertaken to assess the noise levels likely to be produced from the artificial pitch and tennis courts.  Tennis is known for being a quiet game.  The assessment of the artificial pitch has been done without the presence of the indoor tennis facility as the artificial pitch is likely to be the first element implemented so we have taken the worst-case scenario in respect of impact on neighbours.  The noise survey indicates that the provision of a 2m acoustic fence within the boundary of the school site will provide for noise levels commensurate with the recommended levels.

- Geotechnical work. Detailed analysis including boreholes has been undertaken in respect of the worsening dip in the ground.  It has concluded that there is an underground water source running through the chalk layer eroding it away.  The dip has been consistently falling over the past 30 years.  Attached is a photo of the fields in the 1960’s.  Work is urgently required to stop the erosion and to ensure that the playing fields are retained.  Currently one pitch is located in the dip and it provides a substandard area for sport.  We propose to excavate the soils and insert a geotextile layer into the ground above the chalk that will be anchored into the surrounding ground.  This will hold the infill material that will be imported to raise the levels of the ground.  Drainage will be inserted into the ground to ensure that the ground drains correctly but also to ensure that penetrating water does not allow erosion to occur anew.  The solution has been designed to ensure that there is minimal movement in the grounds over the next 50 years.  There will be an increase in the quality of the natural turf playing surface as a result.

SECTION 1 – INTRODUCTION

  - This appraisal accompanies the planning application for the development given above at Queen Anne’s School Reading.

  - Boddingtons Planning is an established company with considerable experience in sports development especially in artificial and natural turf surfaces and floodlighting.  They have been involved in many of the developments within the world-class programme for hockey involving the planning design and project management of new pitches countrywide for hockey, tennis, football and other sports.  They have been involved in the planning and design and construction of approx. 200 pitches countrywide.  The location of pitches includes Green Belt, Conservation Areas, AONB’s and within the setting of listed buildings. We have been responsible for the design planning and development of pitches in the close vicinity including The Abbey School Kendrick Road (in Reading BC), Crosfields School Reading, Reddam House School, Reading HC, Sonning HC, (in Wokingham DC) Langtree School Woodcote, Oratory Prep School, and Wallingford Grammar (in SODC)

  - The purpose of this appraisal is to consider the key issues that are raised in the development of pitches such as that proposed.  We are confident that all issues will be overcome leading to a valuable asset for the development of sport in the School and for the health and wellbeing of pupils and the community.

1.05.	We carried out a pre-application enquiry with the Council in respect of a full-sized pitch with floodlighting for the pitch and tennis courts last year.  The Council felt that they could not offer an opinion on the acceptability of the pitches because there were queries raised by Sport England in respect of the detail of the proposals.  We have now consulted with SE and they have indicated that they would be supporting the scheme for the new pitch and th remediation of the natural turf areas.   We feel that we have covered the points raised to make a comprehensive and justified application

SECTION 2 - PLANNING POLICIES

2.01	The National Planning Policy Framework 2019 seeks to establish the ‘golden rule’ of allowing sustainable development which affects the development of the neighbourhood to be approved without delay.

It states:

14. At the heart of the National Planning Policy Framework is a presumption in favour of sustainable development, which should be seen as a golden thread running through both plan-making and decision-taking.

- For plan-making this means that:

- local planning authorities should positively seek opportunities to meet the development needs of their area;

- Local Plans should meet objectively assessed needs, with sufficient flexibility to adapt to rapid change, unless:

- any adverse impacts of doing so would significantly and demonstrably outweigh the benefits, when assessed against the policies in this Framework taken as a whole; or

- specific policies in this Framework indicate development should be

restricted.

For decision-taking this means:

- approving development proposals that accord with the development plan without delay; and

- where the development plan is absent, silent or relevant policies are out‑of‑date, granting permission unless:

- any adverse impacts of doing so would significantly and demonstrably outweigh the benefits, when assessed against the policies in this Framework taken as a whole; or

- specific policies in this Framework indicate development should be

- restricted.

15. Policies in Local Plans should follow the approach of the presumption in favour of sustainable development so that it is clear that development which is sustainable can be approved without delay. All plans should be based upon and reflect the presumption in favour of sustainable development, with clear policies that will guide how the presumption should be applied locally.

8. Promoting healthy communities

69. Planning policies and decisions should aim to achieve healthy, inclusive and safe places which (inter alia c) enable and support healthy lifestyle, especially where this would address identified local health and well-being needs – for example through the provision of ….sports facilities…

92. To provide the social, recreational and cultural facilities and services the

community needs, planning policies and decisions should:

a) plan positively for the provision and use of shared space, community

facilities (such as local shops, meeting places, sports venues, cultural

buildings, public houses and places of worship) and other local services to

enhance the sustainability of communities and residential environments.

As such in connection with the previously approved land levelling the current proposal would provide additional/improved facilities for the school assisting in providing a more consistent surface for hockey and other sports and a better-quality facility overall. In this regard the proposal would support the facilities available at the school for outdoor sport and recreation and this may contribute towards the set of circumstances as to the need for the proposed development

Open space and recreation

96. Planning policies should be based on robust and up-to-date assessments of the need for open space, sport and recreation facilities and opportunities for new provision

97. Existing open space should not be built on unless (inter alia)

c) the development is for alternative sports and recreation provision, the benefits of which clearly outweigh the loss of the current or former use

Paragraph 145 allows for the provision of appropriate facilities for outdoor sport and recreation as long as it preserves the openness of the Green Belt and does not conflict with the purposes of including land within it.  In principle therefore the provision of open-air sports facilities and attendant modest ancillary facilities are in principle acceptable in the Green Belt.

The construction of new buildings in the Green Belt is inappropriate unless it is for certain specified purposes. The NPPF makes it clear that an essential characteristic of Green Belts is their openness. Openness has been described as an ‘absence of development’ irrespective of the degree of visibility of the land in question from public vantage points. Therefore, any development will have an impact on openness and the degree to which that impact is either acceptable or unacceptable will be matter of fact and degree based on the merits of each case. The land is already used as playing fields; ancillary to the use of the school. The proposal seeks to retain and improve the existing sports pitches and therefore this would not comprise a material change of use of the land.

2.02 		Reading BC Core strategy

Policy CS7: Design and the Public Realm

All development must be of high design quality that maintains and enhances the character and appearance of the area of Reading in which it is located. The various components of development form, including: -

• Layout: urban structure and urban grain;

• Landscape;

• Density and mix;

• Scale: height and massing; and

• Architectural detail and materials.

will be assessed to ensure that the development proposed makes a positive contribution to the following urban design objectives: -

• Character - a place with its own identity and sense of place

• Continuity and enclosure

• Quality of the public realm

• Ease of movement and permeability

• Legibility - clear image and easy to understand

• Adaptability – capable of adaptation over time

• Diversity – meets a wide range of needs.

Developments will also be assessed to ensure that they: -

• Respond positively to their local context and create or reinforce local character and distinctiveness, including protecting and enhancing the historic environment of the Borough and providing value to the public realm;

• Create safe and accessible environments where crime and disorder or fear of crime does not undermine quality of life or community cohesion;

• Address the needs of all in society and are accessible, usable and easy to understand by them; and

• Are visually attractive as a result of good high-quality built forms and spaces, the inclusion of public art and appropriate materials and landscaping.

Applications for major and minor developments should be accompanied by a design and access statement that deal with all the above matters

Commentary: We have great experience in the design of the facilities proposed and any likely impact on local residents.  We are able to advise on the best type of fencing to suit the games played on the pitch and the ‘visibility’ of that fencing in the long and short views from outside the site.  Existing boundary fencing and vegetation will be maintained and will be supplemented where necessary.

Additional and Existing Community Facilities

Policy CS31: Additional and Existing Community Facilities

Proposals for new, extended or improved community facilities will be acceptable, particularly where this will involve co-location of facilities on a single site. Community facilities should be located where there is a choice of means of travel (including walking and cycling), and in existing centres where possible.

Proposals involving the redevelopment of existing community facilities for non-community uses will not be permitted, unless it can be clearly demonstrated that there is no longer a need to retain that facility.

Commentary: the need for the pitch for the school is to ensure that the quality of playing facilities is enhanced   The school plays games and trains in late afternoon so floodlighting will be needed especially in winter months.

Policy CS34: Pollution and Water Resources

Development will only be permitted where it would not be damaging to the environment through air, land, noise or light pollution; where it would preserve or ideally enhance ground and surface water quality; and where existing water resources, sewerage and wastewater treatment infrastructure are adequate to support the proposed development.

Proposals for development that are sensitive to the effects of air, noise or light pollution will only be permitted in areas where they will not be subject to high levels of such pollution, unless adequate mitigation measures are provided to minimise the impact of such pollution.

Development will be permitted on land affected by contamination where it can be demonstrated, to the satisfaction of the LPA, that the contamination can be satisfactorily remediated so that it is suitable for the proposed end use.

Commentary: Noise and light pollution are the two main objections brought by the residents in opposing recreational facilities such as these.  We have carefully considered these aspects and have taken expert advice on the impact.  However, as with many other developments the perception of neighbours of these matters is worse than reality.  The school is always keen to be good neighbours and any activity held by the school will be supervised by the staff.  Outside lettings can be terminated if there is any issue of behaviour problems.  Outside lettings will have to sign a contract accepting this issue and they will be held responsible for such actions should they occur.  However, the evidence of other pitches is that noise is not usually an issue.  Light pollution is easily said but the modern lights are very sophisticated, keeping the light concentrated on the pitch and not throwing light in all directions.  We are able to keep light spill to a minimum and in line with the recommendations of the Institute of Lighting Professionals (ILP), without any interruptions from hedging etc which we intend to plant.  These parameters are well defined.  As the floodlights are not intended to be used late at night the disturbance by light will be minimal.

Biodiversity and Geology

CS36: Biodiversity and Geology

a) Development should retain, protect and incorporate features of biodiversity or geological interest (including protected species and their habitats) found within the application site into their schemes.

On sites with recognised biodiversity or geological value, development will not be permitted where there would be a direct or indirect adverse impact on the site, unless it can be clearly demonstrated that: -

i. The need for development clearly outweighs the need to protect the value of the site; and

ii. Appropriate compensation, impact minimisation, mitigation and compensation are provided.

b) Local Nature Reserves and Wildlife

Heritage Sites will be safeguarded and where possible, enhanced. Permission will not be granted for any development that would adversely affect a designated nature reserve or Wildlife Heritage Site.

c) Any development that would sever or threaten the integrity of an established wildlife link, as indicated on an adopted proposals map, will not be permitted. Where applicable, developments should be designed to protect, consolidate, extend and enhance the network of wildlife links and corridors in and adjoining the Borough, working with neighbouring authorities where appropriate.

Commentary:  This is a well maintained and used school field.   We have assessed the site for the potential of roosting bats and badgers and it is considered that there is no threat to any bats as a result of the proposals.  The report is attached.

2.03	SPORT ENGLAND

The new Sport England Design Guidance Note 2018 states:

Advantages of artificial surfaces

The general advantages of artificial pitches over natural grass can be summarised as allowing:

• Greater durability

• More efficient use of space

• Increased usage 2

• Flexibility

• Better overall value for money

• Defined performance characteristics.

A further advantage is that less maintenance is required for a given level of use when compared to natural grass. However, it is a myth that artificial playing surfaces are maintenance-free. For example, an artificial grass pitch is likely to have the same annual maintenance cost as a turf pitch but be able to take ten times as much use.

The Policy on Planning Applications for Development of Playing Fields.

Playing fields are one of the most important resources for sport in England. They provide the space which is required for the playing of team sports on outdoor pitches. Yet as open land, particularly in urban areas, becomes an increasingly scarce resource, they often seem to offer a tempting opportunity for other forms of development. Sport England opposes such development in all but exceptional cases, whether the land is in public, private or educational use. It does so because the loss of any part of a playing field may represent the irretrievable loss of an opportunity for participation in pitch sports, and with it the many benefits which sport brings.

SE defines playing fields and pitches as follows:

D1 “Playing field – the whole of a site which encompasses at least one playing pitch”.

This definition is given in the 1996 Order. Sport England’s policy is to protect all parts of a playing field, not just those which happen, for the time being, to be laid out as pitches. This is because those other parts of a playing field are a resource which may be needed, now or in the future, and it is important that they be afforded the same protection.

D2 “Playing pitch – a delineated area which, together with any run off area, is of 0.2 hectares or more, and which is used for association football, American football, rugby, cricket, hockey, lacrosse, rounders, baseball, softball, Australian football, Gaelic football, shinty, hurling, polo or cycle polo.”

This definition is also found in the 1996 Order. Golf courses, tennis courts and bowling greens are not included in the definition. Playing pitches may have a grass surface or an artificial one.

Its policy on playing fields is as follows:

P1 Sport England will oppose the granting of planning permission for any development which would lead to the loss of, or would prejudice the use of, all or any part of a playing field, or land last used as a playing field in an adopted or draft deposit local plan, unless, in the judgement of Sport England, one of the specific circumstances applies.

P1.1 Reason: Development which would lead to the loss of all or part of a playing field, or which would prejudice its use, should not normally be permitted because it would permanently reduce the opportunities for participation in sporting activities. Government planning policy and the policies of Sport England have recognised the importance of such activities to the social and economic well being of the country.

The specific circumstances are: E1 A carefully quantified and documented assessment of current and future needs has demonstrated to the satisfaction of Sport England that there is an excess of playing field provision in the catchment, and the site has no special significance to the interests of sport.

E.1.1 Reason: Sport England does not seek to impose a blanket restriction on the development on playing field sites if it can be demonstrated that there is an excess of provision which would still be the case after the development of a particular site. In order to demonstrate an excess of provision the applicant, or local planning authority must produce written evidence based upon a comprehensive and carefully documented assessment of supply and demand in the catchment based upon a methodology acceptable to Sport England. This assessment should make allowance for other development for which land has been, or is likely to be, allocated in a development plan or for which planning consent has been or is likely to be granted. E1.2 Sport England may object to the loss of a site, which has special significance, such as one of exceptional quality or importance to the development of sport, even if it can be shown that there is an excess of provision within its catchment.

E2 The proposed development is ancillary to the principal use of the site as a playing field or playing fields, and does not affect the quantity or quality of pitches or adversely affect their use.

E2.1 Reason: Sport England would not normally object to the provision of new/improved changing accommodation and related facilities of an appropriate scale, providing that any detrimental effect on the principal use is not significant.

E3 The proposed development affects only land incapable of forming, or forming part of, a playing pitch, and does not result in the loss of or inability to make use of any playing pitch (including the

maintenance of adequate safety margins), a reduction in the size of the playing areas of any playing pitch or the loss of any other sporting/ancillary

facilities on the site.

E3.1 Reason: The development of minor parts of a site (e.g. frontage land, steep slopes unsuitable for pitches) can provide a useful source of capital for sports organisations to develop their activities without detriment to the facilities. In order to avoid incremental loss of land, Sport England will use the nature and extent of provision available at any time during 1996 as a base for all assessments.

E4 The playing field or playing fields, which would be lost as a result of the proposed development, would be replaced by a playing field or playing fields of an equivalent or better quality and of equivalent or greater quantity, in a suitable location and subject to equivalent or better management arrangements, prior to the commencement of development.

E4.1 Reason: There may be occasions when a replacement facility can be provided which represents an adequate substitute for, or improvement on, the existing facility and where it would be beneficial for sporting interests to take advantage of this opportunity. It would be expected that the replacement facility is secured by way of a legal agreement between the applicant and the planning authority, and that it will be available for use prior to the commencement of any development. Sport England will not apply this exception unless the management arrangements for the new site are equivalent to or better than those for the site to be developed.

E5 The proposed development is for an indoor or outdoor sports facility, the provision of which would be of sufficient benefit to the development of sport as to outweigh the detriment caused by the loss of the playing field or playing fields.

E5.1 Reason: There may be occasions when the development of a facility for sport, whether involving the construction of indoor facilities or

a change in the use of open land, would lead to the loss of all or part of a playing field. If Sport England is satisfied that the development proposal represents an opportunity to obtain benefits for sport which are, in its judgement, unlikely to be obtainable in any other way in the foreseeable future, and that the benefits which will be obtained will outweigh any predictable detriment to sporting interests caused by the loss of playing fields, then it may not object to the proposed development. 
SECTION 3 – DESCRIPTION OF THE PROPOSALS

  - As described above the proposals the subject of this application seek the following:

- a new artificial pitch, sprint track, long jump pit, fencing and floodlights.

- erection of a pavilion and changing rooms

- floodlighting of tennis courts

- erection of an indoor tennis building

- additional car parking (including disabled) and revisions to access

- ground consolidation to remove dip in the natural turf playing fields

3.02	We foresee the following issues may be pertinent in this case:

3.03	  FLOODLIGHTING

From past experience, floodlighting is generally one of the major issues in development proposals.  People’s perception regarding floodlighting often relates to the huge multi-light pylons found at each corner of a football stadium.  However, the technology of floodlighting has progressed significantly in the past years to the stage where lighting is now capable of strict control and the throw of light is directed on to the playing surface alone with a very low overspill into the surrounding area. Light spill calculations are made on the basis of a flat plane, taking no consideration of fencing or landscaping/ mounding etc.  This scheme clearly indicates the rapid drop of light spill and the high degree of containment of light obtained.  Taking into account the vegetation and fencing around the site that will further interruption the spill of light we are confident that there would be little or no out spill of light from the site from these proposals.

3.04	The proposal is to use the latest LED lighting system, the details of which are including.  This is a constantly evolving state of the art lighting system that produces a high performance for the playing surface but strictly controls the throw of light.  It is a leader in its field and is only recently on the market.  The diagrams submitted indicate the position of the floodlighting columns and the orientation of the lamps assuming that all lights are on at the same time.  Also shown is the photometric survey indicating the intensity of light on the pitch.  This information shows the lighting to be a minimum but appropriate level of luminance for the standard of sport required to be played safely. The lights can also be altered on the new pitch to have a lower level which will be used for training and larger ball sports.

The floodlighting scheme indicates that minimal light will fall into the gardens of the residents, even assuming no landscaping or other visual interruption on site, such as fencing, of which there is a substantial amount.  The ILE requirements relate to light against the windows of houses, so we comply with their standards.

3.05	HEIGHT OF THE COLUMNS

It is generally accepted that the higher the column the more accurate the light control on to the pitch.  This is because the light is shone from an almost horizontal position downwards using the shape of the lamp to direct and control the light.  In this case, due to the size of the pitch and the low levels of luminance required, the pitch will be properly lit by an 15m column.  The columns are a galvanised silver colour and will blend into the urban background of the residential area around.   The tennis Courts will be lit by 8m high columns

3.06	NOISE

The pitch will be located on the existing playing field where there is already activity and noise.  It does not appear to have been an issue with residents before.  However, games take place during the school day and will be completed when the school activities cease around 6pm.  The school already has a significant community use of the facilities including the sports halls and pools.  This will add to the community use.  The noise report indicates that the noise will be within WHO standards

3.07	Fencing

It is proposed that the fencing surrounding the pitches would be 3m high green plastic-coated rebound fence around the sides of the pitch, the two ends being 4.5m to prevent balls from existing the pitch when shooting at goal.  There would be only one double gate entrance to the pitch.   The fencing is very visibly permeable, and we attach photos of the fencing demonstrating this.

3.08	We consider that there would be no adverse impact from appearance the proposed fencing to cause undue amenity loss especially taking into consideration that the boundary landscaping.

3.09	PAVILION

The pavilion would be approx. 400m2 on two floors.  The ground floor would primarily be changing rooms and toilets and the upper floor would be a meeting space where parent would have teas and, in the event, that the tennis club joins would be their club room.  There would a balcony that overlooks the courts and the p[itch. A sit is in the centre of the site it will not overlook anyone.

3.10	 THE INDOOR TENNIS FACILITY.  The tennis ‘bubble’ would cover the courts at the end of the block of courts closest to the road.  Along the boundary at this point is a row of evergreen cypress trees that will screen it from the road.  These would be retained.  The bubble sits on a raft foundation running as a strip along each length so would not harm their retention.  The building would be 54m long and 9m high.  The bubble would also form a function in screening the lights from the tennis courts from the road as well as the artificial pitch from the nearest houses on Henley Road

3.11	PARKING/INCREASED ACTIVITY

Parking would continue to take place on the parking area to the front of the school.  There would be no increase in pupils as a result of the development and there is sufficient parking for the school needs.  In the evenings parking would take place in the new car park for the pitch and court users although the main car park will still be open for use.

3.12	Hours of Operation

It is customary for planning permission for floodlighting to contain a condition restricting the hours of operation.  The cut off time proposed is 2100hrs, which is in middle evening.   At the weekends the use of the pitch would cease at 1900hrs as would the lights. The tennis courts are screened by the boundary hedging from resident’s opposite.  Tennis is a quiet game so evening noise will not be an issue and the lights will be contained within the boundary so even the closest resident will not be affected.

3.13	ECOLOGY

Circular 06/2005 (Biodiversity and Geographical Conservation - Statutory Obligations and Their Impact Within the Planning System) states that "it is essential that the presence or otherwise of protected species, and the extent that they may be affected by the proposed development, is established before planning permission is granted otherwise all relevant material considerations may not have been addressed...".

The presence of protected species is therefore a material planning consideration, which needs to be addressed if there is a reasonable likelihood of species being present and affected. The types of applications include proposals involving floodlighting of green spaces within 50m of woodland, water, field hedgerows, or lines of trees with obvious connectivity to woodland or water.

We have carried out a detailed bat survey including three dusk or dawn visits and placed cameras in the vicinity of the badger sett.  Our consultants are content that the lighting will not disturb either of the protected species.

  - SPORT ENGLAND

The proposal is for additional sport facilities is on land that for the most part is not on the existing playing field.  No pitches in either the winter, spring or summer terms will be lost as a result of the development, one lacrosse pitch will be replaced by an artificial pitch.  This is in accord with the Sport England Policies.  They count artificial pitches as pitches so there will be no loss of sport pitches.  The tennis ‘bubble will be an improved facility and there wold be no loss of outside courts. The floodlighting will allow greater access and use. The artificial pitch will allow new sports to be played that are not otherwise available, particularly hockey, used more frequently and flexibly, improving and increasing participation.

3.15	The policies of Sport England are mentioned above.  They are tasked with protecting sport pitches from other forms of development.   The documentation above does not make it clear what type of development but presumably it would relate to non sport development.  Policy D2 specifically refers to artificial surfaces as part of the definition of playing fields and pitches.  As this is for an artificial surface, which can be used much more intensively and is more flexible than grass pitches, this proposal stands full square with their policies.  There is no doubt that artificial turf can take a number of sports and should be specifically designed for the use by the applicant.  This facility will be used for hockey which is these days not played on natural grass.  Its shortcoming must also be balanced by the greater intensity of use that can be obtained on and artificial surface.  Therefore, we consider that the development of the site for an artificial pitch complies with the policy of SE in its own right.  The replacement of grass hockey pitches with artificial is appropriate as hockey is only played on artificial surfaces.

3.16	However, it appears that SE see artificial pitches as ‘other development’ despite clearly allowing artificial pitches within the definition of sport pitches and the other exception policies come into play where it has to be considered that the benefits outweigh the disbenefits.  Research has shown that a grass pitch can take 2-3 games a week maximum, but that artificial turf can take at least ten times that amount, indeed its use for up to 40 hours a week is normal.  In this case the new artificial pitch will the be considered to be an ‘added over’ in terms of sports provision.

3.17	SE seeks to encourage the wider use of these facilities for the community.  As always, the use at a later time is a balancing act between the benefit to users and the amenity of the local residents and the area around the site as well as the location of the pitch within the grounds of the school.  We have the opportunity to allow the use of the facilities for community use into the evening.  We consider that 2100 hours is a reasonable close down time.

3.18	We carried out a pre application with Sport England who indicated that they considered that the proposals were acceptable subject to conditions and complied with their exception policy (even though they did not need to).  The suggested conditions relating to

- Details of the ground conditions and topography.  We have submitted a topographic survey before and after he proposals.  We have submitted geotechnical details of the area to be affected by the remediation.  The remaining areas will remain as grass pitches and we will review those pitches at a later date.

- A Community Use Agreement to be signed.  We attach a CUA for consultation

- Management and Maintenance.  We attach maintenance details.  The management will be by the School who have appropriately qualified in-house staff who manage all their outdoor facilities.  The pitch will be installed by an appropriate contractor and overseen by an experienced project manager to ensure the quality of the work is of the best quality. The school will set up a sinking fund provided for by the community hire and from a sports fund that will be added to each year to ensure that an appropriate amount is available to ensure re-carpeting and repairs can be ongoing.  They are taking advice from an experienced project supervisor who is experienced in this field and who will ensure all works comply with the technical design guidance that is available from Sport England.

- As part of the proposals we have submitted a specification sheet and sample of the type of artificial pitch proposed, which is in accord with discussions with SE.

We trust that these submissions will be sufficient to avoid the need for conditions to be added.

3.18	TRAFFIC & HIGHWAYS

We believe there will be no connotation in respect of traffic and highways as there will be no additional traffic generated by this facility that does not already go to the school during daytime.  In the evenings the use will be less intense and there is more than sufficient parking on site to accommodate the community use as well as that existing.  The access in Grosvenor Road exists but inside the site has a kink taking across the front of the staff house.  We propose to move the gate piers approx. 3m to the north to allow the drive to be straight into the site.  It will not affect the appearance outside the site nor will affect vision splays from the site.  It will remove the drive from running immediately in front of the school’s house.

3.19	CHANGING ROOM

A small ‘lean to’ type changing room is proposed adjacent to the swimming pool building to improve facilities at this part of the site.  This will allow 2 small changing rooms with showers and toilets.  This will be used as an overflow for the other changing rooms used by the school and in the event of their need for community use.  The siting is on an existing substantial concrete base that will be reused.

3.19	SUSTAINABILITY

The NPPF requires proposals to be sustainable.  This is a site close to the centre of Reading on a site that is well served by public transport.  The site is therefore sustainable.

3.20	REMEDIATION OF THE NATURAL TURF PITCHES.

The grass pitches have a substantial dip in the land to the west of the site.  This dip has been subsiding over the past 50 years from being almost flat to be a substantial hollow.  We have taken geotechnical advice on the cause of this subsidence and the remediation that can be taken to arrest this movement.  The report suggests that the subsidence has occurred by the washing away of the chalk subbase as a result of a water course or the like below ground.  The proposal is to excavate down to the chalk and to insert a geotextile membrane anchored into the surrounding land to consolidate the land and to prevent the land from being washed away.  This is a major piece of land remediation but will lead to the resumption of the previous levels on the site and the improvement of the playing fields including the addition of a good quality pitch to replace the wet and boggy pitch that is currently in this location.

2.21	ACOUSTIC FENCE

An acoustic fence will be installed by the houses in the comer pf the site to ensure that we comply with the provision of the WHO requirements in respect of the use of the artificial pitch

SECTION 4 - CONCLUSIONS

4.01	The proposal has been carefully considered and planned to take into full account the sensitivity of the area in respect of locating facilities.  There would be limited visibility of the facilities from outside the site due to the location of houses in the vicinity and the mature planting that surrounds the site. The proposals will enhance the school facilities and will benefit all sports.

4.02	A high quality lighting system is proposed which will direct the light down onto the field of play and not into the dark sky.  The light spill will not extend outside the site and will be further constrained by the landscaping around the site.  Information included by the ILE indicates that this is not light pollution and that we comply with the guidelines that are taken as standard for compliance these days

  - There are no traffic or parking issues. A new area for parking will be introduced in the event that the community use of the site proves popular and requires additional spaces.  Th e existing access will be improved to allow access to and from the site.  The main car park will still be used for the majority of traffic movements.

  - The pitch and courts have been designed to enable more sport and recreation on this site.  The capacity of the new pitch will allow far more use of the playing fields both in terms of intensity and also in terms of then number of uses.  The ability to use floodlights in the winter times for a restricted period will enable clubs and teams to practice without travelling to other venues.

  - The pitch has been located so that there will be no damage to or impact on the trees surrounding.  The new indoor facility will be screened by the existing trees.  It will in turn screen the tennis courts from the road.

  - The site is sustainable and conforms to NPPF policy.  The pitch and floodlights is supported by Sport England following a pre application with them.  We see no connotations to them in respect of the other facilities proposed.