DELEGATED APPLICATION REPORT 					    Target Date: 17/10/19

Application No: 191276/HOU

Address: 172 Barnsdale Road

Proposal: Part one, part two storey side and rear extensions and erection of front porch

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a two storey detached dwelling with a pitched roof, in a chalet style. A garage extends beyond the southward side elevation, projecting to the rear of the dwelling. The dwelling is set within a sizeable plot, with a long rear garden. The surrounding area is predominantly residential, comprised of dwellings of various styles and design.

Proposal

The proposal is for part one, part two storey side and rear extensions and the erection of a front porch. The existing garage would be demolished to facilitate the proposal. At the ground floor the proposal would extend at a width of 2.9m beyond the original southward side elevation, flush to the principal elevation. The proposal would project at this width to a total depth of 12.9m, 4.5m beyond the original rear elevation. The proposal would extend across the full width of the original dwelling at this depth. A distance of 0.15m would be maintained to the boundary with 174 Barnsdale Road. The rearward single storey part would have a flat roof at a height of 3m. A roof lantern would project to an additional height of 0.3m. Single windows would be located to the front and southward side elevation. Two windows and a set of bi-fold doors would be located to the rear elevation. To the northward side elevation an existing window would be removed and replaced with a further window and single door.

Beyond the southward side elevation, the proposed first floor extension would extend at the same width as the ground floor, set back from the principal elevation by 1.2m. This part would project to a depth flush to the original rear elevation, with a pitched form to the forward roof slope and box-dormered to the rear. This part would be set down from the original ridgeline by 0.5m. A separate dormer window would be located to the forward roof slope, set down from the proposed ridgeline by 0.7m and set back from the proposed eaves by 1m. The first floor part to the rear would also have a box-dormer form, extending across the full width of the original dwelling, projecting to a depth of 4m. Two single windows and a Juliet balcony would be located to the rear elevation. The external faces of the dormers would be clad in Cedral click boards. Other materials have been selected to match those of the existing dwelling.

The proposed porch would project forward of the principal elevation to a depth of 1.3m, extending at a width of 2.3m. The porch would have a pitched roof with a maximum height of 3.5m and a height to eaves of 2.3m. The main entrance doors would be located to the front elevation, with windows to each of the side elevations.

Plans Considered

The agent submitted the following, received on 22/08/19:

Drawing No: 1951/PL/07 – Site Location, Existing and Proposed Site Plans

Drawing No: 1951/PL/01 – Existing Floor Plans

Drawing No: 1951/PL/04 – Proposed Floor Plans

Drawing No: 1951/PL/02 – Existing Roof Plan

Drawing No: 1951/PL/05 – Proposed Roof Plan

Drawing No: 1951/PL/03 – Existing Elevations

Drawing No: 1951/PL/06 – Proposed Elevations

Relevant Planning History

181162/HOU – Part one, part two storey side and rear extension – Permitted 11/02/19

Consultation Responses

Transport Development Control

The proposal would result in the loss of the garage. In accordance with the Council’s Parking SPD, two off-road parking spaces are required. Plans have not been submitted illustrating the proposed parking layout.

Neighbouring owners and occupiers at 170 and 174 Barnsdale Road, 42 Cressingham Road and Flats 1-4, 152 and 154 Windermere Road were consulted by letter. No letters of representation were received.

Summary of discussions with applicant/agent

The agent was advised on 04/10/19 of significant concerns with the application, which was subject for refusal. The agent was invited to withdraw the application, but no response has been received.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Development Framework Core Strategy (2008)

Policy CS7 - Design and the Public Realm

Reading Borough Local Development Framework Sites and Detailed Policies Document (2012)

Policy DM4 - Safeguarding Amenity

Policy DM9 - House Extensions and Ancillary Accommodation

Policy DM12 – Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Emerging Reading Borough Council Local Plan (2019)

The new Reading Borough Council Local Plan is at an advanced stage. The Inspector’s report into the Proposed Modifications has now been received and the new Local Plan is proposed to be adopted by the Council on 4th November 2019. The following polices are of relevance to this application:

Policy CC7 – Design and the Public Realm

Policy CC8 – Safeguarding Amenity

Policy H9 – House Extensions and Ancillary Accommodation

Policy TR3 – Access, Traffic and Highway-Related Matters

Appraisal

Design and impact on the character of the surrounding area

Policy CS7 of the Core Strategy requires that all development be of a high design quality that maintains and enhances the character and appearance of the area of Reading in which it is located. Policy DM9 of the Sites and Detailed Policies Document states that an extension should respect the character of the house and the pattern of neighbouring properties in terms of scale, location, materials and design, and any important existing building line. The Council’s Design Guide to House Extensions states that planning permission will not normally be granted for rear extensions that are longer than 4m, and that they should be located as far away from side boundaries as possible. The acceptability of rear extensions will be derived from the particular circumstances of the application site. The Design Guide also states that two storey side extensions should normally be designed to be smaller in scale than the main house. This can be achieved by setting the proposed front elevation further back than the front elevation of the original house and by bringing the upper floor away from sideward boundaries. This should result in a ridgeline significantly lower than that of the original house, with the overall effect being an extension that is in proportion with the original house and the surrounding properties. Roof design should complement the roof of the original house in terms of design and angle of pitch. With regard to porches, the Design Guide states that permission will not normally be granted for extensions that are excessive in size in terms of height and depth relative to the dwelling, which fail to demonstrate a balance of proportions.

In assessing the application, officers are mindful of the recently approved extensions under 181162/HOU. The footprint of the side and rear extensions proposed under this application are the same as that approved under 181162/HOU. The key changes between the applications are the enlarged box dormer to the rear part of the side extension, and the replacement of the rearward pitched form with a large box dormer. The dormer window to the forward roof slope would be altered slightly.

The application site is a two storey detached dwelling, in a chalet style, set within a sizeable plot. The extensions approved under 181162/HOU represent a substantial enlargement of the original dwelling, at the limit of acceptability in terms of retaining subservience. Though there would be no increase in the footprint of the extensions, the current proposal significantly increases the useable floorspace within the dwelling, facilitated by the provision of large, boxy dormers to the rear. Where the previously approved extensions comprised a conventional first floor side extension with relatively modest front and rear dormers, and a continuation of the original pitched roof form to the rear, the proposed extensions are significant in both scale and visibility from public areas.

The proposals are considered to be an overdevelopment of the original dwelling, lacking subservience to it. This is exacerbated where the first floor part projects beyond the original northward roof plane. This element would be clearly visible from public areas, an incongruous feature, jarring in the street scene. The rearward dormer extensions would bear no visual relationship to the original roof form. In addition, the proposed cladding is not considered to complement the original buildings appearance. The resultant appearance of the extensions would therefore detract from the character of the original dwelling, to the detriment of the character of the area.

In the context of the previously approved extensions, the altered front dormer window and provision of porch are considered acceptable. However, due to the harm identified above the proposals would be contrary to Policy CS7 of the Core Strategy, Policy DM9 of the Sites and Detailed Polices Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

Impact on residential amenity

Under application 191276/HOU, it was acknowledged that the side extension would extend to within 0.15m of the boundary with 174 Barnsdale Road; however the depth of the first floor would be contained within the side elevation of the original dwelling. This was considered sufficient to mitigate any harmful impacts on the residential amenities of this neighbouring dwelling. The current proposal adds significant bulk to the sideward part. This squaring of the extension is considered to cause visually dominant and overbearing effects to occur to the detriment of the amenity of residents of 174 Barnsdale Road. The additional scale and bulk of the rearward part is also considered to have a similarly harmful impact on the amenities of 170 Barnsdale Road. The proposals are therefore contrary to Policy DM4 of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

RECOMMENDATION

REFUSE