DELEGATED APPLICATION REPORT			                         Target Date: 03/04/20

Application No: 200176/FUL

Address: 110 Henley Road

Proposal: Two storey rear extension, altered access to flat and provision of terrace

Case Officer: Tom Hughes

Site Description and Surrounding Area

The application site comprises a two storey terraced building, at the end of a shopping parade. The building has been substantially extended to the rear. A bicycle shop occupies the ground floor, and part of the first floor, with a service area and store/office to the rear. A two bedroom flat is located at the first floor. The flat benefits from an exposed first floor roof terrace to the western flank of the building. A further enclosed terrace is located on the opposite side of the building. The flat is accessed by an external staircase, via a service yard at the rear. Access to the service yard is from Tilbury Close, an unadopted road off Chiltern Road. A public footpath runs the length of the western site boundary. The surrounding area comprises a mix of residential, commercial and industrial uses.

Proposal

The proposal is for a two storey rear extension, including the provision of a garage, an altered access to the flat and an additional roof terrace. To facilitate the proposal, the existing external access would be removed, and the service yard replaced by formalised vehicle parking. The proposed extension would infill the area aside the existing two storey extension, projecting to the same depth and extending out to the eastward site boundary. The ground floor of the proposal would comprise an integral garage, noted on the Proposed Floor Plan to be for the use of the first floor flat. Two single doors would provide access from the integral garage to the bicycle shop. The proposed two storey element would have a flat roof at a height of 7m, projecting to a depth of 3.5m. The proposed first floor extension would provide an enlarged entrance lobby and additional bathroom for the flat. To the rear of the two storey element would be located a roof terrace, with an external staircase beyond. The terrace would stand at a height of 4.2m, bounded by railings, and would comprise the new access to the flat. To the rear elevation would be located a garage door, and a window and the main entrance to the flat at the first floor. Materials have been selected to match those of the existing building.

Plans Considered

The agent supplied the following, received 04/02/20:

Drawing No: PL-01 – Site Location

Drawing No: PL-02 – Proposed Site Plan

Drawing No: PL-03 – Existing Floor and Roof Plans

Drawing No: PL-05 – Proposed Floor and Roof Plans

Drawing No: PL-05 – Existing and Proposed Elevations

Relevant Planning History

990158/FUL – Single storey rear extension – Permitted 22/05/00

990626/FUL – First floor rear extension – Permitted 30/10/00

990742/FUL – Alterations to shop front – Permitted 27/10/00

010704/FUL – Single storey rear extension – Permitted 22/10/01

030403/FUL – Change of use of flat to retail use, with various extensions – Permitted 19/03/03

030696/FUL – Alteration of extension and enclosure of external staircase to flat – Permitted 08/09/03

030209/FUL – First floor rear extension – Permitted 03/02/04

041066/FUL – Two storey rear extension – Withdrawn 10/02/05

051015/FUL – Three storey rear extension – Permitted 27/01/06

081404/FUL – Provision of additional flat and altered stairway access – Withdrawn 04/11/08

081645/FUL – Three storey rear extension – Refused 27/01/09

131171/FUL – First floor extension – Refused 07/10/13

Consultation Responses

Neighbouring owners and occupiers at 106, 108, 110 and 112 Henley Road; 26 Donkin Hill; 12 Tilbury Close; 7 and 12 Longhurst Close; and 6 and 7 Banbury Gardens were consulted by letter. A site notice was displayed. Two letters of representation were received with regard to the following:

- Overdevelopment of site

- Previous permissions have not been fully implemented, with resulting unsightly appearance

- Requirement for Tilbury Close to become an adopted road as a result of the residential development

- Impact on drainage

- Contractor parking and storage of equipment during construction

- Parking congestion on Tilbury Close

- Concerns regarding access and deliver to the bicycle shop

- Difficulty manoeuvring into the proposed garage

- Parking spaces not of required size

- Lack of space for bin storage

- Development would be overbearing and not aesthetically pleasing

- Proposed terrace would overlook neighbouring properties

Ecology

Given the structure of the building, it is considered unlikely that it hosts a bat roost. As such, there are no objections to the application on ecological grounds.

Transport Development Control

The plans suggest a garage is proposed, located at the rear and accessed via Tilbury Close off Chiltern Road. Tilbury Close is an unadopted road and not currently part of the public highway. Two further parking spaces are illustrated on the proposed plans. It is unclear whether the parking spaces are allocated to the flat or the retail unit. The existing parking provision has not been illustrated. Clarification regarding the allocation of vehicle parking spaces is required.

Environmental Protection

Concerns regarding the construction and demolition phases of development through noise and dust generated. Informatives recommended.

Natural Environment

There are a number of significant trees growing on the far side of the footpath, to the west of the development site. The canopies of these trees overhang the property. Although the generic RPA of the trees would hypothetically extend into the curtilage of 110 Henley Road, I am satisfied that the majority of roots will be prevented from growing into the property by the footings of the substantial boundary wall.

The proposed amendments and extensions are on the east side of the property, and well away from the off-site trees. In view of this, I do not foresee any harm to the trees as a result of the proposals, and no tree or landscape conditions are considered necessary.

Equality Act 2010

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Plan (2019)

Policy CC1 – Presumption in Favour of Sustainable Development

Policy CC2 – Sustainable Design and Construction

Policy CC5 – Waste Minimisation and Storage

Policy CC6 – Accessibility and the Intensity of Development

Policy CC7 – Design and the Public Realm

Policy CC8 – Safeguarding Amenity

Policy EN12 – Biodiversity and the Green Network

Policy EN14 – Trees, Hedges and Woodland

Policy H9 – House Extensions and Ancillary Accommodation

Policy H10 – Private and Communal Outdoor Space

Policy TR3 – Access, Traffic and Highway-Related Matters

Policy TR5 – Car and Cycle Parking and Electric Vehicle Charging

Supplementary Planning Guidance – A Design Guide to House Extensions (2003)

Revised Parking Standards and Design Supplementary Planning Document (2011)

Appraisal

Design and impact on the character of the surrounding area

Policy CC7 states that all development must be of a high design quality that maintains and enhances the character and appearance of the area. 110 Henley Road was originally a modest, two-storey building located at the end of a shopping parade, benefitting from a comfortable rear yard/garden. The building has undergone significant extension and alteration, in a piecemeal fashion. As a consequence of these extensions, the building now has a bulky, ungainly and visually jarring appearance when viewed from the rear. The proposal would infill the remaining undeveloped section of the site, currently used as a service yard for deliveries. The cumulative impact of the proposal and the existing extensions constitute a significant overdevelopment of the site. The proposal would exacerbate the existing jarring appearance of the building, introducing another flat roofed element which does not correspond with the roof heights of the other parts of the building. The proposal would bear no subservience to the original building, and would add to its confusing design with a mis-match of roof forms and external finishes. As such, the proposal is contrary to Policy CC7.

Impact on neighbouring amenity

Policy CC8 states that development will not cause a detrimental impact to the living environment of existing residential properties. Above each of the units in the shopping parade is residential accommodation, some benefitting from rear gardens. As noted above, 110 Henley Road has been substantially extended to the rear. The proposal would introduce further development directly along the eastward site boundary, adjacent to the rear garden of 112 Henley Road. The extension would exacerbate the existing overbearing effect and visual dominance caused to the neighbouring occupiers, to the detriment of their residential amenity. Further significant harm would be caused by the provision of the proposed roof terrace. From the proposed terrace, clear views would extend out over the rear of the neighbouring properties. This would result in direct overlooking, and a resultant loss of privacy, of the rear garden of 112 Henley Road. At such proximity, the detrimental impact on neighbouring amenity would be significant. As such, the proposal is contrary to Policy CC8.

Amenity of future occupiers

The existing internal layout of the flat provides a poor quality of accommodation for its occupiers, not particularly improved by the proposal. It would appear from the Proposed Floor Plans that the kitchen window is internal, looking directly into space only accessible from the bicycle shop. There would also be direct access from areas of the retail unit into the proposed garage. This is chiefly troubling from a crime and safety perspective, contrary to Policy CC8.

Transport

It is not clear from the supplied plans whether the proposed parking would be attributed to the flat or the bicycle shop. There are also concerns that the proposed garage would not be readily accessible, falling short of the standard set by the Parking Standards and Design SPD. Where the proposal infills an area previously in use as a yard and service area, this removes the potential for storage capacity for bins (both for the flat and the bicycle shop), and bicycle storage for occupants of the flat. It has not been demonstrated that this provision could be provided elsewhere on the site. The proposal is therefore contrary to Policies TR3 and TR5.

Conclusion

The proposal has been assessed and found to be unacceptable, contrary to the identified policy and guidance.

RECOMMENDATION

REFUSE