DELEGATED APPLICATION REPORT 			Target Date: 09/01/2019

Application No: 182004/HOU

Address: 25 Hogarth Avenue, Reading

Proposal: Single storey rear extension and garage conversion

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a two storey semi-detached property with a single storey side extension forming a garage.  The property is adjacent to a footpath leading to Poplar Avenue.  The adjoining property at 23 Hogarth Avenue has a two storey rear extension.  The surrounding area is predominantly residential.

Proposal:

The proposal is for the conversion of the existing single storey side garage and a single storey extension to the rear elevation.  The proposal has been amended slightly to overcome previous concerns.  The proposal includes a parapet wall along the side extension with a mono pitch roof.  The parapet wall measures 2.9m in height.  The proposed rear extension measures 4.6m in depth and has a hipped roof measuring 2.6m in height to eaves level and 3.7m in height at its highest point.  The proposed rear extension is 0.2m from the boundary with 23 Hogarth Avenue.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 14th November 2018:

Location Plan

Drawing No: A1

Drawing No: A2

Drawing No: A3

Drawing No: A4

Drawing No: A5

The agent submitted the following amended drawings, received on 6th January 2019:

Drawing No: A1

Drawing No: A2

Drawing No: A3

Drawing No: A4

Drawing No: A5

Relevant Planning History:

None.

Consultation Responses:

Transport – no comments at the time of writing however two parking spaces are shown on the Block Plan and two cars were parked on the frontage at the time of the Officers site visit.  This complies with the Council’s standards.

Properties at 23 Hogarth Avenue and ‘Brambles’ Poplar Avenue were consulted.  No letters have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 21st December 2018 that the principle of the extension was acceptable however there was a concern with the mono pitched roof design at the rear and the impact on the patio doors at the neighbouring property and the proposed elevations suggested the proposed extension encroached across the boundary line with Poplar Avenue.

Amended plans were submitted on 6th January 2019 which hipped the roof at the rear and a parapet wall introduced along the side boundary.  These amendments overcame the concerns raised.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed rear extension extends 4.6m in depth which exceeds the Council’s guidance of 4m this is the same depth as the two storey rear extension at 23 Hogarth Avenue and does not detract from the appearance of the existing property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; The neighbouring property by reason of their own two storey rear extension has reduced the amount of light to a pair of double doors between their extension and the boundary with 25 Hogarth Avenue.  The proposed single storey rear extension will by reason of its depth and proximity to the boundary further reduce the amount of light to these doors however, the eaves height is 2.6m and the roof design amended so that it hips away from the boundary with this neighbouring property.  As such, the proposal is not considered to significantly reduce the amount of light beyond the existing situation.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Approved plans

- Materials to match existing

Informatives relating to:

- Positive and proactive

- Terms and conditions

- Building regulations

- Encroachment and Party Wall

- Construction and demolition