Delegated Application Report 				          Target Date: 20/09/19

Address: 1 Links Drive, Tilehurst, Reading, RG30 4YT

Application No: 191000/CLP

Proposal: Loft conversion with hip to gable roof alteration and flat roof reardormer

Case Officer: Brian Conlon

Introduction and Proposal

The application site is a two-storey semi-detached dwelling with a pitched roof. The surrounding area is predominantly residential. The property is not a Listed Building and it doesn’t fall within a Conservation Area.

The proposal is for a loft conversion, a hip to gable extension, with the provision of a rear flat roof dormer. The dormer would extend across the rearward roof slope at a width of 5.6m, projecting to a depth of 3.4m with a height of 2.35m. The dormer would equal the height as the existing ridgeline, and would be set back from the existing eaves by a minimum of 0.2m. Two windows would be located to the rearward elevation. Materials have been selected to match those of the existing dwelling.

Relevant Planning History

190988/HOU - Two storey side extension - Refused

181932/HOU – Single storey rear extension –Permitted

181802/HPA – Single storey rear extension – Withdrawn 05/11/18

181789/HOU – Two storey side extension – Refused

011427/FUL – Two storey side and single storey rear extension – Permitted

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed loft conversion with the provision of a rear box dormer must be assessed to ensure that it complies with the conditions of Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relate to ‘additions etc to the roof of a dwellinghouse’.

The relevant conditions of the GPDO for a loft conversion with the provision of a rear box dormer which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); Complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; Complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; Complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, N/A or (ii) 50 cubic metres in any other case; Complies

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; Complies or

B.1 (f) the dwellinghouse is on article 2(3) land. Complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; Complies, as confirmed via email dated 13 Sept 2019.

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 24/06/19:

Drawing No. PD-01 - Location Plan

Drawing No. PD-03 – Proposed Block Plan

Drawing No. PD-06 – Proposed Plans

Drawing No. PL-07 – Proposed Plans

Drawing No. PL-08 – Proposed Elevations

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed loft conversion, hipped to gable end extension and the provision of a rear flat roof dormer. Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Bats