Delegated Application Report				Target Date: 09/01/2019

Address: 77 Whitely Wood Road, Reading, RG2 8HY
Application No: 182007
Proposal: Application for a Lawful Development Certificate for a proposed change of use of existing garage and store to a residential annex and creation of additional parking.
Case Officer: James Overall

Introduction and Proposal
The proposed scheme is to convert the existing garage to a “residential annex”, which contains all the elements of a self-contained dwelling (kitchen, lounge, bedroom, bathroom, cycle storage). A planning statement has not been provided to support the application.

The proposal also involves the changing of the roof from a flat roof to a dual pitched roof.

The application also proposes a new parking layout plan, although the existing parking layout has not been provided. Furthermore, parking layout is not something which can be considered under permitted development.

Plans Submitted

| Plan Type | Description | Drawing No. | Date Received |
| --- | --- | --- | --- |
| Location Plan |  | N/A | 14 Nov-18 |
| Block Plan |  | N/A | 14 Nov-18 |
| Plans & Elevations | Existing | WWR2 | 14 Nov-18 |
| Plans & Elevations | Proposed | WWR1 | 14 Nov-18 |
| Block Plan | Proposed Parking Layout | N/A | 14 Nov-18 |

Relevant Planning History
None.

Summary of Consultations
No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance
National Planning Practice Guidance
The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)
Permitted development rights for householders – Technical Guidance April 2017

Assessment
A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

E. The provision within the curtilage of the dwellinghouse of—

- any building or enclosure, swimming or other pool required for a purpose incidental to the enjoyment of the dwellinghouse as such, or the maintenance, improvement or other alteration of such a building or enclosure; or

It is evident from submitted drawing (No. WWR1) that the proposal is for potentially separate self-contained accommodation and without doubt primary living accommodation with a bedroom, bathroom, kitchen, lounge and cycle store all shown on this plan. As such, in practice (based on the plan submitted and assessment against the Technical Guidance) it is considered to be clear that the certificate of lawfulness cannot be granted for the use sought.

The ‘Permitted development rights for householders Technical Guidance’ document (which complements the GPDO) makes clear in the Class E sub-section (page 42) “A purpose incidental to a house would not, however, cover normal residential uses, such as separate self-contained accommodation nor the use of an outbuilding for primary living accommodation such as a bedroom, bathroom, or kitchen”.

- a container used for domestic heating purposes for the storage of oil or liquid petroleum gas. 

This criterion is not relevant as the proposal is not a container.

E.1 Development is not permitted by Class E if:

- permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use);

The dwellinghouse has always been in use as a dwellinghouse.

- the total area of ground covered by buildings, enclosures and containers within the curtilage (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse);

The total footprint (including the dwellinghouse) of the structures within the curtilage of the dwellinghouse, will not be increasing, and will not exceed 50% of the total area of the curtilage.

- any part of the building, enclosure, pool or container would be situated on land forward of a wall forming the principal elevation of the original dwellinghouse;

The building does not protrude in-front of the principle elevation of the original dwellinghouse.

- the building would have more than a single storey;

The building will only have a single storey.

- the height of the building, enclosure or container would exceed (i) 4 metres in the case of a building with a dual-pitched roof, (ii) 2.5 metres in the case of a building, enclosure or container within 2 metres of the boundary of the curtilage of the dwellinghouse, or (iii) 3 metres in any other case;

This building is within 2 metres of the boundary of the curtilage of the dwellinghouse and will have a dual-pitched roof with a maximum height of 2.9 metres. Therefore the proposal does not comply with criterion (e).

- the height of the eaves of the building would exceed 2.5 metres;

The eaves height of the building will be 2.1 metres.

- the building, enclosure, pool or container would be situated within the curtilage of a listed building; 

The building is not situated within the curtilage of a listed building.

- it would include the construction or provision of a verandah, balcony or raised platform;

The proposal does not include the construction or provision of a verandah, balcony or raised platform.

- it relates to a dwelling or a microwave antenna; or

The proposal does not relate to a microwave antenna, however could relate to a dwelling as the building is proposed to contain elements which would classify it as being capable of running as an independent property.

- the capacity of the container would exceed 3,500 litres.

This criterion is not relevant.

E.2 In the case of any land within the curtilage of the dwellinghouse which is within:

- an area of outstanding natural beauty;

- the Broads;

- a National Park; or

- a World Heritage Site,

development is not permitted by Class E if the total area of ground covered by buildings, enclosures, pools and containers situated more than 20 metres from any wall of the dwellinghouse would exceed 10 square metres.

The property is not located within any of these designations.

E.3 In the case of any land within the curtilage of the dwellinghouse which is article 2(3) land, development is not permitted by Class E if any part of the building, enclosure, pool or container would be situated on land between a wall forming a side elevation of the dwellinghouse and the boundary of the curtilage of the dwellinghouse.

The property is not located within land designated as article 2(3).

Recommendation
The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class E, Schedule 2, Part 1 of the Town and Country Planning (General Permitted Development) (England) Order 2015.

It can be confirmed that the proposed operation would not be lawful for planning purposes on the basis of the drawings received on 15th November 2018.

Insufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed; and therefore the application will be refused.