DELEGATED APPLICATION REPORT 			Target Date:	07/01/2019

Application Nos: 181985/HOU & 181986/LBC

Address: 121 Castle Hill, Reading

Proposal: Replacement single storey rear extension. Reconstruction of boundary

wall. Damp remedials.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site comprises a three storey mid terrace property with a basement garage.  The property is within the Russell Street/Castle Hill Conservation Area and is a Grade II Listed Building.  The surrounding area is predominantly residential.

Proposal:

The proposal involves the demolition of the existing single storey rear extension and the construction of a single storey replacement of a similar size.  Reconstruction of the retaining wall on the western boundary and damp investigations whilst the boundary wall is being investigated.  Any damp works proposed will be subject to future listed building consent.

The proposed materials will consist of painted render and fairfaced stock bricks with a copper roof and timber windows and doors and reclaimed bricks to the boundary wall.

Plans Considered:

The agent submitted the following drawings, received on 12th November 2018:

Drawing No: 17-1796/1000 – Existing Plans

Drawing No: 17-1796/1001 – Existing Elevations and Sections

Drawing No: 17-1796/4000 – Proposed Plans

Drawing No: 17-1796/4001 – Proposed Elevations and Sections

Drawing No: 17-1796/000 Rev B – Location and Site Plans Roof Plan

Design, Access, Planning and Heritage Statement

Relevant Planning History:

04/01176/LBC (Civica Ref: 041074) - Widen driveway through relocation of existing stone retaining wall. Install railings to garage roof to match existing on road boundary railings. No interior work.  Permitted 24/11/2004.

04/01177/FUL (Civica Ref: 041075) - Widen drive through relocation of existing stone retaining wall. Replace railings around garage roof.  Permitted 24/11/2004.

11/01260/FUL (Civica Ref: 110841) - Proposed pitched roof to replace existing flat roof.  Permitted 13/10/2011.

11/01339/LBC (Civica Ref: 111501) - Proposed pitched roof to replace existing flat roof.  Permitted 13/10/2011.

181181/HOU - Single storey rear extension, conversion of basement garage to habitable room with consequential external alterations.  Withdrawn 28/08/2018.

181182/LBC - Single storey rear extension. Conversion of basement garage to habitable room with consequential external alterations.  Withdrawn 28/08/2018.

181975/HOU - Conversion of basement garage to return it to living space (re-submission of 181181).  Pending consideration.

181976/LBC - Conversion of basement garage to return it to living space (re-submission of 181182).  Pending consideration.

Consultation Responses:

Historic Buildings Consultant advised the proposed works are relatively limited and confined to changes within the existing footprint of the building and would result in the re-building of a single storey extension which is of no special interest with an extension which would improve the appearance and legibility of the building, especially in terms of the rear fanlight.

The existing boundary wall is in poor condition and it also has a retaining wall aspect. The re-building of the boundary wall would help to improve the long-term conservation of the Listed Building and afford the opportunity to investigate damp problems within the building.

Whilst there are no objections in principle to the proposed works, there are limited details of the materials proposed and the re-building of the boundary wall as a ‘structural wall’ clad in brick. Therefore, conditions should be attached detailing the structural engineer’s proposals for re-building the wall, including foundations, and the following:

- sample of copper roof material;

- makers specification for the rooflight (conservation style)

- sample of the replacement and reclaimed bricks proposed

Reading Conservation Area Advisory Committee – no comments received.

Properties at 119 and 123 Castle Hill, 3 (all) and 7 (all) Coley Hill and Hadwyn House Field Road were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Part 12 – Achieving well-designed places

Part 15 – Conserving and enhancing the historic environment

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

CS33: Protection and Enhancement of the Historic Environment

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposed extension is small in scale but is an improvement on the existing addition.  The re-building of the boundary wall is acceptable subject to satisfactory details of its construction and materials which can be dealt with by way of a condition.  The proposals are therefore considered appropriate to the conservation of the Listed Building and would not harm the character of the Russell Street/Castle Hill Conservation Area.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT both applications

Subject to the following conditions:

181985/HOU

- Time limit

- Approved plans

- Details of materials and re-building to be submitted

Informatives relating to:

- Pre-commencement condition

- Positive and proactive

- Terms and conditions

4.	Building regulations

5.	Construction and demolition

6.	Encroachment and Party Wall

181986/LBC

- LBC Time Limit

- Approved Plans

- Details of materials and re-building to be submitted

Informatives relating to:

- Positive and proactive

- Pre-commencement condition