DELEGATED APPLICATION REPORT 			Target Date: 03/12/2018

Application No: 181759/FUL

Address: 930 Oxford Road, Reading

Proposal: Front dormer window and rear gable extension to create a 1 bedroom

apartment

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is occupied by ‘Glam Shotz’ hair and beauty on the ground floor which is at the end of a parade of shops along the Oxford Road.  The site is two storey in height when viewed from the road but three storey at the rear due to a drop in levels.  Part of the building is used as separate residential units following previous prior approval and planning permission.

Proposal:

The proposal involves a pitched roof dormer window to the front elevation and a rear gable roof extension to create a one bedroom apartment.

The materials would consist of brick to the rear gable to match the exisitng building and cladding to match other existing dormers in the row.

Plans Considered:

The agent  submitted the following plans and drawings, received on 8th October  2018:

Drawing No: OXRAI:200 – Site Location Plan

Drawing No: OXRAI:201 – Site Block Plan

Drawing No: OXRAI:202 – Existing Floor Plans and Elevations

Drawing No: OXRAI:203 – Proposed Floor Plans and Elevations

Planning Statement and Design and Access Statement

Relevant Planning History:

11/01357/CLE (Civica Ref: 110877) - Certificate of lawfulness for existing use of lower ground floor and timber building as B8 printers and stationers.  Refused 10/11/2011.

11/01402/FUL (Civica Ref: 111748) - Demolition of existing timber shed and replacement with new extended single storey building.  Withdrawn 31/08/2012.

12/00907/CLE (Civica Ref: 120730) - Certificate of Lawfulness for existing use of lower ground and existing timber building as B8 printers and stationers (Resubmission of 11/01357/CLE).  Withdrawn 31/08/2012.

12/01702/FUL (Civica Ref: 121659) - Change of use of basement to B8 storage and replace existing rear timber shed with new single storey rear extension.  Permitted 06/02/2013.

161917/UPA - Notification of Prior Approval for a Change of Use from Storage or Distribution Buildings (Class B8) and any land within the curtilage to Dwelling houses (Class C3). The proposed development comprises the change of use of the existing basement from storage (B8) to Residential (C3), to 1 x 1 bed dwelling.  Prior Notification under Class P, Part 3 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015.  Prior Notification Approval 05/12/2016.

170252/FUL - Conversion of rear storage facility from use class B8 to C3 (1 bedroom apartment).  Permitted 28/06/2017.

180578/APPCON - Discharge of conditions 1 and 2 of prior approval 161917. Discharged 07/06/2018.

181161/FUL - Glass and metal canopy over shop front.  Refused 28/08/2018.

Consultation Responses:

Transport raised no objection subject to an informative.

Ecology raised no objection.

Environmental Health raised no objection.

Properties at 928-932 (e) Oxford Road and 10-12 Deacon Way were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 30th November 2018 that the proposal was not considred acceptable.  Amended plans were not invited as reducing the rear gable extension in size was not considered to overcome the concerns raised and would also reduce the size of the proposed unit to an unacceptable level.  The agent did not withdraw the application and therefore the application will be refused.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as a starting point for decision making (NPPF paragraph 12).

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework (July 2018)

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS14 (Provision of Housing)

CS15 (Location, Accessibility, Density and Housing Mix)

CS16 (Affordable Housing)

CS24 (Car/Cycle Parking)

CS34 (Pollution and Water Resources)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (2012)

DM4 (Safeguarding Amenity)

DM6 (Affordable Housing)

DM10 (Private and Communal Outdoor Space)

DM12 (Access, Traffic and Highway Related Matters)

Emerging Local Plan - Submission Draft Reading Borough Local Plan (March 2018)

H4 (Standards for New Housing)

Supplementary planning documents/guidance

Parking Standards and Design SPD (Oct 2011)

Revised SPD on Planning Obligations under Section 106 of the Town and Country Planning Act 1990 (2013)

Affordable Housing SPD (2013)

Appraisal

The main issues are:

- Design and impact on the character of the area

- Impact on neighbouring properties

- Amenity of proposed flat

- Traffic and parking

- Affordable Housing

- CIL

a) Design and impact on the character of the area

The Council’s Supplementary Planning Guidance – A Design Guide to House Extensions states that dormer windows should be set within the roof slope and be in proportion with the scale of the rest of the house in terms of overall size and window shape.  Unfortunately Reading has lots of examples of dormers that are excessive in size which demonstrates how harmful this type of extension can be if not designed to take into consideration the impact on the character of the existing property.  Large flat roof dormers do not comply with this guidance or policy CS7 – Design and the Public Realm in the Core Strategy.  CS7 states that all development must be of high design quality that maintains and enhances the character and appearance of the area.  Although the Council’s Supplemetary Planning Guidance specifically refers to house extensions the principles of the guidance are considered relevant in this particular case as the proposal is to convert the current roof space by way of a gable extension to create a one bedroom flat.

There is a distinct rhythm to the row of properties and the proposed gable extension would be the same height as the existing roof and measure 5m in depth.  Due to its corner position the proposed gable extension would be visually dominant which is considered harmful to the character and appearance of the terrace, the street scene and the surrounding area.

Although the proposed front dormer would match the existing dormers along the terrace these existing dormers are equally spaced out with both properties at the end of the terrace having a hipped front projection (and no dormers).  The proposed front dormer would be squashed into the available space and would disrupt the rhythm of the terrace which again is considered harmful to the character and appearance of the terrace, the street scene and the surrounding area.

Taking the above into consideration the proposed rear gable extension and the proposed front dormer results in an inappropriate and unsympathetic development that detracts from the character and appearance of the existing terrace, the street scene and the surrounding area and is therefore contrary to Policy CS7 of the Core Strategy and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

b) Impact on neighbouring properties

Due to the existing rear projection of the section of terrace to which the proposed gable extension would be located the proposal is considered detrimental to the living environment of the residents of the neighbouring dormer window in terms of visual dominance and overbearing effects.  As such the proposal is contrary to Policy DM4: Safeguarding Amenity of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

c) Amenity of proposed flat

The proposed flat would have a front and rear facing window.  The front dormer window is small providing light to the kitchen and living area.  Flats would be expected to provide rooms of an adequate size in line with the Department for Communities and Local Government – Technical housing standards – nationally described space standards.   Policy H4 (Standards for New Housing) of the Draft Reading Borough Local Plan (March 2018) states that ‘new build housing should be built to the following standards: a. All new build housing outside the Central Area as defined on the Proposals Map will comply with the nationally-described space standard’.  Policy H4 also states that ‘ensuring sufficient levels of internal space is essential to the quality of life of residents of the Borough, which is a key element of the vision of the Borough’.

The proposed drawings state the proposed flat (measured at 1.5m minimum height) would be 38sqm.  This calculation just falls short of the nationally described space standards which states that a one bedroom flat for one person should have a floor area of 39sqm but for two people the minimum gross internal floor area should be 50sqm.  The proposal would therefore only comply with policy if there was only one occupant however as the proposed extension is considered excessive in size reducing the size of the roof extension would reduce the internal floor area to an unacceptable standard.

Policy DM10: Private and Communal Outdoor Space sets out that ‘Dwellings will be provided with functional private or communal open space…  Houses will be provided with private outdoor space whereas flats may be provided with communal outdoor space, balconies and/or roof gardens.’  There is no existing outdoor amenity space for the existing units within the building other than an allocated space for the unit in the ground floor extension at the rear.  There is therefore no opportunity to provide outdoor amenity space and in this instance this is acceptable.

d) Traffic and Parking

The site is located within the Zone 3, the Secondary Core Area but on the periphery of Zone 2, Primary Core Area which directly surrounds the Central Core Area and extends to walking distances of 2 kilometres from the centre of Reading.  The site is located adjacent to premier bus routes with frequent service to and from the town centre and other areas of West Reading.

The development site itself is adjacent to a layby that services the existing parade of shops, where there is currently unrestricted parking.

In accordance with the adopted Parking Standards and Design SPD, the proposal would generate the requirement for 1.5 parking spaces for a 1 bedroom flat.  The submitted Design and Access Statement, states that no car parking is to be provided.  Car ownership ward data identifies dwellings for similar sized dwellings generate a car ownership of 0.5 and 50% of occupiers do not own a vehicle.  In addition to this Whilst Oxford Road and the surrounding road network all have extensive parking restrictions preventing on-street parking; therefore, any overflow in parking would not affect the flow of traffic on the classified road network.

There are no Transport objections to this proposal subject to a highways informative.

e) Affordable Housing

The proposed development would need to accord with policy DM6 in terms of making a financial contribution to affordable housing. Policy DM6 (Affordable Housing) of the Local Development Framework (LDF) Sites and Detailed Policies Document (SDPD) seeks to ensure that on sites of 1-4 dwellings, a financial contribution will be made that will enable the provision of affordable housing elsewhere in the Borough.

However, there have been changes to national planning policy in terms of the 2014 Secretary of State’s Written Ministerial Statement (WMS). The changes sought to exempt developments of 10 or less dwellings from planning obligations for affordable housing and social infrastructure contributions and to introduce a new measure known as the Vacant Building Credit.

West Berkshire District Council and Reading Borough Council applied for a Judicial Review of the Secretary of State’s Written Ministerial Statement (WMS) to Parliament in 2014 on changes to national planning policy. The High Court handed down its judgment on the case on 31st July 2015. The High Court found in favour of the challenge by the local authorities and quashed the amendments to the NPPG. The Secretary of State appealed the judgment and the Court of Appeal then quashed the decision of the High Court on11th May 2016.

Reading Borough Council considered its options for implementing its affordable housing policy in the light of this decision. At its meeting of the Strategic Environment Planning and Transport Committee on 13th July 2016, the Council discussed the outcome of the Court of Appeal’s decision on its challenge (http://www.reading.gov.uk/media/5651/Item09-SEPT-C-Report-on-C-of-Appeal-judgement-05-16/pdf/Item09_SEPT_C_Report_on_C_of_Appeal_judgement_05_16.pdf)

It is notable that at paragraph 99(iii) of the judgement, the Judge reports that a statement made to the Court on behalf of the Secretary of State seeking to explain the effect of the new national policy accepted that:

“In the determination of planning applications the effect of the new national policy is that although it would normally be inappropriate to require any affordable housing or social infrastructure contributions on sites below the thresholds stated, local circumstances may justify lower (or no) thresholds as an exception to the national policy. It would then be a matter for the decision-maker to decide how much weight to give to lower thresholds justified by local circumstances as compared with the new national policy;”

Having considered its options, the Committee agreed the following as the basis for determining planning applications where Policy DM6 of the SDPD is relevant:

‘To implement Policy DM6 as currently adopted in the SDPD but excluding proposals that solely involve the conversion of an existing property, where the conversion involves the provision of 10 or less dwelling units (i.e. not HMOs), or the replacement of dwellings by the same number of replacement dwellings where there is no net increase’.

This planning application requires an extension in order to convert the roof space and therefore would need to accord with the requirements of Policy DM6 and make an appropriate financial affordable housing contribution.  The applicant has not submitted two independent valuations in order to calculate an affordable housing figure or confirmed that they would enter into a legal agreement to secure a contribution.  However, as the proposal is not acceptable in other planning terms, the Council has not pursued the S106 legal agreement to secure the financial affordable housing provision.

f) CIL

Had the proposal been acceptable it would have been liable to the Community Infrastructure Levy regime that is applicable to all decisions for planning applications made from the 01/04/2015. The details are available on the Council’s website:

http://www.reading.gov.uk/media/2791/Community-Infrastructure-Levy-Advice-to-Applicants/pdf/CIL_Advice_to_Applicants.pdf

Conclusion

By reason of its design, size and appearance the proposal is not acceptable without resulting in an inappropriate and unsympathethic development that detracts from the character and appearance of the existing terrace and surrounding area and which would be harmful to the living environment of the neighbouring property in terms of visual dominance and overbearing effects which is contrary to the identified planning policies and guidance.

RECOMMENDATION:

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive