### Delegated Application Report	Target Date:01/03/2019 (ext 5/3/19)

Address: 198 Southampton Street, Reading

Application No: 190020/CLP

Proposal: Certificate of Lawfulness for Proposed dormer extension and associated works.

Case Officer:  James Overall

## Introduction & Proposal

The application site is located on the west side of Southampton Street, slightly north of the junction with Alpine Street.

The certificate sought would confirm that the erection of a rear facing dormer, which will involve the removal of a chimney, would be lawful development in planning terms.

Plans Submitted:

| Plan Type | Description | Drawing Number | Date Submitted |
| --- | --- | --- | --- |
| Location Plan |  | 18007-0-001 P1 | 7 Jan-19 |
| Block Plan | Existing & Proposed | 18007-0-003 P1 | 7 Jan-19 |
| Section Plan | Existing | 18007-0-015 P1 | 7 Jan-19 |
| Section Plan | Proposed | 18007-0-016 P1 | 7 Jan-19 |
| Front Elevation | Existing & Proposed | 18007-0-014 P1 | 7 Jan-19 |
| Rear Elevation | Existing & Proposed | 18007-0-013 P1 | 7 Jan-19 |
| Floor Plan | 2nd Floor | 18007-0-012 P1 | 7 Jan-19 |

Relevant Planning History:

None.

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy & Guidance:

Section 10 of the Planning and Compensation Act 1991 established a procedure that enables anyone who wishes to do so, to apply to the local planning authority to determine whether a proposed use or operation, or an existing operational development or an existing use of land, or any other matter constituting a failure to comply with any condition or limitation subject to which planning permission has been granted, is lawful, and if so, be granted a certificate to that effect.

A Lawful Development Certificate is a legal document stating the lawfulness of past, present or future development. If granted by the local planning authority, the certificate means that enforcement action cannot be carried out to the development referred to in the certificate. However, the certificate will not protect from enforcement action by the planning authority if the specified use or development is then changed 'materially' without a planning application for it.

The certificate is not a planning permission. The planning merits of the use, operation or activity in the application are not relevant. The issue of a certificate depends entirely on factual evidence about the history and planning status of the building or other land and the interpretation of any relevant planning law or judicial authority. The responsibility is on the applicant to provide evidence to support the application.

The NPPG states that “In the case of applications for proposed development, an applicant needs to describe the proposal with sufficient clarity and precision to enable a local planning authority to understand exactly what is involved.  In determining an application for a prospective development under section 192 of the Town & Country Planning Act a local planning authority needs to ask “if this proposed change of use had occurred, or if this proposed operation had commenced, on the application date, would it have been lawful for planning purposes."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers that the works to be carried out falls within the permitted development rights of the property.

The development has been assessed to ensure that it complies with the conditions Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘additions etc. to the roof of a dwellinghouse’.

Development not permitted

B.1 Development is not permitted by Class B if:

- permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use);
COMPLIES

- any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof;
COMPLIES

- any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway;
COMPLIES

- the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than—

  - 40 cubic metres in the case of a terrace house, or
COMPLIES

  - 50 cubic metres in any other case;
NOT APPLICABLE

- it would consist of or include—

  - the construction or provision of a verandah, balcony or raised platform, or
COMPLIES

  - the installation, alteration or replacement of a chimney, flue or soil and vent pipe; or
SEE SECTION ON CLASS G

- the dwellinghouse is on article 2(3) land.
COMPLIES

Conditions

B.2 Development is permitted by Class B subject to the following conditions—

- the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse;

- the enlargement must be constructed so that—

  - other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension—

- the eaves of the original roof are maintained or reinstated; and
COMPLIES

- the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; and
COMPLIES

- other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; and
COMPLIES

- any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be—

  - obscure-glazed, and
COMPLIES AS NO SIDE FACING WINDOWS

  - non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed
COMPLIES AS NO SIDE FACING WINDOWS

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

Class G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘chimneys, flues etc on a dwellinghouse’.

G. The installation, alteration or replacement of a chimney, flue or soil and vent pipe on a dwellinghouse.

it was held in Shimizu (UK) Ltd v. Westminster City Council [1997] 1 WLR 168; [1997] 1 All ER 481 that demolition of only part of a building not amounting to demolition of the whole or substantially the whole of the building is to be regarded as an alteration of the building rather than as demolition. As such the removal of the chimney is considered to be an alteration and can be assessed under Glass G.

Development not permitted

G.1 Development is not permitted by Class G if—

- permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- the height of the chimney, flue or soil and vent pipe would exceed the highest part of the roof by 1 metre or more; or COMPLIES

- in the case of a dwellinghouse on article 2(3) land, the chimney, flue or soil and vent pipe would be installed on a wall or roof slope which—

  - fronts a highway, and

  - forms either the principal elevation or a side elevation of the dwellinghouse.

THE APPLICATION SITE IS NOT ON ARTICLE 2(3) LAND

Conclusion

The development has been assessed against Classes B and G of Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the dormer extension and associated works would be lawful, on the basis of the following submitted drawings:

| Location Plan |  | 18007-0-001 P1 | 7 Jan-19 |
| --- | --- | --- | --- |
| Block Plan | Existing & Proposed | 18007-0-003 P1 | 7 Jan-19 |
| Section Plan | Existing | 18007-0-015 P1 | 7 Jan-19 |
| Section Plan | Proposed | 18007-0-016 P1 | 7 Jan-19 |
| Front Elevation | Existing & Proposed | 18007-0-014 P1 | 7 Jan-19 |
| Rear Elevation | Existing & Proposed | 18007-0-013 P1 | 7 Jan-19 |
| Floor Plan | 2nd Floor | 18007-0-012 P1 | 7 Jan-19 |

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Informatives relating to:

  - Building regulations

  - Encroachment

  - Construction and Demolition