DELEGATED APPLICATION REPORT

Target Date: 06/12/2018				Extended Target Date: 08/02/2019

Application No: 181776/HOU

Address: 76 Kingsley Close, Reading

Proposal: Single storey side and rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a two storey end of terrace property located on a corner position.  The properties along Kingsley Close are unusual in design with brick and cladding with some properties (including the application site) having mono pitched roofs.  There is a distinct building line along Kingsley Close.  The surrounding area is predominantly residential.

Proposal:

The proposal has been amended from a two storey side extension to a single storey side and rear extension.

The proposed single storey side extension measures 2.6m in width and extends the full depth of the property.  The proposed extension wraps around the rear of the property and extends a further 1.5m in depth.  Due to the staggered nature of the application property the depth along the boundary with the adjoining property at 77 Kingsley Close would be 2.5m.  The proposal has a mono pitched roof that measures 2.5m in height to eaves level and 3.4m in height at its highest point.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 11th October 2018:

Drawing No: 001 – Existing Location & Block Plans

Drawing No: 002 – Existing Plans & Elevations

Drawing No: 003 – Proposed Location & Block Plans

Drawing No: 004 – Proposed Plans & Elevations

The agent submitted the following amended drawings, received on 11th January 2019:

Drawing No: 003 Rev A – Proposed Location & Block Plans

Drawing No: 004 Rev A – Proposed Plans & Elevations

Relevant Planning History:

None.

Consultation Responses:

Properties at 77, 90 and 91 Kingsley Close were consulted.  No letters have been received.

Amended plans letters were sent to all the above neighbours on 18th January 2019 with a 14 day consultation period of 1st February 2019.  No comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 10th December 2018 that the proposed two storey side extension was not acceptable as it created a visually dominant addition to the property that extended beyond the existing building line along Kingsley Close and directly up to the pavement.  Due to the constraints of the site it was felt that a single storey extension only would be acceptable.  Amended plans were submitted on 11th January 2019 which amended the proposal to a single storey side and rear extension.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposal would be close to the boundary with Kingsley Close and therefore extend past the building line, the single storey nature of the proposal means the extension would not detract from the appearance of the existing property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposal is only 1.5m from the rear of the existing property and even with the staggered nature the total depth along the boundary with 77 Kingsley Close would be 2.5m in depth which is not considered to be harmful on the residents of this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT