Delegated Application Report 				          Target Date: 14/05/18

Address: 43 Oak Tree Road

Application No: 180489/CLP

Proposal: Erection of outbuilding at rear

Case Officer: Tom Hughes

Introduction and Proposal

The application site is a two storey detached dwelling of relatively recent construction. Permitted development rights to alter or extend the roof were removed when planning permission was granted, though all other permitted development right remain. The surrounding area is predominantly residential. The adjacent dwelling at 41 Oak Tree Road was granted under the same permission as the application site and benefits from an outbuilding almost identical to that proposed.

The proposal is for the erection of an outbuilding at the end of the rear garden. The outbuilding would be of parallelogram shape, with a maximum width of 9.1m and a depth of 3.3m. The outbuilding would have a flat roof with a height of 2.5m. Double doors and windows would be located to the northward front elevation. The external faces of the outbuilding would be cedar clad. The use of the outbuilding would be as a summer house. The internal layout would include a shower room.

Plans Submitted

The agent supplied the following, received on 16/03/18:

Drawing No: 01 - Site Location and Block Plan

Drawing No: 02 – Proposed Floor Plan and Elevations

Photograph

Relevant Planning History

111431/FUL – Demolition of dwelling and erection of two 4-bed detached dwellings and one detached garage with new access from highway – Withdrawn 01/09/11

111721/FUL – Resubmission of 111431/FUL – Permitted 04/01/12

111838/NMC – Various changes to 111721/FUL – Agreed 27/01/12

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed outbuilding must be assessed to ensure that it complies with the conditions of Class E, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to the ‘buildings etc incidental to the enjoyment of a dwellinghouse’.

The relevant conditions of the GPDO for an outbuilding, which is not listed or in a conservation area, are as follows:

E.1 Development is not permitted by Class E if:

E.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

E.1 (b) the total area of ground covered by buildings, enclosures and containers within the curtilage (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); complies

E.1 (c) any part of the building, enclosure, pool or container would be situated on land forward of a wall forming the principal elevation of the original dwellinghouse; complies

E.1 (d) the building would have more than a single storey; complies

E.1 (e) the height of the building, enclosure or container would exceed (i) 4 metres in the case of a building with a dual-pitched roof, complies (ii) 2.5 metres in the case of a building, enclosure or container within 2 metres of the boundary of the curtilage of the dwellinghouse, complies or (iii) 3 metres in any other case; complies

E.1 (f) the height of the eaves of the building would exceed 2.5 metres; complies

E.1 (g) the building, enclosure, pool or container would be situated within the curtilage of a listed building; complies

E.1 (h) it would include the construction or provision of a verandah, balcony or raised platform; complies

E.1 (i) it relates to a dwelling or a microwave antenna; complies or

E.1 (j) the capacity of the container would exceed 3,500 litres. complies

E.2 In the case of any land within the curtilage of the dwellinghouse which is within:

(a) an area of outstanding natural beauty;

(b) the Broads;

(c) a National Park; or

(d) a World Heritage Site,

development is not permitted by Class E if the total area of ground covered by buildings, enclosures, pools and containers situated more than 20 metres from any wall of the dwellinghouse would exceed 10 square metres. complies

E.3 In the case of any land within the curtilage of the dwellinghouse which is article 2(3) land, development is not permitted by Class E if any part of the building, enclosure, pool or container would be situated on land between a wall forming a side elevation of the dwellinghouse and the boundary of the curtilage of the dwellinghouse. complies

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class E, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 16/03/18:

Drawing No: 01 - Site Location and Block Plan

Drawing No: 02 – Proposed Floor Plan and Elevations

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed outbuilding. Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Ancillary Use