DELEGATED APPLICATION REPORT 			Extended Target Date: 20/04/2018

Application No: 180138/HOU

Address: 47 Spencer Road, Reading

Ward: Redlands

Proposal: Proposed hip to gable extension with flat roof rear dormer, part two/part single storey side/rear extension and part single storey flat roof rear extension.

Case officer: Ralph Chakadya

Site Description and Surrounding Area:

The application site is a two storey, semi-detached property.  The surrounding area is predominantly residential.  The house is not listed or in a conservation area.

Proposal:

Permission is sought for hip to gable extension with flat roof rear dormer, part two/part single storey side/rear extension and part single storey flat roof rear extension.

Whereas the hip to gable extension with flat roof rear dormer proposal has been included as part of the proposals, it should be noted that a Certificate of Lawfulness was granted for the same under the permitted development regime. Details of the application are highlighted below under the history section.

Relevant Planning History:

171631CLE - Certificate of lawfulness for proposed loft conversion with side gable and flat roof rear dormer. Granted 15/11/2017 but not implemented.

Consultation Responses:

RBC Transport - Plans submitted indicate the proposed extensions do not change or impact on the existing parking arrangements.  Although not illustrated on submitted plans, it appears that there is sufficient space at the front of the property on an area of hard standing, served by an existing dropped kerb, which can accommodate two off road vehicles which would comply with our standards. Transport does not have any objections to this proposal, subject to the informatives on highways damage.

RBC Ecology - The application site comprises a semi-detached house where it is proposed to construct a two-storey rear extension and single-storey side and rear extensions. Considering the extent of the works and the low suitability of the surrounding habitat for use by commuting or foraging bats, it is considered unlikely that the proposals will adversely affect bats or other protected species. As such, there are no objections to this application on ecological grounds.

Public responses

One letter of objection received highlighting the following:

- Lack of parking(addressed in the report below)

- Subsidence and encroachment to neighbouring number 45 (Officer comments – Whilst number 47 is slightly higher ground than neighbouring number 45, there is no evidence of subsidence on the site and the extensions at single storey level would retain 0.3m from the boundary with number 45)

- Drainage cannot cope (Officer comments – the property is still a single family dwelling and there is no evidence to suggest that the existing drainage would be worsened by the extensions)

- Loss of light to bathroom and hallways to number 45 (addressed in the report below)

- Potential damage to number 45 during demolition of boundary wall

- Party wall issues (Officer comments – Party wall issues are not considered planning matters and are dealt with under separate legislation)

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008 and altered 2015)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012 and altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM10: Private and Communal Outdoor Space

Supplementary Planning Guidance “A Design Guide to House Extensions” (2003)

Supplementary Planning Guidance “Revised Parking Standards and Design” (2011)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; YES – Whilst the hip to gable conversion would unbalance this pair of semi-detached dwellings, a Certificate of Lawfulness for the works has already been granted and therefore the hip to gable conversion would still be implemented without the need for planning permission.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO.

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; - NO. Acceptable amended plans were submitted moving the proposed rear/side first floor at least 1m from the boundary with neighbouring number 45. In addition the extensions would not breach the 45-degree angle in relation to habitable room windows to adjoining numbers 45 and 49 despite concerns raised by the occupants of number 45 regarding loss of light to their bathroom and hallway, which are not habitable rooms as well.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; No

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; No.

- Parking – The Transport Officer has assessed the proposal and considered the proposal acceptable as the existing parking arrangements are not altered  and there is adequate space to the front of the property to accommodate two parking spaces

- DM4: Safeguarding Amenity states that an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties. The proposed development has been found to be acceptable when assessed against the relevant criteria.

- Third Party Issues

The issue of drainage blockage on the locality is not considered to be a planning matter in this instance. The applicant will need to liaise with the relevant utility service provider if there are drainage pipes to be affected. The site is not located within a flood zone. Issues to do with parking  have been addressed within the transport section above.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

RECOMMENDATION:

GRANT