DELEGATED APPLICATION REPORT 			Target Date: 10/12/2018

Application No: 181730/HOU

Address: 1 Sylvan Walk, Reading

Proposal: First floor side extension and ground floor rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a two storey semi-detached property at the furthest end of Circuit Lane.  The property has a single storey side addition and a rear conservatory.  The property is adjacent to a large area of green open land.  The surrounding area is predominantly residential.

Proposal:

The proposal is for a first floor side extension above the existing side addition and a single storey rear extension.  The proposed side extension would be set back from the existing front elevation of the property by 3.3m and set down from the main roof by 1m.  The proposal also extends at 2 storey by 2.5m in depth from the rear of the existing side addition with a hipped roof.

The proposed single storey rear extension would extend 2.5m in depth with a mono pitch roof measuring 2.2m in height to eaves level and 3.5m in height at its highest point.  The proposal would extend along the boundary with the neighbouring property at 95 Circuit Lane.

The materials will consist of brick and concrete tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 10th October 2018:

Location Plan

Existing Elevations and Floor Plans

Proposed Elevations and Floor Plans

The agent submitted the following plan, received on 15th October 2018:

Block Plan

Relevant Planning History:

None.

Consultation Responses:

Properties at 95 Circuit Lane and 3 Sylvan Walk were consulted.  No letters have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed first floor side extension is wider than half the width of the existing property due to its location at the end of Circuit Lane and its sufficient set back and set down, it is not considered to detract from the appearance of the original property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposal is a sufficient distance from 3 Sylvan Walk whose side elevation extends directly along the rear boundary of the application site (and there are no windows along this elevation) and the two storey element does not encroach on a line taken at 45 degrees which protects light reaching habitable rooms at 95 Circuit Lane.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Approved plans

- Materials to match existing

Informatives relating to:

- Positive and proactive

- Terms and conditions

- Building regulations

- Encroachment and Party Wall

- Construction and demolition