DELEGATED APPLICATION REPORT 			Target Date: 10/04/2018

Application No: 180281/HOU

Address: 8 Benson Close, Reading

Proposal: Two storey rear extension (re-submission of 170753)

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site consists of a two storey detached property raised up from the road.  Benson Close is a residential cul-de-sac consisting of two storey detached houses with single storey front projections of the same design.  There are levels across the cul-de-sac with 8 Benson Close at a higher level than the neighbouring properties at 7 and 9 Benson Close.  The property has a larger than average plot which includes a TPO Maple tree at the rear.  The surrounding area is predominantly residential.

Proposal:

The proposal involves a two storey rear extension following the withdrawal of planning application 170753/HOU for the same proposal.  The proposed extension measures 3.5m in depth and extends the full width of the property and is set down from the main roof by 0.5m.  The proposal has a gable end with a new side ground floor window and 2 additional first floor side windows.  Two of the first floor side windows (one existing and one proposed) face 7 Benson Close and are to a bathroom and en-suite and these will be conditioned to be obscure glazed and fixed up to 1.7m in height above internal floor level.  The third first floor side window is to a bedroom and faces into the garden of the application site and to the rear of properties on Northcourt Avenue.  There is a distance of approximately 10.5m from this proposed window to the rear boundary of properties on Northcourt Avenue and over 50m to the rear elevations of these properties.

There is a distance of approximately 3.5m between the side elevations of 7 and 8 Benson Close although 7 Benson Close is set further back.

The materials consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 14th February 2018:

Location Plan

Drawing No: A2 – Existing Elevations

Drawing No: A3 – Existing Floor Plans

Drawing No: A4 – Proposed Elevations

Drawing No: A5 - Proposed Floor Plans & Block Plan

Tree Survey by Tree Aware UK Ltd, date of survey 29th March 2017, updated 20th November 2017.

Drawing No: 40905-2-1 – Tree Constraints Plan

The agent submitted the following plan, received on 6th April 2018:

Drawing No: A5 - Proposed Floor Plans & Block Plan

Relevant Planning History:

170753/HOU - 2 storey rear extension.  Withdrawn 18/07/2017.  There were concerns that the proposal may have been within the Root Protection Area of the TPO Norway Maple in the back garden and with regards to the loss of the gap between the properties which was considered to contribute to a sense of space around the buildings.

Consultation Responses:

Transport raised no objection.

Ecology raised no objection.

Natural Environment Trees raised no objection but required a tree protection plan to be submitted.

Properties at 7 and 9 Benson Close, 1-5 (o) Ennerdale Road and 56-60 (e) Northcourt Avenue were consulted.  One letter of objection has been received with regards to:

- Loss of privacy to neighbouring properties.

- Previous subsidence to neighbouring property may be compromised.  Officer Note: this would be a matter for Building Control.

- If the property is intended to house more occupants then parking will become intolerable.  Officer Note: the property could be occupied by up to 6 occupants without the need for planning permission.  The property has 2 parking spaces which meets current parking standards.  If the property was to be used as a large HMO then this would require separate planning permission.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 27th March 2018 that a tree protection plan was required.  The agent submitted a plan on 6th April 2018.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

CS38 (Trees, Hedges and Woodlands)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO, following the submission of a Tree Survey and tree protection plan.

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the neighbouring property at 7 Benson Close has a first floor side window and a ground floor side door however there is approximately 3.5m between the extension and the side elevation of this property and the orientation of the properties means the proposal will not significantly reduce the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, there is sufficient distance between the proposed first floor side window facing the rear of properties on Northcourt Avenue and a condition will be imposed for the first floor side window facing 7 Benson Close to be obscure glazed and fixed up to 1.7m in height above internal floor level.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO, although there were previous concerns with regards to the loss of the gap between 7 and 8 Benson Close it is considered that there will remain a sufficient gap that will retain the spacious character of the area.

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposals they are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

- First floor side windows to 7 Benson Close obscure glazed and fixed

- Tree Protection Plan

Informatives relating to:

- Positive and proactive

- Highways

- Terms and conditions

4.	Building regulations

5.	Construction and demolition

6.	Encroachment