DELEGATED APPLICATION REPORT

Target Date: 10/05/2017				Extended Target Date: 17/05/2017

Application No: 170393/HOU

Address: 3 Delft Close, Tilehurst, Reading

Proposal: Single storey side and rear extension and conversion of garage

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey link-detached property with an attached side garage.  There are levels at the rear of the site and the garden slopes downwards.  There is a shared access to 4 properties on Delft Close.  The surrounding area is predominantly residential.

Proposal:

The proposal involves the conversion of the attached garage and a single storey extension to the side and rear.  The garage door will be replaced with a window and infilled with brick to match the existing property.  The proposed side extension extends from the rear of the garage and extends 4m in depth before wrapping around the rear of the property and extending 2.3m in depth.  The proposed rear extension does not extend the full width of the property.  The proposed side and rear extension has a mono-pitched roof and measures 2.5m in height to eaves level and 3.8m in height at its heights point.  Two rooflights, a rear window and double doors to the side are proposed.

The materials consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 13th March 2017:

Drawing No: 01 – Existing Plans

Drawing No: 02 - Existing Elevations

Drawing No: 03 – Proposed Plans

Drawing No: 04 – Proposed Elevations

Drawing No: 05 – Existing & Proposed Plans

Relevant Planning History:

None.

Consultation Responses:

Transport initially raised an objection as it seemed the parking arrangement would result in the requirement to drive over third party land and consideration must be given to other residents as the shared drive services 4 separate properties.  The applicant confirmed the proposed parking was on her land and as 2 parking spaces can be adequately provided on the site Transport raised removed their objection.

Environmental Health advised that the development lies on the site of a historic clay pit, which has the potential to have caused contaminated land and the proposed development is a sensitive land use.  Environmental Health raised no objection subject to a condition.

Properties at 2 and 4 Delft Close and 41 and 43 Wealden Way were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 6th April 2017 that notice had not been served on the neighbouring properties which shared the access drive.  The agent served notice on the neighbouring properties on 20th April 2017 and the expiry date of the application was extended to cover the statutory 21 day consultation period.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposals meet general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, although the neighbouring property at 4 Delft Close is set forward of the application property and the proposed side and rear extension will extend close to the boundary with this neighbouring property, the hipped roof design together with a low eaves height and the orientation of the properties mitigates against any significant loss of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties will notice the additional height and depth of the proposals they are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

- Reporting of Unexpected Contamination

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Positive and proactive

- Damage to the highway