Details for Proposed works to fit new railings and gate

The new railing will be fixed to the existing plinth.  The new railings will use the existing holes left by the original railings as their fixing points (12 cm apart). The old hole points are to be drilled out and the new railings resined in with 5 star resin grout.

Details for Proposed work to pathway and step

We wish to have the entire garden at the same level. We therefore propose moving the step up forward from its current position, inside the garden, to the gate. The area between the gate and the existing step up is to be filled with concrete to a height of 18cm (this work is already underway). This height matches the existing pathway so that there will be a smooth connection between them for the tiles which are to be laid on top. The area to be filled with concrete is about 1sqm.  The entire of the hard surface of the front garden, including this newly raised area, will then be covered with Victorian Tiles with a depth of 12mm.

The new step up will be finished off to the front with a new stepping stone. The stepping stone will be in complete alignment with the existing plinth in terms of width and height. The stone's specifications are as follows: Darleymoor Buff Sawn Yorkstone which is a sandstone, very similar in appearance to the existing plinth.

The above information is taken from emails sent by the applicant to the Case Officer on 25th April 2018 and the 26th and 27th June 2018.