DELEGATED APPLICATION REPORT 			Target Date: 11/09/2018

Application No: 181251/HOU

Address: 1 Pegs Green Close, Reading

Proposal: Single storey rear extension, garage conversion and raised roof to form a granny annex.  Re-submission of application 180516/HOU.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey detached property on the corner of Pegs Green Close and Tilehurst Road.  The front elevation of the property faces Pegs Green Close and there is a detached garage to the side.  Due to the orientation of the property there is no rear garden area and due to levels at the site the detached garage is at an elevated position.  The property has a single storey rear extension and a single storey side extension.  The surrounding area is predominantly residential.

Proposal:

The proposal is for the conversion of the detached garage and a first floor extension above to create a two storey granny annex.  The applicant has confirmed the annex would be for ancillary purposes to the main house.  The proposal also includes a single storey extension to the rear of the main house.

The existing garage has a pitched roof and measures 5.4m in width and 6.8m in depth.  The garage has a pitched roof and measures 2.4m in height to eaves level and 3.2m in height at its highest point.

The proposed extension to the garage would also have a pitched roof however the eaves height would be increased to 3.8m with a maximum height of 4.7m.  Two ground floor windows and a first floor window are proposed to the Pegs Green Close elevation, a new door to the side elevation is proposed and two existing ground floor side windows will be retained.

The proposed rear extension would extend along the rear elevation of the property adjacent to 354 Tilehurst Road.  The extension would be set back from the Tilehurst Road facing elevation by 0.15m and extend 6.6m in depth (the whole width of the rear elevation).  The proposed extension would be 0.3m from the boundary with 354 Tilehurst Road and would create a store room and utility.  The proposed ground floor plan shows a front door is proposed but this is not shown on the proposed elevations.  It should also be noted that the proposed rear extension as shown on the Block Plan extends to the rear of the existing single storey side extension but this is not shown on the floor plans and elevations.  The proposal would have a mono pitched roof and measures 3.3mm in height to eaves level and 3.8m in height at its highest point.

The materials would consist of red brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 17th July 2018:

Drawing No: 35417-01 – Existing Ground Floor Plan

Drawing No: 35417-02 – Existing Elevations

Drawing No: 35417-03 Rev A – Proposed Ground Floor Plan

Drawing No: 35417-04 Rev A – Proposed First Floor Plan

Drawing No: 35417-05 Rev A – Proposed Elevations

Drawing No: 35417-06 Rev A – Block And Location Plans

Relevant Planning History:

180516/HOU - Garage conversion with raised roof to form 2 storey granny annex with

link extension and single story rear extension to existing house.  Refused 17/05/2018.

Consultation Responses:

Transport advised the proposed side extension will result in the loss of the existing garage and thereby 1 parking space; submitted plans do not illustrate the proposed parking layout.  In accordance with the adopted Parking Standards and Design SPD, a 3 bedroom development in this zone would be required to provide off road parking provision for 2 parking spaces.  Revised plans are required illustrating the proposed parking provision.

Clarification is required if the proposed extension is an extension to the living arrangements of the main house or a unit to be commercially let.  If the latter applies then in accordance with the adopted Parking Standards and Design SPD, the newly created 1 bedroom annex would be required to provide parking for 1 car which should be independently accessible from those from the main dwelling and should be illustrated in amended plans, as well as the parking provision of 2 parking spaces for the existing property. The potential of a scenario for a commercial let dwelling can be controlled via a condition.  Transport do not have any objections to this application subject to conditions and informatives.

Properties at 2, 3 and 4 Pegs Green Close and 354 Tilehurst Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 31st August 2018 that the proposal did not overcome the concerns that were raised under the previous refusal and the application would be refused.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues are:

- Design and impact on the character of the surrounding area

- Impact on neighbours

- Other matters

a) Design and impact on the character of the surrounding area

Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (2012) states that an extension to a house should respect the character of the house in terms of scale, location, materials and design, and respect the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line.  Policy CS7 (Design and the Public Realm) of the Core Strategy also states that all development must be of high design quality that maintains and enhances the character and appearance of the area.

Due to the orientation of the property, 1 Pegs Greens Close has no rear garden.  Pegs Green Close rises up from Tilehurst Road and the detached garage is located on higher ground than the main property although the single storey nature means the garage is not visually prominent within the road.  The proposal creates a two storey building increasing the eaves height from 2.4m to 3.8m and the maximum height from 3.2m to 4.7m.  The proposal is considered overdevelopment of the site and visually dominant and does not reflect the scale and proportions of the original property and the properties within the Close and is not considered to be character with, or subservient to, the existing house.

The proposed rear extension measures 3.3m in height to eaves level and 3.8m in height at its highest point.  The property has an existing single storey rear extension that is set back from the Tilehurst Road elevation by 3.6m and measures 2.4m in height to eaves level and 3.4m in height at its highest point.  By reason of its excessive height, particularly to eaves level, the proposed rear extension is visually dominant that creates a large blank façade.  The cumulative impact of the two storey building and the rear extension is not considered to complement the character of the existing property and is uncharacteristic within the street scene.  The proposed development is therefore contrary to Core Strategy Policy CS7 (Design and the Public Realm), Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

b) Impact on neighbouring properties

The rear boundary of the application site is the side boundary to the neighbouring property at 354 Tilehurst Road and the main entrance into the property is from the side.  354 Tilehurst Road has a detached garage at the rear and the existing boundary wall is staggered in height.  There are a number of rear and side facing windows at this neighbouring property.  The proposal creates a 6.6m expanse of wall measuring 3.3m in height to eaves level and 3.8m in height at its highest point close to the boundary with 354 Tilehurst Road.  The excessive height of the proposed rear extension is considered visually dominant and overbearing on the residents of this neighbouring property.  As such the proposal is contrary to Policy DM4: Safeguarding Amenity of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

c) Other matters

The previously refused application proposed a link between the main property and the proposed annex however this element of the proposal has now been removed.  The proposed annex now has no link to the main property and it is felt that had the application been acceptable a S106 Legal Agreement would have been required to ensure that the annex remains ancillary to the main property and not let or sold as a separate residential unit.

Conclusion

Taking the above into consideration the proposed development fails to comply with the relevant local plan policies.  It represents overdevelopment and a lack of subservience and is considered to cause a detrimental impact to the living environment of the neighbouring property at 354 Tilehurst Road in terms of visual dominance and overbearing effects.

RECOMMENDATION:

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive