DELEGATED APPLICATION REPORT 				Target Date: 10/12/2018

Application No: 181795/HOU

Address: 35 Elsley Road, Tilehurst, Reading

Proposal: Two storey rear extension and part single storey rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site contains a detached two storey property with a two storey side/rear extension and a rear conservatory.  There is a difference in levels between the application property which is at a lower level than 35 Elsley Road but at a higher level than 37 Elsley Road.  The surrounding area is predominantly residential.

Proposal:

The proposal is for a two storey rear extension between the existing two storey side/rear extension and the conservatory.  The proposal would increase the pitch of the existing roof of the extension so that it will extend 0.4m higher than the existing roof (although only a small element of the roof will be visible from the road).  The proposal will involve alterations to the existing conservatory with alterations to the roof.

Some alterations to existing windows are also proposed which would be allowed under permitted development.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 15th October 2018:

Drawing No: H1268.01 – Location Plan

Drawing No: H1268.02 – Existing Floor Plans

Drawing No: H1268.03 – Existing Elevations

Drawing No: H1268.05 – Proposed Floor Plans

Drawing No: H1268.06 – Proposed Elevations

Drawing No: H1268.07 – Proposed Site Plan

Drawing No: H1268.08 – Block Plan

Relevant Planning History:

05/00241/HOU (Civica Ref: 050246) - Two storey side and rear extension.  Permitted 13/05/2005.

170504/HOU - Part single storey, part two storey rear extension & internal alterations.  Permitted 12/05/2017.

Consultation Responses:

Properties at 33 and 37 Elsley Road and 142 Overdown Road were consulted.  One letter has been received with regards to loss of privacy from the window to the proposed bedroom 2.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008 and altered 2015)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012 and altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the roof of the proposed extension will be visible from the road it is not excessive and will not extend any higher than the existing roof.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, there is sufficient distance between the proposal and the neighbouring properties.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, although a concern has been raised with regards to loss of privacy from the proposed bedroom 2 window there is an existing bedroom window in a similar position and bringing this window forward is not considered to create any additional overlooking or loss of privacy.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Approved plans

- Materials to match existing

Informatives relating to:

- Positive and proactive

- Terms and conditions

- Building regulations

- Encroachment and Party Wall

- Construction and demolition