DELEGATED APPLICATION REPORT			                         Target Date: 20/11/18

Application No: 181605/FUL

Address: 234c Shinfield Road

Proposal: Loft conversion with provision of rear box dormer and four roof lights

Case Officer: Tom Hughes

Site Description and Surrounding Area

The site comprises an end-of-terrace building with a pitched roof. The ground floor is occupied by a hot food takeaway, with residential accommodation above. The site is within a mixed area of commercial activity and residential uses. 236 Shinfield Road is located almost directly behind the application site, oriented towards the middle of the terraced row.

Proposal

The proposal is for a loft conversion with the provision of a rear box dormer and four roof lights, three of which would be located to the forward roof slope. The dormer would extend at a width of 3m, with a distance of 1.1m maintained to the boundary with the adjoining neighbour. The dormer would have a maximum height of 1.4m, set down from the ridgeline by 0.3m and set back from the existing eaves by 0.8m. An obscure-glazed and non-opening window would be located to the rear elevation of the dormer. The rearward roof light would also be non-opening. The loft conversion would facilitate additional residential accommodation, ancillary to the existing residential use.

Plans Considered

The agent supplied the following, received 12/09/18:

Site Location

Block Plan

Drawing No: JS/09/13/1 – Existing First Floor Plan and Elevations

Drawing No: JS/09/13/1 Rev. 1 – Proposed Floor Plans and Elevations

Relevant Planning History

181258/FUL – Loft conversion with rear box dormer and three front dormer windows – Withdrawn 03/09/18

Consultation Responses

Neighbouring owners and occupiers at 211, 221, 234 and 236 Shinfield Road were consulted by letter. A site notice was displayed. No letters of representation were received.

Environmental Protection raised no objection to the proposal.

Transport Development Control advised that the proposal would be unlikely to significantly increase the demand for parking in the area. There is no off-road parking currently attributed to the site. Due to this and the on-street parking restrictions in place along Shinfield Road, no objections were raised to the proposal.

Equality Act 2010

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Development Framework Core Strategy (2008)

Policy CS7 – Design and the Public Realm

Sites and Detailed Policies Document (2012)

Policy DM4 – Safeguarding Amenity

Supplementary Planning Guidance – A Design Guide to House Extensions (2003)

Appraisal

Design and impact on the character of the surrounding area

Policy CS7 of the Core Strategy requires that all development be of a high design quality that maintains and enhances the character and appearance of the area. The Council’s Design Guide to House Extensions identifies criteria against which proposals for dormer extensions should comply. Dormers should be set within the roof slope and be in scale and proportion to other window openings below.

The positioning of the dormer has been designed to line through vertically with the existing window opening below. The dormer is fairly modestly proportioned, set down from the original ridgeline and comfortably set back from the existing eaves. While it would be preferable for the window opening to be slightly smaller than the one directly below, it is not considered to be so harmful as to be unacceptable. The installation of roof lights to the forward and rearward roof slopes is not considered to detract from the original buildings appearance. As such, the proposal is in accordance with Policy CS7 of the Core Strategy.

Impact on residential amenity

A distance of approximately 15m separates the rear elevation of 234c Shinfield Road and the front elevation of 236 Shinfield Road. Accordingly, concerns were raised with regard to a loss of privacy through overlooking that could occur as a result of the rear dormer. To mitigate any overlooking, or the perception of overlooking, the window to the rear dormer would be obscure-glazed and non-opening as secured by condition. This, in combination with the angular orientation of the buildings is considered to mitigate any significantly detrimental impacts that might arise as a result of the proposed development.

To the rear of the building projects an external flue, from the takeaway use at the ground floor. The flue exhausts currently towards the rearward roof slope of the building. As such, concern was raised that the positioning of window openings to the rearward roof slope may cause issues in terms of fumes and smells. On the advice of officer’s, the window openings to the rearward roof slope would be non-opening as secured by condition. To ensure adequate ventilation to the rooms at the second floor, the forward roof lights would be openable. Accordingly, the proposal is in accordance with Policy DM4 of the Sites and Detailed Policies Document.

Conclusion

The proposed development has been assessed and found to be acceptable as it complies with the identified policies and guidelines.

RECOMMENDATION

GRANT