DELEGATED APPLICATION REPORT			          		    Target Date: 17/10/18

Application No: 181403/ADV

Address: Morrisons, Basingstoke Road

Proposal: Display of various non-illuminated signage to car park valet concession unit

Case Officer: Tom Hughes

Site Description and Surrounding Area

The application site relates to a section of car park, affiliated to Morrisons supermarket. The area subject of the application is located to the southern boundary of the site, east of the store building, and comprises nine parking spaces. A secondary access into the car park, via Basingstoke Road, is located adjacent to the site. The surrounding area comprises a mix of commercial, industrial and residential uses within the Basingstoke North Local Centre.

Proposal

The proposal is for the display of various non-illuminated signage to a car park valet concession unit. The valet unit is the subject of a separate current application (ref: 181402/FUL) which is still to be determined at the time of writing. The valet unit comprises wet and dry washing areas with a canopy, a cabin and perimeter fencing.

To the rear and side elevations, fascia signs would extend around the cabin. To each side elevation the signs would extend at a width of 2.9m at a height of 0.32m, projecting to a depth of 0.05m. To the rear elevation the sign would extend at a width of 5.2m. 2.283m would be maintained from the ground to the base of the signs. The signs would comprise composite panels with digitally printed applied vinyl in RAL 9016, Pantone 343c and RAL 5017. The signs would read ‘Car Park Valeting at Morrisons’. The signs would not be illuminated.

To the front and rear elevations of the cabin, adhesive-backed digitally printed vinyl wrap would be applied. The vinyl would be in RAL 5017 with text in RAL 1023 and RAL 9016 to read ‘SHOP WHILE WE WASH’ and ‘Car Park Valeting Hand Car Wash’.

Three hoarding panels would be located to the perimeter fence and end elevation of the cabin. Each sign would extend at a width of 1.2m at a height of 0.8m. 0.5m would be maintained from the ground to the base of the signs. The signs would comprise 5mm Foamex PVC composite panels with digitally printed graphics and posters. The signs would display the price list and services offered, company branding and health and safety information. The signs would not be illuminated.

Between the posts of the canopy would extend a banner sign. The sign would extend at a width of 7.7m at a height of 0.32m. 2.283m would be maintained from the ground to the base of the sign. The sign would comprise a double sided DTec mesh banner, with RAL 5017 and Pantone 343c text on a RAL 9016 background. The text would read ‘HAND CAR WASH at Morrisons’. The sign would not be illuminated.

Plans Considered

The agent supplied the following, received on 09/08/18:

Drawing No: CHQ.18.15380-PL01 – Site Location

Drawing No: CHQ.18.15380-PL02 – Existing Site Plan

Drawing No: CHQ.18.15380-PL03 – Proposed Site Plan

Drawing No: CHQ.18.15380-PL04 – Existing and Proposed Detailed Site Plans

Drawing No: CHQ.18.15380-PL05 – Proposed Valet Unit Details

Relevant Planning History

121237/ADV – Display of various signage – Permitted 21/05/12

180497/ADV – Display of various signage – Permitted 11/06/18

181402/FUL – Installation of car park valet unit with canopy – to be determined

Consultation Responses

Neighbouring owners and occupiers at ATS Euromaster, Basingstoke Road were consulted by letter. Three site notices were displayed. No letters of representation were received.

Equality Act 2010

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy, Sites and Detailed Policies Document and Reading Central Area Action Plan) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

Regulation 3 of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007 requires the Local Planning Authority to exercise its powers under these regulations in the interests of amenity and public safety taking into account the provisions of the development plan, so far as they are material; and any other relevant factors.  Regulation 3 states that factors relevant to amenity include the general characteristics of the locality, including the presence of any feature of historic, architectural, cultural, or similar interest.

Factors relevant to public safety include highway safety and whether the advert would hinder security or surveillance devices, including speed cameras.

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

Reading Borough Local Development Framework Core Strategy (2008) Policies:

CS7: Design and the Public Realm

Sites and Detailed Policies Document (2012)

DM4: Safeguarding Amenity

DM12: Access, Traffic and Highway-Related Matters

DM22: Advertisements

The main issues to be considered are:

- Amenity

- Public safety

Amenity

Policy DM22 of the Sites and Detailed Policies Document states that advertisements will respect the building or structure on which they are located and/or their surroundings and setting in terms of size, location, design, materials, colour,  noise, lettering, amount and type of text, illumination and luminance. The proposed signage would be located to a new car park valet unit, located within the existing store car park. It is considered that in this location and in the form proposed, no significant harmful design impacts, nor impact on the host building or character of the surrounding area, would result. The proposal is therefore in accordance with Policy DM22 of the Sites and Detailed Policies Document.

Public Safety

The proposed signage would be located in a position that would not cause a physical obstruction to members of the public using the highway. The signage would not be illuminated and as such is unlikely to cause a distraction to users of the highway. The proposal is therefore considered acceptable in public safety terms in accordance with Policy DM12 of the Sites and Detailed Policies Document.

RECOMMENDATION

CONSENT