### Delegated Application Report				Target Date: 20/07/2018

Address: 92 Kidmore Road

Application No: 180871/CLP

Proposal: Dormer roof extension on south (side) elevation and three rooflights on north (side) roofslope.

Case Officer: Heather Banks

## Introduction & Proposal

The application site is a two storey detached, pitched roof property, orientated so that the gable end of the property faces the road. The property has a partial width, two storey rear gable, a two storey bay window to the front of the property and a single storey, full depth garage to the northern side of the property. There is also driveway parking to the front of the property. Planning permission has recently granted to re-roof the garage and construct a single storey extension to the rear of the property, of the same depth as the two storey rear gable. The three properties to the north of the application property are of the same style as the application property, and two of these properties have similar loft extensions to that proposed here.

The applicant proposes to convert the loft space of the property to habitable accommodation and construct a dormer extension to the south facing, side roof slope of the property to facilitate this. It is proposed that the dormer will have a roof that pitched up to a narrow section of flat roof at its highest point. The dormer will be 8m wide at its base and 0.9m wide at its high point. The high point of the dormer is to be level with the ridge line of the main roof, and the bottom of the dormer is to be set in 0.2m from the eaves of the main roof. It is proposed to inset two side facing windows into the southern face of the dormer. It is also proposed to insert three sets of roof lights into the north facing roof slope of the main property. Each set of roof lights will consist of a lower, fixed shut window and a higher opening window.

Plans Submitted:

The agent supplied the following drawings:

Block/Site Plan

Drawing No: 0046-852 – Plans, Sections and Elevations

Received 25th May 2018

Email confirming details of materials and windows

Received 12th July 2018

Relevant Planning History:

180659/HOU - Single storey rear extension plus new roof to full length of existing double length garage.  Part conversion of garage to habitable space with new side access. Permitted 07/06/2018

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions Class B and C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘the enlargement of a dwelling house consisting of an addition or alteration to its roof’ and ‘other alterations to the roof of a dwelling house’.

Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES.

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; COMPLIES.

- B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; COMPLIES.

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than 50 cubic metres in the case of a detached house; COMPLIES, volume is approx. – 30.6 cubic metres.

- B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe COMPLIES

- B.1 (f) the dwellinghouse is on article 2(3) land.  COMPLIES.

Development is permitted by Class B subject to the following conditions:

- B.2 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES. The email received from the agent on 12th July states the following with regards to materials:

The existing roof is to remain as it is, apart from the installation of the Velux windows. The gable construction will be tiled on its pitches, and have vertical tile-hanging on its side elevations. This is to be done with tiles of a similar colour and texture to those existing on the roof at present.  The concealed flat roof would be weathered using Sarnafil single ply roofing membrane.

- B.2 (b) the enlargement must be constructed so that –

- other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; COMPLIES, and

- other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; COMPLIES.

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the windows which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. COMPLIES. The email received from the agent on the 12th July confirms that this will be the case for the two windows proposed in the dormer extension.

Development is not permitted by Class C if:

- C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- C.1 (b) the alteration would protrude more than 0.15m beyond the plane of the slope of the original roof; COMPLIES. This is confirmed in the email received from the agent on the 12th July

- C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; COMPLIES or

- C.1 (d) it would consist of or include (i) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. COMPLIES

Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of a dwellinghouse shall be:

- C.2 (a) obscure glazed; and (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. COMPLIES. The lower roof lights are to be fixed shut, and the higher roof lights are set higher than 1.7m above internal floor level and so will be openable. All the velux windows will be fitted with a film to make them obscure glazed. This is confirmed in the email received from the agent on the 12th July.

Conclusion

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Classes B and C of Part 1 of, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings and documents:

Block/Site Plan

Drawing No: 0046-852 – Plans, Sections and Elevations

Received 25th May 2018

Email confirming details of materials and windows

Received 12th July 2018

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Conditions

As conclusion.

Informatives

- The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

- During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

- The construction (and demolition) of the development will be subject to environmental health law.

- Work on a loft or roof may affect bats.  The applicant will need to consider protected species when planning work of this type.  A survey may be needed, and if bats are using the building, a licence may be required.  To view specific information regarding bats use the following link:

https://www.gov.uk/wildlife-licences

Site Map