DELEGATED APPLICATION REPORT

Ward: Peppard

App Ref: 181711/CLP

Address: 21 Evesham Road, Emmer Green

Proposals: Loft conversion with rear dormer and two rooflights in front roofslope

Case Officer: Connie Davis

Applicant: Mrs Majlinda Morina

Date received (valid from): 28 September 2018

Target Decision date: 23 November 2018

INTRODUCTION

The application property it a two storey, semi-detached dwelling with a ‘jerkin head’ roof form located on the southern side of Evesham Road. The surrounding area is residential, characterised by predominantly sets of semi-detached dwellings.

PROPOSAL AND SUPPORTING INFORMATION

The proposal is for a loft conversion with a rear dormer and front rooflights. The rear dormer would extend at a width of almost 4m and would project to a maximum depth of almost 2.7m. The dormer would have a maximum height of 2.5m and would be set back from the existing eaves by approximately 0.6m. The dormer would have a volume of 13.5 cubic metres. Materials have been selected to match those of the existing property. Two rooflights are proposed on the front roofslope, which based on the elevation plans project marginally from the roofslope.

Plans Submitted

The agent supplied the following drawings:

Drawing No. P1057-01 Location & Block Plan
Received 13th November 2018.

Drawing No. P1057-02 Existing Plans and Elevations
Received 28th September 2018.

Drawing No. P1057-03 Rev A Proposed Plans and Elevations
Received 20th November 2018.

Relevant Planning History

180555 - Single storey rear extension and two storey side extension - Application Permitted

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Equality Act

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. The key equalities protected characteristics include age, disability, gender, gender reassignment, marriage and civil partnership, pregnancy and maternity, race, religion or belief, sexual orientation.  It is considered that there is no indication or evidence (including from consultation on the current applications) that the protected groups have or will have different needs, experiences, issues and priorities in relation to this particular planning application.

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed loft conversion with rear dormer and front rooflights must be assessed to ensure that it complies with the conditions of Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘additions etc to the roof of a dwellinghouse’ and ‘other alterations to the roof of a dwellinghouse’ respectively.

The relevant conditions of the GPDO for a loft conversion with rear dormer which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, Not Applicable or (ii) 50 cubic metres in any other case complies, the cubic content of the resulting roof space would exceed the cubic content of the original roof space by 13.5 cubic metres.

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies or

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

C.1 Development is not permitted by Class C if:

C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

C.1 (b) the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies

C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

C.1 (d) it would consist of or include (i) the installation, alteration, or replacement of a chimney, flue or soil and vent pipe, complies or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment, complies

Conditions

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

C.2 (a) obscure-glazed; n/a complies and

C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, n/a complies

Recommendation

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed loft conversion with rear dormer and two rooflights on the front roofslope.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of:

Drawing No. P1057-01 Location & Block Plan
Received 13th November 2018.

Drawing No. P1057-02 Existing Plans and Elevations
Received 28th September 2018.

Drawing No. P1057-03 Rev A Proposed Plans and Elevations
Received 20th November 2018.

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives:

Building Regulations

Construction and demolition

Encroachment and Party Wall

Bats
Terms

Positive and proactive Working

1:1250 Location Plan (not to scale)