DELEGATED APPLICATION REPORT

Target Date: 05/12/2018				Extended Target Date: 19/12/2018

Application No: 181768/HOU

Address: 45 Ashmore Road, Reading

Proposal: Single storey side extension and rear two storey extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a two storey end of terrace property with a single storey side addition that is forward of the existing front elevation of the property.  The surrounding area is predominantly residential.

Proposal:

The proposal is for a single storey side extension and a part two storey part single storey rear extension.  The proposed single storey side extension has been amended as the height of 4.7m was considered excessive and it has been set back in line with the existing front elevation of the property.  The proposed side extension is the same depth as the existing property and extends up to the boundary.  The proposal has a mono pitched roof with a parapet wall and measures 3.4m in height to the top of the parapet wall and 4.1m in height at its maximum height.  There is a detached garage adjacent to the proposal which belongs to 47 Ashmore Road.

The proposed two storey rear extension is 2.4m from the boundary with the neighbouring property at 43 Ashmore Road and measures 3m in depth.  The proposal is 2.3m from the boundary with 24 Stockton Road.  The proposal has been amended from a gable end roof to a hipped roof and set down from the main roof slightly.  The proposed two storey extension does not encroach on a line taken at 45 degrees from the middle of the closest first floor window at 43 Ashmore Road.

The proposed single storey rear extension measures 3m in depth and infills the side of the two storey rear extension up to the boundary with 43 Ashmore Road.  The proposal has been amended from a flat roof (measuring 3.4m) to a mono pitched roof with a flat top measuring 2.7m in height to eaves level and 3.8m in height at its highest point.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawing, received on 10th October 2018:

Drawing No: 45ASH/002 Rev A – Existing And Proposed Plans And Elevations Location And Block Plans

The agent submitted the following amended drawing, received on 5th December 2018:

Drawing No: 45ASH/002 Rev B – Existing And Proposed Plans And Elevations Location And Block Plans

The agent submitted the following amended drawing, received on 17th December 2018:

Drawing No: 45ASH/002 Rev C – Existing And Proposed Plans And Elevations Location And Block Plans

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to an informative.

Properties at 43 and 47 Ashmore Road and 20-24 (e) Stockton Road were consulted.  No letters have been received.

Summary of discussions with applicant/agent:

The agent was advised that the principle of the proposal was acceptable however the heights of the proposed extensions were considered excessive.  An amended plan was submitted on 17th December 2018 addressing these concerns.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposed rear extensions only extend 3m and the proposed two storey does not encroach on a line taken at 45 degrees which protects light reaching habitable rooms to 43 Ashmore Road and the proposed side extension is a sufficient distance from 47 Ashmore Road.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposals they are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Approved plans

- Materials to match existing

Informatives relating to:

- Positive and proactive

- Highways

- Terms and conditions

- Building regulations

- Encroachment and Party Wall

- Construction and demolition