### Delegated Application Report			Target Date: 14/9/18 (extended)

Address: Land at Island Road West

Application No: 180613/NMC

Proposal: Non material amendment comprising alterations to the external appearance of Unit 1 and 2 and the incorporation of minor changes to the approved levels, drainage and landscape drawings

Case Officer: Alison Amoah

Proposal: Non material changes relating to –

The proposed changes relate to the external appearance of units 1 and 2 as approved under 170444 and minor changes to landscape, drainage and levels drawings, summarised as follows:

External Changes to Unit 1 (PL203 & PL204)

- Removal of a door on the southern elevation;

- Louvres to the west elevation for the plant room; and

- Clarification on materials.

External Changes to Unit 2 (PL206 & PL207)

- Increase height by 1 m;

- Level access door to southern elevation

- Inclusion of retaining structures and level details.

Drainage Plan

Addition of rainwater harvesting tanks and revisions to sizing and location of groundwater storage (as approved under discharge of condition – submitted to regularise the position).

Levels Plan

- Revised location of steps around Unit 1

- Minor amendments to levels around access road, service yard and car parking for Unit 1 to co-ordinate with drainage strategy.

Landscape Plan

Revised planting arrangement and mix to accommodate the power company’s requirements for cabling and acess to the proposed sub-station (as approved under discharge of condition – submitted to regularise the position).

The amended plans are documented in the table below:

| Original Approved Plan | Revised Plan |
| --- | --- |
| Site Layout Plan – Drawing no: 30912 PL 101 Rev D | Site Layout Plan – Drawing no: 30912 PL 201 |
| Unit 1 Floor Plans – Drawing no: 30912 PL 103 Rev A | Unit 1 Floor Plans – Drawing no: 30912 PL 203 |
| Unit 1 Elevations – Drawing no: 30912 PL 104 Rev A | Unit 1 Elevations – Drawing no: 30912 PL 204 |
| Unit 2 Floor Plans – Drawing no: 30912 PL 106 Rev B | Unit 2 Floor Plans – Drawing no: 30912 PL 206 Rev A |
| Unit 2 Elevations – Drawing no: 30912 PL 107 Rev B | Unit 2 Elevations – Drawing no: 30912 PL 207 Rev A |
| Landscape Concept Proposal 01F (it should be noted that 01L was approved under the subsequent discharge of condition approval 172105). | Landscape Concept Proposal 01N |
| Proposed Drainage Plan – Drawing no: BED-DRG-160016-11(00)0300 A1 P04 | Proposed Drainage Plan – BED-DRG-160016-00-XX-C-0300 Rev C04 |
| Proposed Levels Plan – Drawing no: BED-DRG-160016-11(00)600 Rev P06 | Proposed Levels Plan – Drawing no: BED-DRG-160016-00-XX-C-0600 Rev P09 |
| Unit 1 Illustrative Elevations – Drawing no: 30912 PL 105 Rev A | No change |
| Unit 2 Illustrative Elevations – Drawing no: 30912 PL 108 Rev B | No change |
| External Works – Drawing no: – 30912 PL 109 Rev C | No change |
| Illustrative Landscape Concept Sections - Drawing no: 02 D | No change |

The applicant has set out that the proposed changes have been derived from the prospective tenant requirements of level access and greater internal height within Unit 2, and that detailed design development has continued and this has led to minor changes to Unit 1, levels, drainage and landscape.

Objections Received (Summary):

Applications under S96A are not applications for planning permission, so are not covered by the publicity requirements as set out in the DMPO.  No external consultation was undertaken.  However the Natural Environment and Transport teams were consulted.

Consultee Responses:

Natural Environment: I’ve read the application letter for an explanation of the changes and compared the submitted Landscape Concept Proposals drawing 01 N with 01 L approved under 172105/APPCON.  As is indicated in the supporting letter, the changes (from 170444) were largely agreed under 172105.  I note since then that two species have been removed from the indigenous hedge; those being Ulex and Ligustrum, which is fine.  The hedge is still comprised of a mix of 5 natives.  I have no objection to the proposed changes and change in approved plans (compared to 170444 and 172105).

SUDS: I have reviewed the latest plan and it is noted that the foul rising main has been included within the area secured for MRT at the frontage of the site which was not identified by officers when the proposal was submitted at the discharge of condition stage as the condition on related to the surface water drainage and the management and maintenance procedure.  It is noted that the original scheme illustrated the foul rising main to be located within the carriageway and so has been relocated even though previous proposals included a land drain and catch pits within this land which were removed following RBC request, please see the attached correspondence.

As a result I am unable to agree to the submitted drawings as they could prejudice the Councils ability in bringing forward future transport infrastructure and therefore further information should be provided to ensure that the MRT scheme can be implemented.

Planning Officer note: The agent sent an email 23/5/18 to state that “The drainage details submitted under this NMA are the same as those details that were submitted and approved as part of the discharge of conditions process, which are attached for ease of reference.  The NMA is therefore to regularise the position in respect of the approved drainage scheme so that the decision notice refers to the most recent layout.”

SUDS response: The discharge of condition application solely relates to the Sustainable Drainage design and maintenance and as such this is the only element that was assessed.  The covering letter accompanying the discharge of condition application also makes no reference to the changes of the foul rising main which was originally illustrated as being located within the carriageway so that this could be looked at in detail.

This has now only been able to be fully assessed as part of this NMA application, which should have been submitted initially given the overall changes to the drainage scheme.

For me to be happy with the scheme as currently presented you will need to demonstrate that this will not prejudice the Highways ability in implementing the MRT scheme on the land in question.

Planning Officer note: Following submission of an amended plan to identify that the foul main would be 1.6 metres under ground level.  SUDS officer confirmed that this was acceptable.

Relevant Planning History:

170444 – Erection of two no. employment buildings (B1c, B2 and B8 Use Classes) totalling 11,067sqm (GIA) including ancillary offices and structures, servicing areas and access roads, car parking and landscaping – Approved 24/10/17

172105 - Application for discharge of planning conditions 3, 5, 6, 9, 14, 15, 18, 22, 23, 28, 30 and 31 of planning permission 170444 – Discharged 12/3/18

Appraisal:

- The maximum height of Unit 2 would increase by 1m and this change is not considered to be harmful to the character and appearance of the proposal nor would it lead to a significant increase in the bulk or massing across the site, thereby not visually impacting the amenity of the area, creating overlooking or overshadowing of nearby properties, beyond what was considered at original application stage.

- The remaining minor amendments were largely approved through discharge of condition permissions and have been submitted to regularise the position and are considered acceptable within the context of the original permission.

- The changes are therefore considered to be acceptable as an NMC.

As the original permission was with regard to a major application, confirmation was sought from and approved by Councillors that this could be a delegated decision, rather than being considered by Planning Applications Committee.

Conclusion:

The changes as proposed are considered to be acceptable non-material alterations to the approved scheme.

Recommendation:

Approve non-material amendment subject to conditions. Although Section 96(A) of the Act does not allow for the original decision to be re-issued, it does allow the modification of, or imposition of, conditions. In this respect it is considered appropriate to impose an additional condition:

Condition

The following amended conditions approved by this Non-Material Change (under Section 96A of the Town and Country Planning Act 1990) are as follows:

Amended Condition 2 (amendments in bold)

2. The development hereby permitted shall be carried out in accordance with the following approved plans:

• Site Layout Plan – 30912 PL 201

• Unit 1 Floor Plans – 30912 PL 203

• Unit 1 Elevations – 30912 PL 204 Rev A

• Unit 1 Illustrative Elevations – 30912 PL 105 Rev A

• Unit 2 Floor Plans – 30912 PL 206

• Unit 2 Elevations – 30912 PL 207 Rev A

• Unit 2 Illustrative Elevations – 30912 PL 108 Rev B

• External Works – 30912 PL 109 Rev C

• Landscape Concept Proposal – 01 N

• Illustrative Landscape Concept Sections – 02 D

• Proposed Drainage Plan – BED-DRG-160016-00-XX-C-0300 Rev C04

Reason: because permission is granted in respect of the development as detailed in the approved plans only.

Amended condition 8 (amendments in bold)

8. Landscaping

All hard and soft landscape works shall be carried out in accordance with the approved details:

• SOFT LANDSCAPE MAINTENANCE AND MANAGEMENT SPECIFICATION – BARRY CHIN ASSOCIATES ref: 1736/17-RP01 rev F dated: 28 July 2017

• External Works – 30912 PL 109 Rev C

• Landscape Concept Proposal – 01 N

Reason: To ensure the provision of amenity, and biodiversity enhancements, afforded by appropriate landscape design and to comply with Policy CS36 and the NPPF

Amended condition 27 (amendments in bold)

Flooding mitigation

The development permitted by this planning permission shall be carried in accordance with drawing number BED-DRG-160016-00-XX-C-0600 Rev P09 prepared by Built Environment Design Partnership and the following mitigation measures:

• the development will be located on land above the 1% AEP plus 25% allowance for climate change flood level of 39.57mAOD

• Finished floor levels of Units 1 and 2 will be set respectively no lower than 43.20m AOD and 42.37m AOD.

The mitigation measure(s) shall be fully implemented prior to occupation and subsequently in accordance with the timing / phasing arrangements embodied within the scheme, or within any other period as may subsequently be agreed, in writing, by the Local Planning Authority.

Reason: To reduce the risk of flooding to the proposed development and future occupants and to ensure the proposed development does not increase flood risk onsite or elsewhere. This condition is sought in accordance with paragraph 103 of the National Planning Policy Framework (NPPF). In accordance with NPPF Paragraph 103, Core Strategy Policy CS1 and Sites and Detailed Policies Document Policy DM1

INFORMATIVES

1. This notice relates only to the changes to conditions 2, 8, and 27 of planning permission 170444.  It shall only be read in the context of that permission, and is bound by all the conditions and obligations attached to those permissions.  Those permissions, which still stand, and this notice (and any other notices issued under section 96a for this development) should be read together.

2. In reaching this decision, the local planning authority is satisfied that the changes proposed by this application do not constitute material changes to the approved planning permission.