Delegated Application Report

Target Date: 05/06/2018				Extended Target Date: 14/09/2018

Address: 34 Chapel Hill, Tilehurst, Reading

Application No: 180580/FUL

Proposal: Demolition of existing bungalow and erection of a two storey detached house

Case Officer: Claire Ringwood

Site Description and Surrounding Area

The application site is a detached bungalow on the corner of Chapel Hill and Lower Elmstone Drive.  The surrounding area is predominantly two storey residential properties of varying styles and designs.

Proposal:

The proposal is for the demolition of the existing bungalow and the construction of a two storey, three bedroom detached property of modern contemporary design.  The neighbouring property at 32A Chapel Hill is a two storey detached property that is at a higher level than the application site.  101 Lower Elmstone Drive is a two storey semi-detached property.  The proposed dwelling would have a similar footprint to the existing bungalow and retain similar spacing between the neighbouring properties as the existing bungalow.  A low level front boundary wall will be retained.

The proposal will largely retain the existing garden and the existing access from Lower Elmstone Drive is to remain as existing.

The proposed materials will consist of a blockwork plinth on the ground floor and blue vertical metal cladding sheets to the first floor with cedar window surrounds in varying shapes.

Plans Submitted:

The agent submitted the following information, received on 4th April 2018:

Design and Access Statement

The agent submitted the following drawings, received on 10th April 2018:

Drawing No: P(00)000 – Rev B – Existing Site Location

Drawing No: P(00)001 Rev B – Proposed Block Plan

Drawing No: 17112_D(00)_010 Rev B – Existing Ground Floor Plan

Drawing No: 17112_D(00)_020 Rev B – Existing Section AA

Drawing No: 17112_D(00)_021 Rev B – Existing Section BB

Drawing No: 17112_D(00)_030 Rev B – Existing Rear (South) Elevation A

Drawing No: 17112_D(00)_031 Rev B – Existing Front (North) Elevation B

Drawing No: 17112_D(00)_032 Rev B – Existing Side (West) Elevation C

Drawing No: 17112_D(00)_033 Rev B – Existing Side (East) Elevation D

Drawing No: 17112_D(00)_100 Rev B – Proposed Ground Floor Plan

Drawing No: 17112_D(00)_101 Rev B – Proposed First Floor Plan

Drawing No: 17112_D_(00)_103 Rev B – Proposed Roof Plan

Drawing No: 17112_D(00)_200 Rev B – Proposed Section AA

Drawing No: 17112_D(00)_201 Rev B – Proposed Section BB

Drawing No: 17112_D(00)_300 Rev B – Proposed Rear (South) Elevation A

Drawing No: 17112_D(00)_301 Rev B – Proposed Front (North) Elevation B

Drawing No: 17112_D(00)_302 Rev B – Proposed Side (West) Elevation C

Drawing No: 17112_D(00)_303 Rev B – Proposed Side (East) Elevation D

Drawing No: 17112_D(00)_400 Rev B – Proposed 1:500 South and West Street Elevation

Drawing No: 17112_D(00)_401 Rev B – Proposed 1:100 South and West Street Elevation

Relevant Planning History:

180039/PREAPP - Demolition of existing bungalow and for the development of a new two-storey house. Proposed floor area 94m2.  Observations sent 19/02/2018.

Consultation Responses

Transport – raised no objection subject to conditions and informatives.

Natural Environment Trees – raised no objection subject to conditions.

Ecology – raised no objection.

Environmental Health Protection – Standing advice states that a statement detailing how noise and dust will be effectively managed during demolition (if relevant) and construction and who will be responsible for this.  Conditions will include 1) the control of noise and dust, 2) construction and demolition hours of work, 3) no bonfires.

Reading Design Review Panel – the proposal was presented to the Reading Design Review Panel on the 7th June 2018.  The panel stated that ‘whilst in principle the panel accepts that a replacement dwelling is policy compliant, the bold design will stand or fall on the precision and accuracy of the detailing….Whilst the panel are supportive of an ambitious design, this needs to be matched by the precision and accuracy of the material selection and detailing.  We are yet to be convinced the ambition is matched by the quality of detail.’

Public Consultation

Properties at 32A Chapel Hill and 101 Lower Elmstone Drive were consulted.  No comments have been received.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance and Legal Context

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

National Planning Policy

National Planning Policy Framework (2018)

Reading Borough Local Development Framework Core Strategy (2008)

CS1: Sustainable Construction and Design

CS2: Waste Minimisation

CS5: Inclusive Access

CS7: Design and the Public Realm

CS14: Provision of Housing

CS15: Location, Accessibility, Density and Housing Mix

CS16: Affordable Housing

CS24: Car/Cycle Parking

CS34: Pollution and Water Resources

CS38: Trees, Hedges and Woodlands

Sites and Detailed Policies Document

SD1: Presumption in Favour of Sustainable Development

DM1: Adaptation to Climate Change

DM4: Safeguarding Amenity

DM5: Housing Mix

DM6: Affordable Housing

DM10: Private and Communal Outdoor Space

DM11: Development of Private Residential Gardens

DM12: Access, Traffic and Highway-Relates Matters

DM18: Tree Planting

Supplementary Planning Documents

Revised Parking Standards and Design (2011)

Revised Sustainable Design and Construction (2011)

Revised S106 Planning Obligations (2013)

Affordable Housing (2013)

APPRAISAL

The main issues to be considered are:

- Principle of development

- Design/appearance of development

- Residential amenity of new and existing residents

- Amenity space

- Transport implications

- Trees and Landscaping

- Other considerations

- Principle of development

The site is located within a largely residential area.  The principle of residential use is already established in terms of the existing bungalow and its redevelopment is in line with Core Strategy Policy CS14 (Provision of Housing) and paragraph 118 (c) of the NPPF which encourages the effective use of land by reusing land that has been previously developed (brownfield land). The principle of a residential use is therefore acceptable, subject to meeting other policy considerations set out below.

- Design/appearance of development

Core Strategy Policy CS7 states that “all development must be of high design quality that maintains and enhances the character and appearance of the area”.  The development proposals need to be assessed in terms of the relationship of the proposal to the existing built form and spaces around buildings within the surrounding area.

The proposed dwelling would have a similar footprint to the existing bungalow, would retain similar distances between neighbouring properties and would not extend forward of any existing building lines and in this regard the replacement of the existing bungalow is considered acceptable.

However the application site is located on a prominent corner position and the contemporary modern design and materials, which will include blue metal cladding, will be significantly different to neighbouring properties.  The proposed dwelling will be subservient to neighbouring properties and has been designed to be lower in height however its unusual form and material selection does mean the proposed dwelling will ‘stand out’ quite significantly.

Officers agree with the Reading Design Review Panel in that the design will stand or fall on the precision and accuracy of the detailing.  It would have been preferable for more detailed drawings to be submitted prior to a decision being made however the agent felt these matters could be adequately addressed through planning conditions and did not wish to provide these drawings prior to a decision.  Robust conditions relating to the submission of detailed drawings and materials will therefore be imposed.  Officers are supportive of this ambitious design and if the material selection and detailing prove to be of a quality that will maintain and enhance the character and appearance of the area the proposed dwelling, although not in keeping with the character and appearance of the surrounding area, could make a positive contribution to the character of the area and therefore in accordance with policy CS7 of the Core Strategy.

- Residential amenity of new and existing residents

The amenity of neighbouring properties and future occupiers of the proposal would typically be assessed in terms of any impact with regard to loss of privacy and overlooking, access to sunlight and daylight and visual dominance and overbearing effects of a development.

Policy DM4 of the Sites and Detailed Policies Document states ‘… Development will not cause a significant detrimental impact to the living environment of existing or new residential properties…’

The application site is located at the bottom of Chapel Hill and the neighbouring property at 32A Chapel Hill is at a much higher level.  This neighbouring property has two lower ground floor side windows (it is assumed to a garage) and a side dormer window facing the application site.  Four ground floor and first floor windows are proposed to the elevation facing this neighbouring property however as the proposed dwelling will be at a lower level than 32A Chapel Hill there is not considered to be any impact on the residents of this neighbouring property in terms of loss of privacy and overlooking.  A ground floor and first floor window is proposed to the elevation facing 101 Lower Elmstone Drive however this neighbouring property has one first floor side window (it is assumed to a landing) and there is a sufficient gap between the properties and therefore the proposal is not considered to impact on the residents of this neighbouring property in terms of loss of privacy and overlooking.

Due to the changing ground levels the application site is at a lower level to the neighbouring properties at 101 Lower Elmstone Drive and 32A Chapel Hill.  The proposal also retains similar spacing around the proposed dwelling to that of the existing bungalow.  The proposed dwelling has also been designed to be lower in height than the neighbouring properties and although it is considerably different in terms of design, appearance and materials it is considered subservient to the surrounding properties within the locality.  Taking all of the above into consideration the proposal is not considered visually dominant or overbearing with no effect of sunlight and daylight to neighbouring properties.

The habitable rooms in the proposed dwelling are considered to be in line with the prescribed national standards and would therefore not be detrimental to the amenities of the future occupants.

The development is therefore considered to accord with the Policies DM4, DM10 and DM11 of the Sites and Detailed Policies Document and the National Planning Policy Framework.

- Amenity space

Policy DM10: Private and Communal Outdoor Space sets out that ‘Dwellings will be provided with functional private or communal open space…  Houses will be provided with private outdoor space whereas flats may be provided with communal outdoor space, balconies and/ or roof gardens.’  The standards previously set out in the Space Around Dwellings Supplementary Planning Guidance have been carried forward to the supporting text of the Sites and Detailed Policies Document at paragraph 5.6.10.

Paragraph 5.6.10 sets out that as a guide a house should be provided with useable private outdoor space no less than the gross floor area of the dwelling to which it relates (measured externally and including garage space).

In this instance the area of the proposed amenity space is of a similar size to the existing garden which is considered adequate and is therefore in accordance with Policy DM10: Private and Communal Outdoor Space of the Sites and Detailed Policies Document.

- Transport implications

The site is located within Zone 3, Secondary Core Area, of the Council’s adopted Parking Standards and Design SPD.  Typically these areas are within 400m of a Reading Buses high frequency ‘Premier Route’, which provides high quality bus routes to and from Reading town centre and other local centre facilities.

The proposed site is situated on the corner of Chapel Hill and Lower Elmstone Drive; both are designated as classified roads (C407).  In accordance with the adopted Parking Standards and Design SPD, the new replacement dwelling would be required to provide 2 off road parking spaces.

Drawing No: 17112_D(00)_100 Rev B - Proposed Ground Floor Plan, illustrates 2 off road parking spaces one of which is in a car port.   It is recommended that the internal dimensions for a car port are 5m long by 3m wide; whilst the length of the car port meets with the current standards, the width does not.  However the driveway provides space for the parking of two cars not including the car port and therefore this meets the Council’s standards.

Following the Transport observations made in the previous pre-application, the original access point is now to be retained with no changes to the width. This is therefore deemed acceptable.

It should be noted that any relocation of the lamp post adjacent to the property boundary, illustrated on submitted plans will need to be met by the applicant.

In accordance with the Council’s Parking Standards and Design SPD, secure cycle storage for 2 cycles should be provided.  Plans indicate that this provision will be provided within the workshop.  This is therefore deemed acceptable.

Bin storage has been illustrated on plans; this should be located no further than 15m from the access point of the site to avoid the stationing of service vehicles on the carriageway for excessive periods. Collection for refuse will be the same as is for the existing dwelling and is therefore deemed acceptable.

Therefore, there are no traffic related objections for the proposed new dwelling subject to conditions and informatives.

- Trees and Landscaping

The Design and Access Statement advises the existing landscape and garden will largely be retained as the garden has always been an important feature of the existing house.  To ensure the garden is preserved and enhanced the Council’s Natural Environment Tree Officer has recommended appropriate conditions.

- Other considerations

CIL

The Council implemented its Community Infrastructure Levy (CIL) on 1st April 2015 and planning applications decided after the 1st of April 2015 may be liable to pay the levy. This proposal would be liable to pay the levy. An appropriate informative would be added.

Affordable Housing

Policy DM6 (Affordable Housing) of the Local Development Framework (LDF) Sites and Detailed Policies Document (SDPD) seeks to ensure that on sites of 1-4 dwellings, a financial contribution will be made that will enable the provision of affordable housing elsewhere in the Borough.  However in this case a financial contribution is excluded as the proposal replaces one existing dwelling with one new dwelling and there is no net increase.

Conclusion

GRANT PERMISSION

Subject to the following conditions:

- Time Limit

- Approved Plans

- Materials – details to be submitted

- CMS

- Vehicle parking space in accordance with approved plans

- Bin storage

- Hours of work

- No Bonfires

- Control of noise and dust

- Remove PD rights

- Landscaping – details to be approved (include boundary walls)

- Landscaping Implementation

- Landscaping Maintenance

- Detailed drawings

Informatives relating to:

- Pre-commencement conditions

- Positive and proactive

- Highways

- Damage to verge

- Terms and conditions

- Building Regs

- Construction and Demolition

- Encroachment and Party Wall

- CIL