DELEGATED APPLICATION REPORT 				Target Date:	29/06/2018

Application No: 180579/HOU

Address: 54 Brunel Road, Reading

Proposal: First floor rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site consists of a two storey semi-detached property with a single storey rear extension.  The property has a detached garage to the rear and is set back from the adjacent property at 56 Brunel Road.  Both 50 and 56 Brunel Road have single storey rear extensions.  The surrounding area is predominantly residential.

Proposal:

The proposal has been amended as it originally involved a first floor rear extension that extended the full width of the property.  As the original proposal encroached on a line taken at 45 degrees from the middle of the closest window at 50 Brunel Road the proposal was amended and brought away from the boundary.

The proposal now extends 1.5m along the boundary with 50 Brunel Road and then steps in from the boundary by 2.75m.  The proposed first floor extension then extends the full depth of the existing single storey rear extension (4.2m in depth).  This section of the proposal extends up to the side elevation adjacent to 56 Brunel Road.  The proposal has two hipped roofs that are set well down from the main roof with an eaves height that matches the existing property.

Rear windows and doors are proposed along with a first floor side window (condition to be obscure glazed and fixed up to 1.7m in height above internal floor level).

A mono pitch roof and rooflight are proposed above the single storey extension.

The materials will consist of white render and tiles to match the existing property.

Plans Considered:

The applicant submitted the following plans, received on 4th April 2018:

Location Plan

Block Plan

The applicant submitted the following drawing, received on 4th May 2018:

Drawing No: 001 – House Extension Plans

The applicant submitted the following amended drawings, received on 16th June 2018:

Drawing No: SB-B54-RE01 Rev 003 – Existing Rear Elevation

Drawing No: SB-B54-RP01 Rev 003 – Existing Roof Plan

Drawing No: SB-B54-SE01 Rev 003 – Existing Side Elevation

Drawing No: SB-B54-FF01 – Rev 003 – Existing First Floor Plan

Drawing No: SB-B54-FF02 – Rev 003 – Proposed First Floor Plan

Drawing No: SB-B54-RE01 Rev 003 – Existing Ground Floor Plan

Drawing No: SB-B54-RE02 Rev 003 – Proposed Rear Elevation

Drawing No: SB-B54-RP02 Rev 003 – Proposed Roof Plan

Drawing No: SB-B54-SE02 Rev 003 – Proposed Side Elevation

The applicant submitted the following amended plans, received on 22nd June 2018:

Drawing No: SB-B54-FF02 – Rev 003 – Proposed First Floor Plan

Drawing No: SB_B54_1:100 Rev 03

Relevant Planning History:

None.

Consultation Responses:

Properties at 52 and 56 Brunel Road and The Coach House Burghfield Road were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

The applicant was advised in an email dated 5th June 2018 that the proposal was not considered acceptable as it extended the full width of the property and encroached on a line taken at 45 degrees from the middle of the closest window at 50 Brunel Road.  Amendments were submitted on 9th June 2018 however the amendment had introduced a splayed design to overcome the 45 degree line issue.  This was not considered in keeping with the character of the existing property.  Further amendments were submitted on 16th June 2018 which were considered to overcome the previous concerns raised.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposed first floor rear extension does not encroach on a line taken at 45 degrees from the middle of the closest habitable windows at neighbouring properties which protects light reaching these rooms which is in line with the Council’s SPG.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, however a condition will be imposed that restricts first floor side windows.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO, although the proposal has a staggered appearance it meets general policy and guidance and a similar proposal was granted planning permission at 73 Brunel Road (application ref: 151912).

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties will notice the additional height and depth the proposal is not considered harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

- First floor side window obscure glazed and fixed

- No first floor side windows

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment