Ward:	Whitley

App No.:	171243/OUT

Address: 	45 Whitley Wood Lane, Reading

Proposal:	An outline application for the erection of a new detached dwelling at land to the rear of 45 Whitley Wood Lane with access off Byworth Close with all matters reserved apart from access and layout.

Case officer:	    Ralph Chakadya

INTRODUCTION

- This application site, outlined in red below, comprises part of the rear garden of 45 Whitley Wood Lane that is overgrown and fenced off from the main rear garden.

- The site is currently bounded by an existing 1800mmhigh timber panel fence to the north-east and north-west site boundary and an 1800mm high painted blockwork wall to the south eastern boundary which forms the boundary demarcation to an adjacent public footpath between Whitley Wood lane and Byworth Close.

- Access to the site is from Byworth Close.

Location Plan (not to scale)

PROPOSAL

- The proposal seeks outline permission with all matters reserved except access and layout for the erection of a new 1.5 storey detached dwelling.

- Access to the site is gained through a garage forecourt area / turning head of Byworth Close. The site is served by an existing double gated site entrance with an existing dropped kerb for the full extent of this site entrance.

- The overall length of the existing garden is 43M nom with the subdivision set at appx 17.5M leaving an overall development site depth of 28.5M.

- The application is supported by the following documents:

- Location Plan No. 1709-100 Rev A

- Existing Site Plan No. 1709-101 A

- Block Plan layout proposals No. 1709-300 A

- Proposed Site Layout Plan No. 1709-301 A

- Design and Access Statement

RELEVANT PLANNING HISTORY

- 171133PRE - New detached dwelling. Withdrawn

- 071249CLE - Certificate of Lawfulness for existing use of property as 2 flats. Permitted 28/01/2008

- 890819 - Conversion and two storey side extension of dwelling to form two flats with car parking. Permited30/06/1989

CONSULTATIONS

Non-statutory

- RBC Transport Strategy – This application seeks outline approval for the erection of a new detached dwelling at land to the rear of 45 Whitley Wood Lane with access off Byworth Close.  No.45 Whitley Wood Lane currently benefits from front access from Whitley Wood Lane and rear access via Byworth Close.

- The site is located within Zone 3, Secondary Core Area, of the Council’s adopted Parking Standards and Design SPD.  In accordance with the adopted SPD, the development is required to provide a parking provision of 2 spaces per 2+ bedroom dwelling whilst retaining sufficient off-street parking for the existing dwelling.

- Firstly the existing dwelling (No45) requires 2 parking spaces, which can be accommodated on the proposed driveway at the side of the house via an existing dropped crossing onto Whitey Wood Lane. The proposed new detached dwelling will be accessed via Byworth Close and is provided with 2no. car parking spaces which complies with the adopted standards.  No additional access needs to be introduced as the site currently benefits from two access points.

- In accordance with the Council’s Parking Standards and Design SPD, a 2+bedroom house should be provided with 2 secure cycle storage spaces.  This can be provided in the form of a garden shed or store.  However, I am happy to deal with this at reserved matters stage.

- On site refuse storage will be provided within the secure garden area. The residents of the dwellings will be required to place the bins outside for kerbside collection.

- There are no transport objections to the application subject to the conditions attached.

- RBC Natural Environment (Trees) – There are no significant tree or landscape features on or adjacent to the site therefore I have no objections in principle to a new property in this location.

- The new dwelling would be visible in views from Whitley Wood Lane. To soften the visual impact of the development we would want to see a quality scheme of landscape planting to enhance both the front of the plot and to provide a green screen between the gardens of the new dwelling and that of 45 Whitley Wood Lane.

Public consultation

- One letter was received commenting the  following:

- Parking of vehicle blocking access to garages on Byworth Close

- Need to retain existing street light for public safety

- Need to provide adequate parking for the proposed dwelling

- RELEVANT PLANNING POLICY AND GUIDANCE

National planning policy guidance:

- NPPF

Core Strategy (2008) policies (altered 2015):

- CS1 (Sustainable Construction and Design)

- CS3 (Social Inclusion and Diversity)

- CS4 (Accessibility and the Intensity of Development)

- CS5 (Inclusive Access)

- CS7 (Design and the Public Realm)

- CS9 (Infrastructure, Services, Resources and Amenities)

- CS14 (Provision of Housing)

- CS15 (Location, Accessibility, Density and Housing Mix)

- CS16 (Affordable Housing)

- CS24 (Car/Cycle Parking)

- CS36 (Biodiversity and Geology)

- CS38 (Trees, Hedges and Woodlands)

Sites and Detailed Policies Document (2012) policies (altered 2015):

- DM1 (Adaptation to Climate Change)

- DM4 (Safeguarding Amenity)

- DM5 (Housing Mix)

- DM6 (Affordable Housing)

- DM10 (Private and Communal Outdoor Space)

- DM11 (Development of Private Residential Gardens)

- DM12 (Access, Traffic and Highway-related matters)

- DM17 (Green Network)

- DM18 (Tree Planting)

Other relevant local planning documents:

- Revised Parking Standards and Design SPD (Oct 2011), and

- Sustainable Design and Construction SPD (July 2011).

- Affordable Housing SPD(2013)

APPRAISAL

Principle of development

- Whilst the site is fenced off, the application site forms part of the rear garden to 45 Whitley Wood Lane. The proposal would therefore constitute development of rear garden land.  The principle of the development would therefore need to be assessed in terms of the requirements of the National Planning Policy Framework (NPPF) paragraph 53 that seeks to ensure that policies are set out to resist inappropriate development of residential gardens. Also, at local level, the Council’s LDF SDPD policy DM11 that establishes a number of criteria against which applications for the development of private residential gardens need to be assessed. Council’s LDF SDPD policy DM10 seeks to ensure that adequate private and communal outdoor space is retained in terms of development proposals.

- The properties on this part of Whitley Wood Lane are characterised by deep rear gardens. The overall length of the existing garden at 45 Whitley Wood Lane is 43m, with the subdivision set at appx 17.5m leaving an overall development site depth of 28.5m. The subdivision of the garden is considered to result in plot sizes that are capable of accommodating and providing adequate garden amenity space to the existing 2 flats at number 45 Whitley Wood Lane as well as the proposed dwelling. Number 16 Byworth Close, a detached bungalow that adjoins the bottom end of the rear gardens to 41, 43 and 45 Whitley Wood Lane with access from Byworth Close, appears to be a rear garden development based on available historical maps. There are also associated outbuildings in this locality defining the built form of the area.  As such the introduction of detached dwelling as proposed, with access from Byworth Close is considered acceptable in principle.

Layout

- As highlighted within the proposals section above, the proposed dwelling would front the turning head of Byworth Close, creating an end to a cul de sac with existing number 16. With a back to back relationship with number 45 Whitley Wood Lane, the resultant layout will define the end of the cul de sac. The proposed dwelling would retain minimum 1m distances from the side boundaries, separation distances considered acceptable.

- The siting of the dwelling within the layout would need to address potential impact to the adjoining properties in terms of potential loss of privacy. Therefore, whilst no elevation details have been provided, the positioning of upper floor windows would need to be restricted to the east (rear), west (front) and south (side) elevation of the proposed dwelling.

Access

- Access to the site is gained through a garage forecourt area / turning head of Byworth Close. The site is served by an existing double gated site entrance with an existing dropped kerb for the full extent of this site entrance. Whilst concerns have been raised by an objector, the Council’s Transport officer has assessed the existing access and considered it acceptable in terms capacity and highway safety.

- The proposed development would provide 2 parking spaces to the front that accord with the adopted SPD, whilst retaining sufficient off-street parking for the existing 45 Whitley Wood Lane.  On site refuse storage will be provided within the secure garden area.

Landscaping

- Whilst landscaping is a reserved matter, the Council’s Natural Environment officer has assessed the proposal and recommended that the proposal has opportunity for soft landscaping to soften the impact of the development as well as enhancing the visual amenity of the area.

Appearance

- Appearance is a reserved matter in this instance and no elevation details have been submitted by the applicant. However, the proposed development at 1.5 storey level is considered in principle not to be detrimental to the character of the area and therefore accords with the policies DM10, CS7 and CS15 and the National Planning Policy Framework.

Representations

- The proposal is provided with adequate onsite parking and therefore is not considered to result in blocking of access to garages on Byworth Close. Whilst the street lamp on the pedestrian access is shown to be relocated, it would be retained in consultation with the Council’s Streetcare team.

Planning obligations

- The Council implemented its Community Infrastructure Levy (CIL) on 1st April 2015.  This proposal would be liable to pay the levy.

S106 Contributions - Affordable Housing

- The proposed development would also need to accord with policy DM6 in terms of making a financial contribution to affordable housing. Policy DM6 (Affordable Housing) of the Local Development Framework (LDF) Sites and Detailed Policies Document (SDPD) seeks to ensure that on sites of 1-4 dwellings, a financial contribution will be made that will enable the provision of affordable housing elsewhere in the Borough. The applicant has agreed to enter into a S106 agreement to make 5% of Gross Development Value (GDV) as off-site financial affordable housing contribution. The proposal therefore accords with the Council’s LDF SDPD Policy DM6 (Affordable Housing) and ‘Affordable Housing’ SPD (2013).

Other considerations

- In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. The key equalities protected characteristics include age, disability, gender, gender reassignment, marriage and civil partnership, pregnancy and maternity, race, religion or belief, sexual orientation. 17 of the car parking spaces proposed would be compliant with dimensions for disabled users. It is considered that there is no indication or evidence (including from consultation on the current application) that the protected groups would have different needs, experiences, issues and priorities in relation to this particular planning application.

CONCLUSION

- Grant subject to S106 Agreement.